3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220405

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2219 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 7.1.1.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR-DC test case 7.1.1.11.1 to the IWD_21wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.11.1required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.11.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	f_TC_7_1_1_11_1_NR5GC	3

2.2	f_GetTestcaseAttrib_NR_DC	7

2.3	f_TC_7_1_1_11_1_NR_TestBody	8

2.4	f_GetTestcaseAttrib_InterBandTC ()	12

3	Branches executed	13

4	Execution Log Files	13

4.1	Qualcomm SM 8450,SDX65 UE	13

5	References	13

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.11.1 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Shaun Harry

shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7_1_1_11_1_NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform and Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450,SDX65 UE

Verification Status:	PASS

# Corrections required

## f_TC_7_1_1_11_1_NR5GC

| Function name | f_TC_7_1_1_11_1_NR5GC |
| --- | --- |
| Reason for change | - The PHR configuration defined in Table 7.1.1.11.1.3.3-2 is not clear on which Cell this configuration needs to be applied. Since the test purpose is to check reporting of PHR on both PCell and PSCell, it is proposed to add PHR configuration on PCell and PSCell - Based on the above update the UE will send PSCell PHR separatelty and needs to be handled in the alt function - NR Cell1 power is configured to -82 at T0 and therfore the max cell power needs to be initialised an power changes for NRCell 1 needs to be called - UE can only report PHR on NRCell 10 when there is uplink grant configured. Periodic uplink grant does not spefiy the subframe to use - In order to avoid BSR timer related issues due to UE capability being too long it is proposed to update the BSR time after the preamble (see R5s210862) |
| Summary of change | - Created variables to configure cs_38508_CellGroupConfig for SCG and build RRCReconfiguration message with PHR configuration for PCell and PSCell Changes related to this change in Turquoise - Added additional condition to handle PSCell PHR as part of the alt procedure. Alt procedure is exited only when PCell and PSCell PHR is reported. Changes related to this change is highlighted in green - Added f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -82, -82); and called f_NR_SetCellPower (nr_Cell1, -82, -82); to configure power level to -82. Changes related to this change is highlighted in pink - Added uplink grant configuration for Cell 10 and specified the subframe to be used for sending periodic uplink grant. Updated the NR Cell 1 uplink grant to specify the specific subframe. Changes related to this change is highlighted in Teal - Moved f_NR_CellInfo_SetMAC_CellGroupConfigL2 after the preamble |
| TTCN module | MAC_CA_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_11_1_NR5GC() runs on NR5GC_PTC { //DC power headroom reporting / PSCell activation and DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo; f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);// f_NR_SetCellPower (nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_RRCReconfig_AddSCG_DRBs_NRDC(nr_Cell1, nr_Cell10, MCG_SCG);// f_NR_TestBody_Set(true); v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 2 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_711111); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //@siclog "Steps 1" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10); //@siclog "Steps 2" siclog@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_PHR(enable)); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@siclog "Steps 3" siclog@ alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } [not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?)))-> value v_NR_SYSTEM_IND { v_TimingInfo := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour"); v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } }; //@siclog "Steps 5-18" siclog@ f_TC_7_1_1_11_1_NR_TestBody(nr_Cell1, nr_Cell10, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?),v_TimingInfo); //@siclog "Steps 19-20" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_TestBody_Set(false); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_PHR(disable)); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_11_1_NR5GC() runs on NR5GC_PTC { //DC power headroom reporting / PSCell activation and DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) CellGroupConfig v_MCGConfig; // var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig_SCG; // var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var boolean v_PHR_PSCell_Rcvd; // var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo; var SubFrameTiming_Type v_TimingInfo_PSCell; // f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -82, -82); //WA#WI=889297 max pwr for FR1 //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);// f_NR_SetCellPower (nr_Cell1, -82, -82); //WA#WI=889297 max pwr T0 f_NR_SetCellPower (nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //WA#WI=889297 moved the configuration after preamble to avoid UE capability procedure issue due to BSR timer f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); f_NR_RRCReconfig_AddSCG_DRBs_NRDC(nr_Cell1, nr_Cell10, MCG_SCG);// f_NR_TestBody_Set(true); v_RRC_MsgRcvd:= false; v_PHR_Rcvd := false; v_PHR_PSCell_Rcvd :=false; // v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); //@siclog "Steps 2 preconfig" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig_SCG := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); // v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_711111); v_MAC_CellGroupConfig_SCG.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_711111); // f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell10, v_MAC_CellGroupConfig_SCG); // v_MCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, // omit, omit, v_MAC_CellGroupConfig_SCG); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_SCGConfig, v_MCGConfig); //WA#WI=889297 to update phr for scgconfig //@siclog "Steps 1" siclog@ //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(2), v_NR_ResourceAllocation, 10); // v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell10, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell10, cs_TimingInfo_SubFrame(5), v_NR_ResourceAllocation, 10); //@siclog "Steps 2" siclog@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_PHR(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell10, cds_NR_SystemIndCtrl_MAC_PHR(enable)); // f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@siclog "Steps 3" siclog@ alt { ////WA#WI=889297 handle PSCell PHR entry [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_RRC_MsgRcvd := true; if((not v_PHR_Rcvd) or (not v_PHR_PSCell_Rcvd)) { repeat;} } [not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?)))-> value v_NR_SYSTEM_IND { v_TimingInfo := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour PCell PHR received"); v_PHR_Rcvd := true; if((not v_RRC_MsgRcvd) or (not v_PHR_PSCell_Rcvd)){ repeat;} } [not v_PHR_PSCell_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell10, cr_NR_MAC_CE_MultiEntryPHR(?)))-> value v_NR_SYSTEM_IND { v_TimingInfo_PSCell := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour PSCell PHR received"); v_PHR_PSCell_Rcvd := true; if((not v_RRC_MsgRcvd) or (not v_PHR_Rcvd)){ repeat;} } }; //@siclog "Steps 5-18" siclog@ f_TC_7_1_1_11_1_NR_TestBody(nr_Cell1, nr_Cell10, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?),v_TimingInfo, v_TimingInfo_PSCell); //@siclog "Steps 19-20" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_TestBody_Set(false); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_MAC_PHR(disable)); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## f_GetTestcaseAttrib_NR_DC

| Function name | f_GetTestcaseAttrib_NR_DC |
| --- | --- |
| Reason for change | TC is an NR-DC test case and needs to be added to the function f_GetTestcaseAttrib_NR_DC |
| Summary of change | Added case ("TC_7_1_1_11_1_NR5GC") { return true; } |
| TTCN module | TestcaseProperties |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_NR_DC(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_7_1_3_5_2_NR5GC") { return true; } case ("TC_7_1_3_5_3_NR5GC") { return true; } case ("TC_7_1_3_5_5_NR5GC") { return true; } case ("TC_8_2_2_1_2_NR5GC") { return true; }

After Change:

function f_GetTestcaseAttrib_NR_DC(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_7_1_1_11_1_NR5GC") { return true; } // case ("TC_7_1_3_5_2_NR5GC") { return true; } case ("TC_7_1_3_5_3_NR5GC") { return true; } case ("TC_7_1_3_5_5_NR5GC") { return true; } case ("TC_8_2_2_1_2_NR5GC") { return true; }

## f_TC_7_1_1_11_1_NR_TestBody

| Function name | f_TC_7_1_1_11_1_NR_TestBody |
| --- | --- |
| Reason for change | PCell power levels are not according to the prose Based on changes proposed in 2.1 the PHR reporting in Step 15 and Step 18 will be on PSCell as the power level changes in Cell 10 will triggere this Based on the agreed prose CR R5-217806 steps 6 to 12 are executed if PCell is FR1, therefore added the condigition to take this into consideration When executed with FR1-FR2 combination Step 14 timing is checked against Step 3A (See R5-217806) |
| Summary of change | Modified the Pcell power level according to prose. Updated the procedure at Step 15 and Step 18 to received the PHR on Cell10 Steps 6 to 12 are executed based on scs for Pcell (highlighted in green) Updated the timing to check against 3A |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_11_1_NR_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_PSCell, template (present) NR_MAC_CE_MultiEntryPHR_Type p_NR_MAC_CE_MultiEntryPHR, template (value) SubFrameTiming_Type p_TimingInfo) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1 :=valueof (p_TimingInfo); var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_TimingInfo4; var SubFrameTiming_Type v_TimingInfo5; var float v_TimerMax :=f_NR_SetTimerToleranceMax (p_PCell, l2Timer, 0.5); var float v_TimerMin :=f_NR_SetTimerToleranceMin (p_PCell, l2Timer, 0.5); var float v_TimerDuration ; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PCell, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell,tsc_NR_ServingCellSSS_EPRE_FR1 , tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2_T4 := {cs_NR_CellPower(p_PCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell,tsc_NR_ServingCellSSS_EPRE_FR1 , tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3 := {cs_NR_CellPower(p_PCell, tsc_NR_ServingCellSSS_EPRE_FR1 , tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2)}; //@siclog "Step 6" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo1, v_TimingInfo2 ); //@sic R5s210336 sic@ if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); }; //@siclog "Step 9" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4); //@siclog "Steps 10-11" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_TimingInfo3 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo2, v_TimingInfo3); if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); } //@siclog "Step 12-13" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); v_TimingInfo4 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo3, v_TimingInfo4); if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); } //@siclog "Step 16" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4); //@siclog "Steps 17-18" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); v_TimingInfo5 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo4, v_TimingInfo5);//@sic R5s210336 sic@ if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18: TP3 failed, UE didn't send PHR after periodic PHR expiry"); } };

After Change:

function f_TC_7_1_1_11_1_NR_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_PSCell, template (present) NR_MAC_CE_MultiEntryPHR_Type p_NR_MAC_CE_MultiEntryPHR, template (value) SubFrameTiming_Type p_TimingInfo, template (value) SubFrameTiming_Type p_TimingInfo2) runs on NR_BASE_PTC //WA#WI=889297 { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1 :=valueof (p_TimingInfo); var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_TimingInfo3a :=valueof(p_TimingInfo2); //WA#WI=889297 PSCell timing info var SubFrameTiming_Type v_TimingInfo4; var SubFrameTiming_Type v_TimingInfo5; var float v_TimerMax :=f_NR_SetTimerToleranceMax (p_PCell, l2Timer, 0.5); var float v_TimerMin :=f_NR_SetTimerToleranceMin (p_PCell, l2Timer, 0.5); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_PSCell); // var float v_TimerDuration ; // var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PCell, -91, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell,tsc_NR_ServingCellSSS_EPRE_FR1 , tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2_T4 := {cs_NR_CellPower(p_PCell, -82, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell,tsc_NR_ServingCellSSS_EPRE_FR1 , tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3 := {cs_NR_CellPower(p_PCell, -82 , tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_PSCell, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2)}; //@siclog "Step 6" siclog@ if ((v_SCS == kHz15) or (v_SCS == kHz30)){ // to skip steps based on FR1 or FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo1, v_TimingInfo2 ); //@sic R5s210336 sic@ if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); }; //@siclog "Step 9" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4); //@siclog "Steps 10-11" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, p_NR_MAC_CE_MultiEntryPHR)) -> value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_TimingInfo3 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo2, v_TimingInfo3); if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); } } //@siclog "Step 12-13" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PSCell, cr_NR_MAC_CE_MultiEntryPHR(?))) -> value v_NR_SYSTEM_IND; //WA#WI=889297 changed to PSCell f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); v_TimingInfo4 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; // v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo3a, v_TimingInfo4); // if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15: TP3 failed, UE didn't send PHR after prohibit PHR expiry"); } //@siclog "Step 16" siclog@ f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4); //@siclog "Steps 17-18" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PSCell, cr_NR_MAC_CE_MultiEntryPHR(?))) -> value v_NR_SYSTEM_IND; //WA#WI=889297 changed to PSCell f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); v_TimingInfo5 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo4, v_TimingInfo5);//@sic R5s210336 sic@ if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18: TP3 failed, UE didn't send PHR after periodic PHR expiry"); } };

## f_GetTestcaseAttrib_InterBandTC ()

| Function name | f_GetTestcaseAttrib_InterBandTC |
| --- | --- |
| Reason for change | Test case 7.1.1.11.1 need to bessedto the List of InterBand TC’s |
| Summary of change | Test case 7.1.1.11.1 added to list of InterBand TC’s |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change

function f_GetTestcaseAttrib_InterBandTC(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_6_1_2_23_NR5GC") { return true; } case ("TC_6_1_2_4_NR5GC") { return true; } case ("TC_6_1_2_5_NR5GC") { return true; } case ("TC_7_1_3_5_2_NR5GC") { return true; } case ("TC_7_1_3_5_3_NR5GC") { return true; } case ("TC_7_1_3_5_5_NR5GC") { return true; } case ("TC_8_1_3_1_10_NR5GC") { return true; } …. } return false; }

After Change

function f_GetTestcaseAttrib_InterBandTC(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_6_1_2_23_NR5GC") { return true; } case ("TC_6_1_2_4_NR5GC") { return true; } case ("TC_6_1_2_5_NR5GC") { return true; } case ("TC_7_1_1_11_1_NR5GC") { return true; } case ("TC_7_1_3_5_2_NR5GC") { return true; } case ("TC_7_1_3_5_3_NR5GC") { return true; } case ("TC_7_1_3_5_5_NR5GC") { return true; } ….. } return false; }

# Branches executed

This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) using DC_n79A_n257A NRDC band combination on Rohde & Schwarz 5G Protocol Conformance Test platform and Keysight S8704A Protocol Conformance Toolset.

# Execution Log Files

## Qualcomm SM 8450,SDX65 UE

The Qualcomm UE passed this test case on R&S® 5G Protocol Conformance Test platform in band combination n79A_n257A using nea1/nia1 algorithms. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

R&S logs\ TC_7_1_1_11_1_NR5GC_QC.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

Keysight logs\ TC_7_1_1_11_1_LOG.html

- PICS/PIXIT parameter file:

Keysight logs\ TC_7_1_1_11_1_PIXIT.xml

# References

| [1] | R5s211699: Supporting information for addition of NR5GC TC 7.1.1.11.1. |
| --- | --- |