3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220417

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2419 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC postamble function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For STATE_EMERGENCY_RELEASE_IDLE, the UE can immediately send DEREGISTRATION REQUEST on switch off, so TTCN should not block waiting for UT CNF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Call f_UT_SwitchOffUE with option NO_CNF_REQUIRED so TTCN is immediately ready to handle de-registration signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some emergency calling test cases in postamble steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_Postamble() |
| --- | --- |
| Reason for change | For STATE_EMERGENCY_RELEASE_IDLE, the UE can immediately send DEREGISTRATION REQUEST on switch off, so TTCN should not block waiting for UT CNF. |
| Summary of change | Call f_UT_SwitchOffUE with option NO_CNF_REQUIRED so TTCN is immediately ready to handle de-registration signalling. |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment |  |

Before Change

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_DetachReq; var boolean v_RxdMsg := false; timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ ……… } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; // @sic R5s211344 sic@ v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait); if (v_RxdMsg) { f_NR_RRCSetup_Def (p_NR_CellId); v_DetachReq := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } f_NR_RRCRelease(p_NR_CellId); } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, p_SRB); // @sic R5-195241, R5s21163 sic@ } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

After Change

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_DetachReq; var boolean v_RxdMsg := false; timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ …… } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false); t_Wait.start; // @sic R5s211344 sic@ v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait); if (v_RxdMsg) { f_NR_RRCSetup_Def (p_NR_CellId); v_DetachReq := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } f_NR_RRCRelease(p_NR_CellId); } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, p_SRB); // @sic R5-195241, R5s21163 sic@ } //And now remove the cell(s) f_NR_ReleaseAllCells(); }