3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220481

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2457 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The 2 minute timer for higher priority PLMN selection will be started at the UE on receiving REGISTRATION ACCEPT. Currently the starting of the matching timer in TTCN is delayed by the call to f_NR5GC_RRC_Idle_Steps15_20() 2. Steps 19Aa1-20 of TS 38.508-1 Table 4.5.3.2-1 are already performed in the scope of function f_NR5GC_RRC_Idle_Steps15_20() and are currently duplicated in this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Move starting of Tmin and Tmax timers to before call to f_NR5GC_RRC_Idle_Steps15_20() 2. Remove the duplicated code after call to f_NR5GC_RRC_Idle_Steps15_20() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The 2 minute timer for higher priority PLMN selection will be started at the UE on receiving REGISTRATION ACCEPT. Currently the starting of the matching timer in TTCN is delayed by the call to f_NR5GC_RRC_Idle_Steps15_20() - Steps 19Aa1-20 of TS 38.508-1 Table 4.5.3.2-1 are already performed in the scope of function f_NR5GC_RRC_Idle_Steps15_20() and ar ecurrently duplicated in this test case. |
| Summary of change | - Move starting of Tmin and Tmax timers to before call to f_NR5GC_RRC_Idle_Steps15_20() - Remove the duplicated code after call to f_NR5GC_RRC_Idle_Steps15_20() |
| TTCN module | Inactive_PLMNSelection_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC {// Power levels on FR2 are FFS // @sic R5s201047, R5-203382, R5s210094, R5-210062, R5s210454, R5-216995, R5-220543 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NR_DRBInfoList_Type v_ExistingDRBs; var template(value) RadioBearerConfig v_RadioBearerConfig; var integer i := 0; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig; var NR_UplinkBWP_Type v_NR_UplinkBWP; timer t_Tmin; timer t_Tmax; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, -78, -78), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 0A" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 0B" siclog@ // The UE is switched on f_UT_SwitchOnUE(UT); // @siclog "Step 0C1-0C19a1" siclog@ // The steps 1-19a1 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); //MOVE // @siclog "Step 0D" siclog@ // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); // @siclog "Step 0Ea1-0F" siclog@ // steps 19Aa1-20 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell12); f_NR5GC_RRC_Idle_Extension(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); } f_NR_RRC_InactiveDef(nr_Cell12, -, false); ………………….. }

After Change

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC {// Power levels on FR2 are FFS // @sic R5s201047, R5-203382, R5s210094, R5-210062, R5s210454, R5-216995, R5-220543 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NR_DRBInfoList_Type v_ExistingDRBs; var template(value) RadioBearerConfig v_RadioBearerConfig; var integer i := 0; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig; var NR_UplinkBWP_Type v_NR_UplinkBWP; timer t_Tmin; timer t_Tmax; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell13, -78, -78), cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 0A" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 0B" siclog@ // The UE is switched on f_UT_SwitchOnUE(UT); // @siclog "Step 0C1-0C19a1" siclog@ // The steps 1-19a1 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 0D" siclog@ // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); f_NR5GC_RRC_Idle_Steps15_20(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); //MOVED t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); //MOVED t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); // @siclog "Step 0Ea1-0F" siclog@ // steps 19Aa1-20 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed /*REMOVE if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell12); f_NR5GC_RRC_Idle_Extension(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease); }*/ f_NR_RRC_InactiveDef(nr_Cell12, -, false);