3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220482

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2458 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With implementation of R5-221406, usage of vc_EUTRA_Multi5GS_Global.NetworkSupport with set and get operations are done for NR5GC_IRAT test cases. Usually the value of this global variable mentioned above is set IRAT common functions but 9.3.1.2 on his EUTRA preamble does not make use of them, leaving vc_EUTRA_Multi5GS_Global.NetworkSupport uninitialized during the test execution. Later in test case value is trying to be accessed at f_EUTRA38_MultiPDN_GetPCOs provoking an error as variable is uninitialized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialized global variable vc_EUTRA_Multi5GS_Global.NetworkSupport at the start of the test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_TC_9_3_1_2_NR5GC_EUTRA	3

# Corrections required

## Correction to function f_TC_9_3_1_2_NR5GC_EUTRA

| Function name | f_TC_9_3_1_2_NR5GC_EUTRA |
| --- | --- |
| Reason for change | vc_EUTRA_Multi5GS_Global.NetworkSupport global variable initiaöization is done in IRAT common functions but 9.3.1.2 on his EUTRA preamble does not make use of them, leaving vc_EUTRA_Multi5GS_Global.NetworkSupport uninitialized during the test execution. |
| Summary of change | Initialized global variable vc_EUTRA_Multi5GS_Global.NetworkSupport at the start of the test case. |
| TTCN module | NR5GC_IRAT\9_3_1\Mobility_Registration_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_9_3_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //Mobility Registration Update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC var NAS_MSG_Indication_Type v_NAS_Ind; f_EUTRA38_IRAT_NAS_Init(LTE_AllCellsOnSamePLMN, c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_CellA); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_9_3_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //Mobility Registration Update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC var NAS_MSG_Indication_Type v_NAS_Ind; f_EUTRA38_IRAT_NAS_Init(LTE_AllCellsOnSamePLMN, c31); f_EUTRA38_SetNetworkSupport(NOT_SUPPORTED); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_CellA); <<SKIPPED CODE>>