3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220489

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2463 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current test case the function f_NR5GC_RRCReestablishment_Steps0To2 is used to handle the re-establishment procedure, this results in updating the C-RNTI and also providing the DL/UL DCCH configuration to use CSS search space. Since this is not a successful reestablishment procedure, this reconfiguration is not required as we only need to handle RRCRestablishment. Therefore, it is proposed to handle the RRCRestablishment directly and reset the SRBs/DRBs on Cell1 for the subsequent connections |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the code to handle the reestablishment message in the test case and reset SRBs / DRBs for Cell 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required to test case 8.1.6.1.3.3	2

1.1	f_TC_8_1_6_1_3_3_TestBody	2

# Corrections required to test case 8.1.6.1.3.3

## f_TC_8_1_6_1_3_3_TestBody

| Function name | f_TC_8_1_6_1_3_3_TestBody |
| --- | --- |
| Reason for change | In current test case the function f_NR5GC_RRCReestablishment_Steps0To2 is used to handle the re-establishment procedure, this results in updating the C-RNTI and also providing the DL/UL DCCH configuration to use CSS search space. Since this is not a successful reestablishment procedure, this reconfiguration is not required as we only need to handle RRCRestablishment. Therefore, it is proposed to handle the RRCRestablishment directly and reset the SRBs/DRBs on Cell1 for the subsequent connections |
| Summary of change | Modified the code to handle the reestablishment message in the test case and reset SRBs / DRBs for Cell 1 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell2; // @sic R5-216290, R5-221579 sic@ var PhysCellId v_PhysCellId_Cell2 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var ARFCN_ValueNR v_SSB_Freq_Cell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var CellIdentity v_CellIdentity_Cell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var PLMN_Identity v_PLMN_Identity_Cell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var TrackingAreaCode v_TAC_Cell2 := f_NR_CellInfo_GetTAC (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var CellIdentity v_CellIdentity_Cell4 := f_NR_CellInfo_GetCellIdentity(nr_Cell4); var PLMN_Identity v_PLMN_Identity_Cell4 := f_NR_CellInfo_GetPLMN (nr_Cell4); var TrackingAreaCode v_TAC_Cell4 := f_NR_CellInfo_GetTAC (nr_Cell4); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var integer v_WaitTime := 1;//T301 is ms1000 by default // @sic R5s211600 sic@ var float v_T301Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); // @sic R5s211600 sic@ //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467, R5-221579 sic@ v_RNTI_Value_Cell2 := f_NR_CellInfo_GetRNTI(nr_Cell2); // @sic R5-216290, R5-221579 sic@ //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(v_T301Min); // @sic R5s211600 sic@ //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell2); // @sic R5-221579 sic@ f_NR_RRCSetup_Def(nr_Cell2); //@siclog "Step 6" siclog@

After Change:

module RRC_MDT_IntraNR_RLF_NR5GC { … import from NR5GC_RRC_Templates all; … … function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell2, v_RNTI_Value; // @sic R5-216290, R5-221579 sic@ var PhysCellId v_PhysCellId_Cell2 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var PhysCellId v_PhysCellId_Cell1 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ARFCN_ValueNR v_SSB_Freq_Cell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var CellIdentity v_CellIdentity_Cell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var PLMN_Identity v_PLMN_Identity_Cell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var TrackingAreaCode v_TAC_Cell2 := f_NR_CellInfo_GetTAC (nr_Cell2); // @sic R5-217922, R5s211600, R5-221579 sic@ var CellIdentity v_CellIdentity_Cell4 := f_NR_CellInfo_GetCellIdentity(nr_Cell4); var PLMN_Identity v_PLMN_Identity_Cell4 := f_NR_CellInfo_GetPLMN (nr_Cell4); var TrackingAreaCode v_TAC_Cell4 := f_NR_CellInfo_GetTAC (nr_Cell4); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) // @sic R5-221579 sic@ }; var integer v_WaitTime := 1;//T301 is ms1000 by default // @sic R5s211600 sic@ var float v_T301Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); // @sic R5s211600 sic@ var ShortMAC_I v_ShortMAC_I; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. //f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467, R5-221579 sic@ // handle reestablishment procedure and reset DRB/SRBs without any changes to grant v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_ShortMAC_I:= f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId_Cell1, v_ShortMAC_I), otherFailure))); v_RNTI_Value_Cell2 := f_NR_CellInfo_GetRNTI(nr_Cell2); // @sic R5-216290, R5-221579 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit, true); //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(v_T301Min); // @sic R5s211600 sic@ //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell2); // @sic R5-221579 sic@ f_NR_RRCSetup_Def(nr_Cell2); //@siclog "Step 6" siclog@