3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220505

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0807 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS call control test case 7.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Video codec input parameter to create the SDP offer is set to H264, H265 is the required codec for NR video test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed video codec input parameter to to build the SDP offer to H265 . Note : This change was previously accepted in R5s220235 but was missing in IWD 22wk12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.16.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_16_NR5GC_IMS1() |
| --- | --- |
| Reason for change | Video codec input parameter to create the SDP offer is set to H264, H265 is the required codec for NR video test cases |
| Summary of change | Changed video codec input parameter to to build the SDP offer to H265 . Note : This change was previously accepted in R5s220235 but was missing in IWD 22wk12 |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_16_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (value) SDP_Message v_SdpOffer; var SipUrl v_ContactUrl; f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); // @sic R5s220235: IPCAN_VideoCall instead of IPCAN_SpeechCall sic@ f_IMS_TestBody_Set(true); // @siclog "Step 0A - 0H" siclog@ f_IMS_CC_StartCall(IPCAN_MT_VideoCall); // @siclog "Step 1" siclog@ INVITE v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H264_only); v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress …… … }

After Change

| function f_TC_7_16_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (value) SDP_Message v_SdpOffer; var SipUrl v_ContactUrl; f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED); // @sic R5s220235: IPCAN_VideoCall instead of IPCAN_SpeechCall sic@ f_IMS_TestBody_Set(true); // @siclog "Step 0A - 0H" siclog@ f_IMS_CC_StartCall(IPCAN_MT_VideoCall); // @siclog "Step 1" siclog@ INVITE v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H265_only); v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress ……………… } |  |
| --- | --- |