3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220568

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4671 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA MBMS test case 17.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At "Step 0A-OC", UE sends MBMSInterestIndication with the frequency of the serving cell. The current implementation using "dl_CarrierFreq" of TTCN-3 will only check the correct frequency value for bands up to 64, and for bands above it will check for the MAX EARFCN value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use IE "dl_CarrierFreq_r9" instead of “dl_CarrierFreq” to check correct EARFCN for all EUTRA bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE operating on EUTRA band > 64 will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_17_4_1_Common |
| --- | --- |
| Reason for change | At "Step 0A-OC", UE sends MBMSInterestIndication with the frequency of the serving cell. The current implementation using "dl_CarrierFreq" of TTCN-3 will only check the correct frequency value for bands up to 64, and for bands above it will check for the MAX EARFCN value. |
| Summary of change | Use IE "dl_CarrierFreq_r9" instead of “dl_CarrierFreq” to check correct EARFCN for all EUTRA bands. |
| TTCN module | MBMS_SC_TestCases.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_17_4_1_Common(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var integer v_MBMS_CounterValue;

var integer v_MBMS_CounterValue2;

var Frequency_IE_Type v_Frequency_Cell1;

var template (value) CellPowerList_Type v_CellPowerList_AtT0 := {

cs_CellPower(eutra_Cell11, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE),

cs_CellPower(p_CellId, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE)

};

var template (value) CellPowerList_Type v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell11, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE),

cs_CellPower(p_CellId, -79)

};

var EUTRA_FDD_TDD_CellInfo_Type  v_EUTRA_FDD_TDD_CellInfo; //@sic R5-170931 sic@

var default v_DefaultRef;

f_EUTRA_Init(c3);

f_EUTRA_SetSysinfoCombination_MBMS(eutra_Cell1,c16);

f_EUTRA_SetSysinfoCombination_MBMS(eutra_Cell11,c16);

// Cell 1 and  Cell 11 are part of the same MBSFN area @sic R5s140757 sic@

f_EUTRA_InitialiseWithSameCellTiming ( {eutra_Cell1, eutra_Cell11} );

//@sic R5-170931 sic@

v_EUTRA_FDD_TDD_CellInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info( eutra_Cell1);

if (v_EUTRA_FDD_TDD_CellInfo.FDD_TDD == TDD) //@sic R5-170931 sic@

{

v_CellPowerList_AtT0 := {

cs_CellPower(eutra_Cell11, -89),

cs_CellPower(p_CellId, -89)

};

v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell11, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell1, -89),

cs_CellPower(p_CellId, -79)

};

};

if (p_CellId == eutra_Cell10) { //Test case 17.4.1a

f_EUTRA_InitSIB5_Multiband (eutra_Cell1);

f_EUTRA_InitSIB5_Multiband (p_CellId);

}

//@sic R5-155495 sic@

f_EUTRA_CellInfo_SetSib3_NeighCellConfig ( eutra_Cell1,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib3_NeighCellConfig ( eutra_Cell11,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib5_NeighCellConfig ( eutra_Cell1,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib5_NeighCellConfig ( eutra_Cell11,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_MBMS_Def(eutra_Cell1); // Configured MBSFN + MTCH + transmit MBSFNAreaConfiguration

f_EUTRA_CellConfig_Def(eutra_Cell11);

f_EUTRA_CellConfig_MBMS_Def(eutra_Cell11); // Configured MBSFN + MTCH + transmit MBSFNAreaConfiguration

f_EUTRA_CellInfo_InitMaxReferencePower(p_CellId, -79);  //@sic R5s140757 sic@

f_EUTRA_CellConfig_Def(p_CellId);

v_Frequency_Cell1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1);

//Perform Idle updated procedure to make sure UE is in a known state before test case starts

//Bring UE to initial state

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeC);

f_EUTRA_RbEst_Def(eutra_Cell1); // Default RB establishment

f_EUTRA_CloseUE_TestLoopModeC(eutra_Cell1, cs_UE_TestLoopModeC_LB_Setup); // Bring UE in test loop mode C

f_UT_MBMS_SERVICE_Interest(UT, omit, oct2str(tsc_ServiceId_Def), true,tsc_MBMS_SAI_Def);

f_UT_MBMS_SERVICE_Active(UT, oct2str(tsc_ServiceId_Def),true, tsc_MBMS_SAI_Def );

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 0A-OC" siclog@

f_ModifySysinfoCombinationC16_ToC20(eutra_Cell1);

f_EUTRA_ModifySysinfo(eutra_Cell1, true, RRC_CONNECTED);

f_ModifySysinfoCombinationC16_ToC20(eutra_Cell11);

f_EUTRA_ModifySysinfo(eutra_Cell11, false, RRC_CONNECTED);

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MbmsInterestIndication_Common(cr_MbmsInterestIndication( {v_Frequency_Cell1.UL_DL_Earfcn.dl_CarrierFreq} ))));

//@siclog "Step 1" siclog@

//Release RRC connection

f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);

After change:

function f_TC_17_4_1_Common(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var integer v_MBMS_CounterValue;

var integer v_MBMS_CounterValue2;

var Frequency_IE_Type v_Frequency_Cell1;

var template (value) CellPowerList_Type v_CellPowerList_AtT0 := {

cs_CellPower(eutra_Cell11, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE),

cs_CellPower(p_CellId, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE)

};

var template (value) CellPowerList_Type v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell11, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE),

cs_CellPower(p_CellId, -79)

};

var EUTRA_FDD_TDD_CellInfo_Type  v_EUTRA_FDD_TDD_CellInfo; //@sic R5-170931 sic@

var default v_DefaultRef;

f_EUTRA_Init(c3);

f_EUTRA_SetSysinfoCombination_MBMS(eutra_Cell1,c16);

f_EUTRA_SetSysinfoCombination_MBMS(eutra_Cell11,c16);

// Cell 1 and  Cell 11 are part of the same MBSFN area @sic R5s140757 sic@

f_EUTRA_InitialiseWithSameCellTiming ( {eutra_Cell1, eutra_Cell11} );

//@sic R5-170931 sic@

v_EUTRA_FDD_TDD_CellInfo := f_EUTRA_CellInfo_GetFDD_TDD_Info( eutra_Cell1);

if (v_EUTRA_FDD_TDD_CellInfo.FDD_TDD == TDD) //@sic R5-170931 sic@

{

v_CellPowerList_AtT0 := {

cs_CellPower(eutra_Cell11, -89),

cs_CellPower(p_CellId, -89)

};

v_CellPowerList_AtT1 := {

cs_CellPower(eutra_Cell11, tsc_ServingCellRS_EPRE),

cs_CellPower(eutra_Cell1, -89),

cs_CellPower(p_CellId, -79)

};

};

if (p_CellId == eutra_Cell10) { //Test case 17.4.1a

f_EUTRA_InitSIB5_Multiband (eutra_Cell1);

f_EUTRA_InitSIB5_Multiband (p_CellId);

}

//@sic R5-155495 sic@

f_EUTRA_CellInfo_SetSib3_NeighCellConfig ( eutra_Cell1,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib3_NeighCellConfig ( eutra_Cell11,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib5_NeighCellConfig ( eutra_Cell1,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellInfo_SetSib5_NeighCellConfig ( eutra_Cell11,

tsc_NeighCellConfig_NeighbourMBSFN);

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_MBMS_Def(eutra_Cell1); // Configured MBSFN + MTCH + transmit MBSFNAreaConfiguration

f_EUTRA_CellConfig_Def(eutra_Cell11);

f_EUTRA_CellConfig_MBMS_Def(eutra_Cell11); // Configured MBSFN + MTCH + transmit MBSFNAreaConfiguration

f_EUTRA_CellInfo_InitMaxReferencePower(p_CellId, -79);  //@sic R5s140757 sic@

f_EUTRA_CellConfig_Def(p_CellId);

v_Frequency_Cell1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1);

//Perform Idle updated procedure to make sure UE is in a known state before test case starts

//Bring UE to initial state

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeC);

f_EUTRA_RbEst_Def(eutra_Cell1); // Default RB establishment

f_EUTRA_CloseUE_TestLoopModeC(eutra_Cell1, cs_UE_TestLoopModeC_LB_Setup); // Bring UE in test loop mode C

f_UT_MBMS_SERVICE_Interest(UT, omit, oct2str(tsc_ServiceId_Def), true,tsc_MBMS_SAI_Def);

f_UT_MBMS_SERVICE_Active(UT, oct2str(tsc_ServiceId_Def),true, tsc_MBMS_SAI_Def );

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 0A-OC" siclog@

f_ModifySysinfoCombinationC16_ToC20(eutra_Cell1);

f_EUTRA_ModifySysinfo(eutra_Cell1, true, RRC_CONNECTED);

f_ModifySysinfoCombinationC16_ToC20(eutra_Cell11);

f_EUTRA_ModifySysinfo(eutra_Cell11, false, RRC_CONNECTED);

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MbmsInterestIndication_Common(cr_MbmsInterestIndication( {v_Frequency_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9} ))));

//@siclog "Step 1" siclog@

//Release RRC connection

f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);