3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220645

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2399 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.5.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 8.1.5.9.1 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.5.9.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	f_TC_8_1_5_9_1_NR5GC	4

2.2	f_TC_8_1_5_9_1_NR5GC_TestBody	5

2.3	fl_CalculateTiming_8_1_5_9_1	9

3	Branches executed	11

4	Execution Log Files	11

4.1	MediaTek MT6983 UE	11

5	References	11

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.9.1 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.5.9.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## f_TC_8_1_5_9_1_NR5GC

| Function name | f_TC_8_1_5_9_1_NR5GC |
| --- | --- |
| Reason for change | In step 7 the test case turns off NR Cell 1 while the UE is in RRC connected state on NR Cell 1. In step 9 (after 1s (T310 expiry) ) the test case turns on NR Cell 1. During this time both T310 and T311(set to 1s) expire and the UE goes to RRC IDLE state. In step 10 it is expected the the UE attempts to re-establish the RRC connection but since the UE already has been in RRC IDLE for about 2s that will not happen |
| Summary of change | Increased the T311 value |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_9_1_NR5GC() runs on NR5GC_PTC {// RACS / UL Message Segment transfer / UECapabilityInformation f_NR5GC_Init(NR_1); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_5_9_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } //end of f_TC_8_1_5_9_1_NR5GC

After Change:

function f_TC_8_1_5_9_1_NR5GC() runs on NR5GC_PTC {// RACS / UL Message Segment transfer / UECapabilityInformation f_NR5GC_Init(NR_1); f_NR_Set_RLF_TimersAndConstants_t311(nr_Cell1); //WA# // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-1 f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_5_9_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } //end of f_TC_8_1_5_9_1_NR5GC

## f_TC_8_1_5_9_1_NR5GC_TestBody

| Function name | f_TC_8_1_5_9_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - According to TS 38.323, the maximum supported size of a PDCP SDU is 9000 bytes. Hence, the lengthof of all received segments should be compared against 9000 bytes rather than 18432 bytes - According to 38.331, the received segments are part of the UL-DCCH-Message: rrc-MessageSegmentContainer Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough that the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit. Hence, the decode of the segments should be done for the UL-DCCH_Messaget Type rather than UE_NR_Capability - According to the Prose, at Step 5, SS should transmit periodic UL grants (80∙2μ+1+TDL) slots after step 4a3. This should be translated correctly in TTCN to be able to transmit UL grants every 10 Slots. - According to the Prose, SS has to wait for T310 expiry before switching on NR Cell 1 again. T310 is of duration 1s, hence f_Delay should be for 1s rather than 1000s |
| Summary of change | - Corrected the lengthof comparison against 9000 bytes - Used the corrected type to decode the received UL Segments, new function introduced for this. - Used the correct template to be able to transmit the UL grants every 10 slots as well corrected the function fl_CalculateTiming_8_1_5_9_1 to satisfy the (80∙2μ+1+TDL) slots after step 4a3 - Corrected the f_Delay parameter to use 1.0 rather than 1000.0 |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ <<SKIPPED CODE>> // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < 18432) { //<9K f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } f_NR_DecodeNrCapMsg(v_Rx_SegmentOct); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } <<SKIPPED CODE>> //@siclog "Step 4a4" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4a3 to schedule PUSCH till the ULDedicatedMessageSegment at step 4a5 is received. v_TimingInfoStep4a4 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4a3); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4a4), valueof (cs_NR_ResourceAllocationDef), 10); //periodic grant every 10ms <<SKIPPED CODE>> //@siclog "Step 4a6" siclog@ // The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. f_NR_SetCellPowerList (v_CellPowerList_AtT1); //@siclog "Step 4a7" siclog@ // Wait for T310 to expire f_Delay(1000.0); //@siclog "Step 4a8" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. f_NR_SetCellPowerList (v_CellPowerList_AtT2); <<SKIPPED CODE>>

After Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200933 R5-204456 R5-216276 sic@ <<SKIPPED CODE>> // Decode and store the UE-CapabilityRAT-Container if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments if (lengthof(v_Rx_SegmentOct) < 9000) { //<9K //WA# f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size"); } f_NR_DecodeULDcchMsg(v_Rx_SegmentOct); //WA# f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a1"); } <<SKIPPED CODE>> //@siclog "Step 4a4" siclog@ //The SS transmits periodic grant from (80*2microsec+1+TDL) slots after step 4a3 to schedule PUSCH till the ULDedicatedMessageSegment at step 4a5 is received. v_TimingInfoStep4a4 := fl_CalculateTiming_8_1_5_9_1(v_TimingInfoStep4a3); f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingInfoStep4a4), -, cs_NR_FreqDomainSchedulExplicitUL_Def, -, cs_UL_GrantConfig_ContPeriodic(10)); <<SKIPPED CODE>> //@siclog "Step 4a6" siclog@ // The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. f_NR_SetCellPowerList (v_CellPowerList_AtT1); //@siclog "Step 4a7" siclog@ // Wait for T310 to expire f_Delay(1.0); //@siclog "Step 4a8" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. f_NR_SetCellPowerList (v_CellPowerList_AtT2); <<SKIPPED CODE>>

New Function:

function f_NR_DecodeULDcchMsg(octetstring p_Octetstring) { var UL_DCCH_Message v_UL_Dcch_Msg; var bitstring v_Bitstring := oct2bit(p_Octetstring); var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE NR Capability cannot be decoded"); } }

## fl_CalculateTiming_8_1_5_9_1

| Function name | fl_CalculateTiming_8_1_5_9_1 |
| --- | --- |
| Reason for change | According to the Prose, at Step 5, SS should transmit periodic UL grants (80∙2μ+1+TDL) slots after step 4a3. This should be translated correctly in TTCN to be able to transmit UL grants every 10 Slots as well as calculating the TDL slots accurately |
| Summary of change | Corrected fl_CalculateTiming_8_1_5_9_1 to satisfy the (80∙2μ+1+TDL) slots after step 4a3 |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_CalculateTiming_8_1_5_9_1(SubFrameTiming_Type p_TimingInfoStep4a3) runs on NR_BASE_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_TimingInfo; //Step 4a4 var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var integer v_TimingSlots; var integer v_Timing2Slots; var integer v_TDL; v_TimingInfo := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingSlots := f_NR_ConvertTiming_To_TimingInSlots(p_TimingInfoStep4a3, v_SlotsNumberPerSubframe); select(v_SubcarrierSpacing) { case(kHz15) { if (not(f_NR_CellInfo_GetIsFDD(nr_Cell1))) { if (p_TimingInfoStep4a3.SFN.Number == 2) { v_TDL := 3; }else { v_TDL := 4; } } } case else { v_TDL := 1; } } v_Timing2Slots := v_TimingSlots + 80*v_SlotsNumberPerSubframe + 1 + v_TDL; v_TimingInfo := f_NR_ConvertTimingInSlots_To_Timing(v_Timing2Slots, v_SubcarrierSpacing, v_SlotsNumberPerSubframe); return v_TimingInfo; }

After Change:

function fl_CalculateTiming_8_1_5_9_1(SubFrameTiming_Type p_TimingInfoStep4a3) runs on NR_BASE_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_TimingInfo; //Step 4a4 var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var integer v_Nslots := 80*v_SlotsNumberPerSubframe + 1; var integer v_TimingSlots; var integer v_Timing2Slots; var integer v_TDL; //REMOVED v_TimingInfo := f_NR_GetNextSendOccasion_DL(nr_Cell1); WA# v_TimingSlots := f_NR_ConvertTiming_To_TimingInSlots(p_TimingInfoStep4a3, v_SlotsNumberPerSubframe); select(v_SubcarrierSpacing) { case(kHz15) { if (not(f_NR_CellInfo_GetIsFDD(nr_Cell1))) { if (p_TimingInfoStep4a3.SFN.Number == 2) { v_TDL := 3; }else { v_TDL := 4; } } } case else { v_TDL := (4 - (v_TimingSlots + v_Nslots) mod 10) mod 10; } } v_Timing2Slots := v_TimingSlots + v_Nslots + v_TDL; v_TimingInfo := f_NR_ConvertTimingInSlots_To_Timing(v_Timing2Slots, v_SubcarrierSpacing, v_SlotsNumberPerSubframe); return v_TimingInfo; }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NEA1/NIA1  algorithms.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_5_9_1_NR5GC_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220331: Supporting information for addition of NR5GC test case 8.1.5.9.1 |
| --- | --- |