3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220684

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2539 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test cases 8.1.5.8.2.x and 8.1.5.8.1 for FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these RRC Procedure Delay testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For FDD, set the PdschHarqTimingIndicator to “000”B so that K1 would be 2SF as per TS 38.501-1 Table 4.6.3-112 - For FDD, Set the variable v_K1 = 2 in the function f_NR5GC_RRC_ProcedureDelay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC, 8.1.5.8.2.3.NR5GC, 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for NR5GC test cases 8.1.5.8.2.x and 8.1.5.8.1	2

1.1	Correction to f_TC_8_1_5_8_2_X_NR5GC_TestBody	2

1.2	Correction to f_NR_ConfigPCell_AddRelSCells	5

1.3	Correction to f_NR5GC_RRC_ProcedureDelay	8

# Corrections required for NR5GC test cases 8.1.5.8.2.x and 8.1.5.8.1

## Correction to f_TC_8_1_5_8_2_X_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |
| Summary of change | For FDD, set the PdschHarqTimingIndicator to “000”B so that K1 would be 2SF as per TS 38.501-1 Table 4.6.3-112 |
| TTCN module | RRC_Others_CA_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> var integer i; var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ / @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_TimingInfo), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext)),-,-,crs_MacBearerRouting(p_PCell)), car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ var template (value) NR_DciFormat_1_X_SpecificInfo_Type v_DciFormatAndParams; //WA# var NR_PhysicalParameters_Type v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCell); //WA if (f_NR_CellInfo_GetIsFDD(p_PCell)){ v_DciFormatAndParams := cds_NR_DciFormat_1_1_Params_81582x(v_PhysicalParameters.SubcarrierSpacing); f_NR_SS_CommonCellConfig(p_PCell, cas_NR_DciDlInfo_REQ(p_PCell, cs_TimingInfo_Now, cs_NR_DciDlInfo(-, -, v_DciFormatAndParams))); } // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_TimingInfo), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext)),-,-,crs_MacBearerRouting(p_PCell)), car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>>

##### New Template:

| Template name | cds_NR_DciFormat_1_1_Params_81582x |
| --- | --- |
| Reason for change | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |
| Summary of change | For FDD, set the PdschHarqTimingIndicator to “000”B so that K1 would be 2SF as per TS 38.501-1 Table 4.6.3-112 |
| TTCN module | NR_DCI_Configuration |
| MCC160 Comment |  |

template (value) NR_DciFormat_1_X_SpecificInfo_Type cds_NR_DciFormat_1_1_Params_81582x(SubcarrierSpacing p_SCS) modifies cs_NR_DciFormat_1_1_Params := { /* @status APPROVED (ENDC, NR5GC) */ Format_1_1 := { PdschHarqTimingIndicator := cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('000'B) } };

## Correction to f_NR_ConfigPCell_AddRelSCells

| Function name | f_NR_ConfigPCell_AddRelSCells |
| --- | --- |
| Reason for change | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |
| Summary of change | For FDD, set the PdschHarqTimingIndicator to “000”B so that K1 would be 2SF as per TS 38.501-1 Table 4.6.3-112 |
| TTCN module | NR_CommonFunctions_CA |
| MCC160 Comment |  |

Before Change:

function f_NR_ConfigPCell_AddRelSCells(NR_CellId_Type p_PCellId, NR_CellIdList_Type p_SCellList, NR_ServingCellIndex_Type p_PServingCellIndex, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value1, template (omit) NR_RadioBearerList_Type p_SS_RadioBearerList := omit, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) runs on NR_BASE_PTC { //@sic R5-206369: added p_CnfFlag and p_FollowOnFlag sic@ var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_PCellId); //@sic R5s201387 sic@ var integer i; var template (value) NR_DciFormat_1_X_SpecificInfo_Type v_DciFormatAndParams; //@sic R5-202677, R5-201572 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCellId); if (lengthof(p_SCellList) == 0) { // No SCell - reconfigure DAI as single serving cell v_DciFormatAndParams := cs_NR_DciFormat_1_1_Params(v_PhysicalParameters.SubcarrierSpacing); } else { // SCell added - reconfigure DAI as multiple serving cell - B4 type v_DciFormatAndParams := cds_NR_DciFormat_1_1_Params_DAI_MultipleServingCells(v_PhysicalParameters.SubcarrierSpacing) } <<SKIPPED CODE>>

After Change:

function f_NR_ConfigPCell_AddRelSCells(NR_CellId_Type p_PCellId, NR_CellIdList_Type p_SCellList, NR_ServingCellIndex_Type p_PServingCellIndex, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value1, template (omit) NR_RadioBearerList_Type p_SS_RadioBearerList := omit, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) runs on NR_BASE_PTC { //@sic R5-206369: added p_CnfFlag and p_FollowOnFlag sic@ var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_PCellId); //@sic R5s201387 sic@ var integer i; var template (value) NR_DciFormat_1_X_SpecificInfo_Type v_DciFormatAndParams; //@sic R5-202677, R5-201572 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCellId); if (lengthof(p_SCellList) == 0) { // No SCell - reconfigure DAI as single serving cell v_DciFormatAndParams := cs_NR_DciFormat_1_1_Params(v_PhysicalParameters.SubcarrierSpacing); } else { if (f_NR_CellInfo_GetIsFDD(p_PCellId)){ v_DciFormatAndParams := cds_NR_DciFormat_1_1_Params_DAI_MultipleServingCells_81582x(v_PhysicalParameters.SubcarrierSpacing); //WA# } else { // SCell added - reconfigure DAI as multiple serving cell - B4 type v_DciFormatAndParams := cds_NR_DciFormat_1_1_Params_DAI_MultipleServingCells(v_PhysicalParameters.SubcarrierSpacing) } }

##### New Template:

| Template name | cds_NR_DciFormat_1_1_Params_DAI_MultipleServingCells_81582x |
| --- | --- |
| Reason for change | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |
| Summary of change | For FDD, set the PdschHarqTimingIndicator to “000”B so that K1 would be 2SF as per TS 38.501-1 Table 4.6.3-112 |
| TTCN module | NR_DCI_Configuration |
| MCC160 Comment |  |

template (value) NR_DciFormat_1_X_SpecificInfo_Type cds_NR_DciFormat_1_1_Params_DAI_MultipleServingCells_81582x(SubcarrierSpacing p_SCS) modifies cs_NR_DciFormat_1_1_Params := { /* @status APPROVED (ENDC, NR5GC) */ Format_1_1 := { DAI := cs_NR_DciFormat_1_1_DAI_MultipleServingCells(cs_DAI_B4_Automatic), /* 4 bits for multiple serving cells */ PdschHarqTimingIndicator := cs_NR_DciFormat_1_X_PdschHarqTimingIndicator('000'B) } };

## Correction to f_NR5GC_RRC_ProcedureDelay

| Function name | f_NR5GC_RRC_ProcedureDelay |
| --- | --- |
| Reason for change | For the FDD bands with SCS 15 Khz, it would not be possible to receive the UL HARQ ACK before the UL Signalling message as per the requirements of these testcases to calculate T4Slots and T2Slots as K2 is 4SFs and K1 is 7SFs. In function f_NR5GC_RRC_ProcedureDelay, the implementation expects to receive UL HARQ ACK first to set the v_T2Flag to TRUE and if this is not the case, TC would fail with cause as "UL ACK not received" |
| Summary of change | For FDD, Set the variable v_K1 = 2 in the function f_NR5GC_RRC_ProcedureDelay |
| TTCN module | NR5GC_RRCSteps |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { <<SKIPPED CODE>> //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } <<SKIPPED CODE>>

After Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { <<SKIPPED CODE>> //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 2; //WA# } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } <<SKIPPED CODE>>