3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220691

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2543 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.3.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.523-1 Table 8.1.3.3.2.3.3-4 and TS 38.508-1 Table 4.6.3-141, Hysteresis and timeToTrigger for event B2 is not correct in current TTCN implementation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set Hysteresis to 3 and timeToTrigger to ms1024 when setup inter-RAT measurement and reporting for event B2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1 Corrections required	2

1.1 Correction to f_TC_8_1_3_3_2_TestBody	2

- Corrections required

Note: Change2 in R5s220690 is also applied for this test case.

  - Correction to f_TC_8_1_3_3_2_TestBody

| Function name | f_TC_8_1_3_3_2_TestBody |
| --- | --- |
| Reason for change | - According to TS38.523-1 Table 8.1.3.3.2.3.3-4 and TS 38.508-1 Table 4.6.3-141, Hysteresis and timeToTrigger for event B2 is not correct in current TTCN implementation |
| Summary of change | Set Hysteresis to 3 when setup inter-RAT measurement and reporting for event B2. |
| TTCN module | RRC_Measurement_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_3_3_2_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -85; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 0; //@sic R5-218335, R5s200713 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 77; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 56; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var PLMN_Identity v_PLMN_Identity; var B16_Type v_TrackingAreaCode_Eutra_Epc; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (present) CGI_InfoEUTRA v_NR_MeasResultEUTRA_CGI_InfoEUTRA; var template (value) CellAccessRelatedInfo_EUTRA_EPC v_CellAccessRelatedInfo_EUTRA_EPC; var template (value) FreqBandIndicatorEUTRA v_FreqBandIndicatorEUTRA; ... }

After Change:

function f_TC_8_1_3_3_2_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -85; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 3; //@sic R5-218335, R5s200713 sic@ var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1; var RSRP_Range v_B2_Threshold1 := 77; var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 56; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var PLMN_Identity v_PLMN_Identity; var B16_Type v_TrackingAreaCode_Eutra_Epc; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (present) CGI_InfoEUTRA v_NR_MeasResultEUTRA_CGI_InfoEUTRA; var template (value) CellAccessRelatedInfo_EUTRA_EPC v_CellAccessRelatedInfo_EUTRA_EPC; var template (value) FreqBandIndicatorEUTRA v_FreqBandIndicatorEUTRA; ... }