3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220760

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0833 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of 4G/5G session timer test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-06-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 34.229-1 A.2.5 in an UPDATE sent by the SS within an MO call the Contact header field shall be the same as “sent by SS in this dialog (e.g. in a 183 or 180 response)”; nevertheless in TTCN the Contact parameters are set to omit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TTCN modified so that the UPDATE’s Contact header field contains the same Contact parameters as at call establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN and prose remain inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1, 22.2, 22.3, 22.4, 22.5, 7.27.NR5GC, 7.28.NR5GC, 7.29.NR5GC, 7.30.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Change 1

| Function name | f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec |
| --- | --- |
| Reason for change | Function needs Contact header field as parameter instead of Contact URI |
| Summary of change | Contact header field as parameter instead of Contact URI |
| TTCN module | IMS_CC_SessionTimerCommon |

function f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(INVITE_Request p_InviteRequest, template (value) IMS_RoutingInfo_Type p_RoutingInfo_DL, template (omit) Contact p_Contact := omit, SipUrl p_ContactUrlUE, integer p_DeltaSec, charstring p_RefresherValue) runs on IMS_PTC return template (value) MessageHeader { /* p_ContactUrlSS needs to be provided only in case of MO Call; in case of MT call the whole Contact header is included in p_InviteRequest */ var template (omit) MessageBody v_MessageBody := omit; var template (value) MessageHeader v_MessageHeader_Update; v_MessageHeader_Update := f_IMS_UpdateRequest_MessageHeaderTX(p_InviteRequest, NoPreconditionIndication, p_Contact); v_MessageHeader_Update.supported := cs_Supported_timer; v_MessageHeader_Update.sessionExpires := cs_SessionExpires(p_DeltaSec, p_RefresherValue); v_MessageHeader_Update.contentType := omit; // @sic R5-198895 sic@ f_Delay(int2float(p_DeltaSec / 2)); IMS_Client.send(cas_IMS_Update_Request(p_RoutingInfo_DL, cs_UPDATE_Request(p_ContactUrlUE, v_MessageHeader_Update, v_MessageBody))); return v_MessageHeader_Update; }

## 2.2 Change 2

| Function name | f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_10 |
| --- | --- |
| Reason for change | Functions needs to return Contact rather than Contact URI |
| Summary of change | return Contact instead of Contact URI |
| TTCN module | IMS_CC_CommonFunctions5G |

function f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_10(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (omit) Allow p_Allow := omit) runs on IMS_PTC return template (omit) Contact { var MessageHeader v_MessageHeader183; v_MessageHeader183 := f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_5(p_InviteRequestWithSdp, -, p_Allow); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); // to trigger step 13 of 38.508-1 Table 4.9.15.2.2-1 f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7(p_InviteRequestWithSdp); f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_10(p_InviteRequestWithSdp); return v_MessageHeader183.contact; }

## 2.3 Change 3

| Function name | f_IMS_MOCallSetup_AnnexC21_Steps3_11 |
| --- | --- |
| Reason for change | Functions needs to return Contact rather than Contact URI |
| Summary of change | return Contact instead of Contact URI |
| TTCN module | IMS_SessionTimerTestcases |

function f_IMS_MOCallSetup_AnnexC21_Steps3_11(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (omit) Allow p_Allow := omit) runs on IMS_PTC return Contact { var template (value) MessageHeader v_MessageHeader_Response183; // Step 3. SS sends a 100 Trying provisional response. f_IMS_MOCallSetup_Send100Trying(p_InviteRequestWithSdp); // Step 4. Send 183 Session In Progress. v_MessageHeader_Response183 := f_IMS_InviteResponse_183_MessageHeaderTX(p_InviteRequestWithSdp.Invite); v_MessageHeader_Response183.allow := p_Allow; /* @sic R5-201447 sic@ */ f_IMS_MOCallSetup_AnnexC21_Step4(p_InviteRequestWithSdp, v_MessageHeader_Response183); // See TS. 36.508 EUTRA/EPS signalling for IMS MO speech call Table 4.5A.6.3-1 Step 9-11 f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); /* trigger IPCAN to continue with activation of secondary PDP context or dedicated EPS bearer */ // Step 5 - 8 f_IMS_MOCallSetup_AnnexC21_Steps5_8(p_InviteRequestWithSdp, v_MessageHeader_Response183); // Step 9 - 11 f_IMS_MOCallSetup_AnnexC21C25_Step9_11(p_InviteRequestWithSdp); return v_MessageHeader_Response183.contact; }

## 2.4 Change 4

| Function name | f_TC_7_27_NR5GC_IMS1, f_TC_7_28_NR5GC_IMS1, f_TC_7_29_NR5GC_IMS1, f_TC_7_30_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Change 1, 2 |
| Summary of change | Necessary changes done to cope with change 1 and 2: f_TC_7_30_NR5GC_IMS1 is shown as example |
| TTCN module | IMS_NR5GC_CallControlTestcases |

function f_TC_7_30_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MO Voice Call / Remote end supports but does not use Session Timer / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var INVITE_Request v_InviteRequest; var MessageHeader v_MessageHeaderInvite; var template (omit) Contact v_ContactSS; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" IPCAN siclog@ // @siclog "Step 8" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_MessageHeaderInvite := v_InviteRequest.msgHeader; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); if (ispresent(v_MessageHeaderInvite.sessionExpires)) { f_IMS_CallWithSessionTimer_CheckSessionExpires(v_MessageHeaderInvite.sessionExpires, v_DeltaSec, "uac" ifpresent); } f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Steps 9 - 17" siclog@ v_ContactSS := f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_10(v_InviteRequestWithSdp); v_ContactUrlSS := f_Contact_GetSipUrl(v_ContactSS); // @siclog "Step 18" siclog@ f_IMS_CallWithSessionTimer_Send200OK(v_MessageHeaderInvite, v_RoutingInfo_DL, -, -, cs_Supported_timer); // @siclog "Step 19" siclog@ f_IMS_CallWithSessionTimer_ReceiveACK(v_InviteRequest, v_ContactUrlSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Step 20 - 21" siclog@ f_Delay(int2float(1860)); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); }

## 2.5 Change 5

| Function name | f_TC_22_1_IMS1, f_TC_22_2_IMS1, f_TC_22_3_IMS1, f_TC_22_4_IMS1, f_TC_22_5_IMS1 |
| --- | --- |
| Reason for change | Change 1, 3 |
| Summary of change | Necessary changes done to cope with change 1 and 3: f_TC_22_5_IMS1 is shown as example |
| TTCN module | IMS_SessionTimerTestcases |

function f_TC_22_5_IMS1() runs on IMS_PTC { var IMS_CC_MT_SpeechCallInfo_Type v_SpeechCallInfo; var INVITE_Request v_InviteRequest; var Contact v_ContactSS; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Steps 1 - 11A" siclog@ v_SpeechCallInfo := f_IMS_MTCallSetupWithSessionTimer_Steps1_11A(); v_InviteRequest := v_SpeechCallInfo.InviteRequestWithSdp.Invite; v_ContactSS := v_InviteRequest.msgHeader.contact; v_ContactUrlSS := f_Contact_GetSipUrl(v_ContactSS); v_ContactUrlUE := v_SpeechCallInfo.ContactUrl; v_RoutingInfo_DL := v_SpeechCallInfo.InviteRequestWithSdp.RoutingInfo; // @siclog "Steps 12 - 13" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(v_SpeechCallInfo.InviteRequestWithSdp, cr_Require_timer, -, v_DeltaSec, "uac"); // @siclog "Step 14" siclog@ v_MessageHeaderUpdate := f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(v_InviteRequest, v_RoutingInfo_DL, v_ContactSS, v_ContactUrlUE, v_DeltaSec, "uac"); // @siclog "Step 15" siclog@ f_IMS_CallWithSessionTimer_Receive200OK(v_RoutingInfo_DL.Protocol, v_MessageHeaderUpdate, -, cr_Supported_timer, v_DeltaSec, "uac"); // @siclog "Steps 16 - 19" siclog@ f_IMS_CallWithSessionTimer_MOCallRelease(v_InviteRequest, v_ContactUrlSS, v_DeltaSec); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); }