3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220777

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2594 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current TTCN implementation for steps 1D and 2 is getting DRB information for the PDU session X using function f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(). Pre-test conditions state “the UE is in 5GS state 3N-A with at least one PDU session (with PDU session ID X where 1 <= X <= 15) active”. The PDU session is not restricted to be non-IMS so it should be allowed that the UE has established an IMS PDU session only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the function f_NR_GetDefaultDRB_ForFirstPDUSession() at steps 1D and 2, to allow for a UE operation which only creates an IMS PDU session during preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | Current TTCN implementation for steps 1D and 2 is getting DRB information for the PDU session X using function f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(). Pre-test conditions state “the UE is in 5GS state 3N-A with at least one PDU session (with PDU session ID X where 1 <= X <= 15) active”. The PDU session is not restricted to be non-IMS so it should be allowed that the UE has established an IMS PDU session only. |
| Summary of change | Use the function f_NR_GetDefaultDRB_ForFirstPDUSession() at steps 1D and 2, to allow for a UE operation which only creates an IMS PDU session during preamble. |
| TTCN module | Service_Request_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) QosFlow_Identification_Type v_QosFlowId; var DRB_ToReleaseList v_DRBToReleaseList; /**** Skiping code ***********/ //@siclog "Steps 1D - 1E" siclog@ //@sic R5-202661 sic@ f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '01'O); //@sic R5s200716 sic@ f_IP_Routing_ChangeToRawmode(IP); //@sic R5s200716 sic@ v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession()); v_DRBToReleaseList := {f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession()}; //Get the first non-IMS PDU session ID v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, -, v_DRBToReleaseList); v_PSI := valueof(v_QosFlowId.PDU_SessionId); /**** Skiping code ***********/ //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCReconfiguration message to release User-plane resources for the PDU session. //@sic R5s201129 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig); v_RlcToReleaseList := {f_NR_LogicalChannelId_DRB(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession())}; //@sic R5s200716 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , //@sic R5s200716 sic@ omit, v_RlcToReleaseList, omit, omit, omit); /**** Skiping code ***********/ }

After Change:

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) QosFlow_Identification_Type v_QosFlowId; var DRB_ToReleaseList v_DRBToReleaseList; /**** Skiping code ***********/ //@siclog "Steps 1D - 1E" siclog@ //@sic R5-202661 sic@ f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '01'O); //@sic R5s200716 sic@ f_IP_Routing_ChangeToRawmode(IP); //@sic R5s200716 sic@ v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRB(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_DRBToReleaseList := {f_NR_GetDefaultDRB_ForFirstPDUSession()}; //Get the first non-IMS PDU session ID v_RadioBearerC:= cs_38508_RadioBearerConfigDef(-, -, v_DRBToReleaseList); /**** Skiping code ***********/ //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCReconfiguration message to release User-plane resources for the PDU session. //@sic R5s201129 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig); v_RlcToReleaseList := {f_NR_LogicalChannelId_DRB(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession())}; //@sic R5s200716 sic@ v_RlcToReleaseList := {f_NR_LogicalChannelId_DRB(f_NR_GetDefaultDRB_ForFirstPDUSession())}; //@sic R5s200716 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , //@sic R5s200716 sic@ omit, v_RlcToReleaseList, omit, omit, omit); /**** Skiping code ***********/ }