3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220795

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2609 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the Steps listed in TS 38.523-3 cl 7.3.5.3.2 to effect the IntraNR IntraCell HO, the TA transmission is already disabled at Step 8 of this procedure. Hence, it is not needed to be stopped again after Step 8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the redundant call to function f_NR_ULGrantConfiguration_Start |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This was raised in R5s220687 Change 1.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcase 7.1.1.9.1	2

2.1	Correction to fl_TC_7_1_1_9_1_NR_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.9.1

## Correction to fl_TC_7_1_1_9_1_NR_TestBody

| Function name | fl_TC_7_1_1_9_1_NR_TestBody() |
| --- | --- |
| Reason for change | As part of the Steps listed in TS 38.523-3 cl 7.3.5.3.2 to effect the IntraNR IntraCell HO, the TA transmission is already disabled at Step 8 of this procedure. Hence, it is not needed to be stopped again after Step 8. |
| Summary of change | Removed the call to function f_NR_ULGrantConfiguration_Start. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ //Initialise CellGroupConfig before sending data to UE to avoid additional delay v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList); //@sic R5s210138 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit), v_SS_DRB_Config, true, v_CellGroupConfig);//@sic R5-213132 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ <<SKIPPED CODE>>

After Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ //Initialise CellGroupConfig before sending data to UE to avoid additional delay v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList); //@sic R5s210138 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, -, -, omit), v_SS_DRB_Config, true, v_CellGroupConfig);//@sic R5-213132 sic@ //REMOVED f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ WA# <<SKIPPED CODE>>