3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220869

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0075 | rev | - | Current version: | 14.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCX Implementation Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R5w220205 reports several issues and inconsistencies of prose and TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementation of proposals according to R5w220205. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN and prose remain inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MCX test suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Change 1

| Function name | f_MCX_InviteRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-1: TTCN is not according to 36.579-1 Table 5.5.2.5.1-1 |
| Summary of change | R5w220205 Proposal 2.3-1: superset removed from the accept header field |
| TTCN module | MCX_SIP_CallControl_Common |

function f_MCX_InviteRequest_MessageHeaderRX(MCX_ServiceType_Type p_ServiceType, MCX_SIP_EstabMod_Type p_EstabModType := CALL_ESTAB_MOD, MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, MCX_CommencementMode_Type p_CommencementMode := NONE, template (present) SipUrl p_ExpectedCalleeUri, boolean p_IsReInvite := false) runs on IMS_PTC return template (present) MessageHeader { … v_MessageHeader.supported := cr_Supported(tsc_OptionTagList_timer); v_MessageHeader.sessionExpires := cr_SessionExpires; v_MessageHeader.require := cr_RequireSecAgree; v_MessageHeader.accept := cr_Accept(v_AcceptBodyList); v_MessageHeader.acceptContact := cr_AcceptContact_Common(f_MCX_AcceptContact_AcValues_RX(p_ServiceType)); … }

## 2.2 Change 2

| Function name | f_MCPTT_Group_Extensions_mcpttgi, f_MCVideo_Group_Extensions_mcpttgi and f_MCData_Group_Extensions_mcpttgi |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-2: level-within-group-hierarchy and level-within-user-hierarchy missing in MCPTT/MCVideo/MCPTT Group Configuration |
| Summary of change | R5w220205 Proposal 2.3-2: level-within-group-hierarchy and level-within-user-hierarchy added to f_MCPTT_Group_Extensions_mcpttgi, f_MCVideo_Group_Extensions_mcpttgi and f_MCData_Group_Extensions_mcpttgi Change shown below for f_MCPTT_Group_Extensions_mcpttgi, the other functions are changed in the same way |
| TTCN module | MCX_XML_GroupConfiguration_MCPTT |

function f_MCPTT_Group_Extensions_mcpttgi(urn_3gpp_ns_mcpttGroupInfo_1_0.Owner p_Owner, XSD.String p_PreferredVoiceCodec, urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_layer_2_group_id p_Off_network_ProSe_layer_2_group_id) return MCX_PocListService_ListService_Type.elem_list { var template (value) MCX_GroupInfo_Encodings_Type v_Preferred_voice_encodings := cs_MCX_GroupInfo_Encodings(p_PreferredVoiceCodec); var urn_3gpp_ns_mcpttGroupInfo_1_0.Level_within_group_hierarchy v_Level_within_group_hierarchy := 0; var urn_3gpp_ns_mcpttGroupInfo_1_0.Level_within_user_hierarchy v_Level_within_user_hierarchy := 0; var urn_3gpp_ns_mcpttGroupInfo_1_0.Level_within_group_hierarchy v_Level_within_group_hierarchy := 0; var urn_3gpp_ns_mcpttGroupInfo_1_0.Level_within_user_hierarchy v_Level_within_user_hierarchy := 0; var urn_3gpp_ns_mcpttGroupInfo_1_0.Protect_media v_Protect_media := tsc_MCX_XML_ConfidentialityProtection; var urn_3gpp_ns_mcpttGroupInfo_1_0.Protect_floor_control_signalling v_Protect_floor_control_signalling := tsc_MCX_XML_ConfidentialityProtection; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_IP_multicast_address v_Off_network_IP_multicast_address := "0.0.0.0"; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_relay_service_code v_Off_network_ProSe_relay_service_code := char2oct("123456"); var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_in_progress_emergency_state_cancellation_timeout v_Off_network_in_progress_emergency_state_cancellation_timeout := "PT18H12M15S"; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_in_progress_imminent_peril_state_cancellation_timeout v_Off_network_in_progress_imminent_peril_state_cancellation_timeout := "PT18H12M15S"; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_hang_timer v_Off_network_hang_timer := "PT5S"; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_maximum_duration v_Off_network_maximum_duration := "PT1M"; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_queue_usage v_Off_network_queue_usage := true; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_signalling_PPPP v_Off_network_ProSe_signalling_PPPP := '01'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_media_PPPP v_Off_network_ProSe_media_PPPP := '01'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_emergency_call_signalling_PPPP v_Off_network_ProSe_emergency_call_signalling_PPPP := '08'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_emergency_call_media_PPPP v_Off_network_ProSe_emergency_call_media_PPPP := '08'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_imminent_peril_call_signalling_PPPP v_Off_network_ProSe_imminent_peril_call_signalling_PPPP := '07'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Off_network_ProSe_imminent_peril_call_media_PPPP v_Off_network_ProSe_imminent_peril_call_media_PPPP := '07'O; var urn_3gpp_ns_mcpttGroupInfo_1_0.Multi_talker_control v_Multi_talker_control := false; var urn_3gpp_ns_mcpttGroupInfo_1_0.Max_number_simultaneous_talkers v_Max_number_simultaneous_talkers := 1; var MCX_PocListService_ListService_Type.elem_list v_Extension; if (f_GetTestcaseAttrib_Multi_talker(testcasename())) { // test case 6.1.1.20 v_Multi_talker_control := true; v_Max_number_simultaneous_talkers := 2; } v_Extension := { f_Bit2XsdString(encvalue(p_Owner)), f_Bit2XsdString(encvalue(v_Preferred_voice_encodings)), f_Bit2XsdString(encvalue(v_Level_within_group_hierarchy)), f_Bit2XsdString(encvalue(v_Level_within_user_hierarchy)), f_Bit2XsdString(encvalue(v_Protect_media)), f_Bit2XsdString(encvalue(v_Protect_floor_control_signalling)), f_Bit2XsdString(encvalue(p_Off_network_ProSe_layer_2_group_id)), f_Bit2XsdString(encvalue(v_Off_network_IP_multicast_address)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_relay_service_code)), f_Bit2XsdString(encvalue(v_Off_network_in_progress_emergency_state_cancellation_timeout)), f_Bit2XsdString(encvalue(v_Off_network_in_progress_imminent_peril_state_cancellation_timeout)), f_Bit2XsdString(encvalue(v_Off_network_hang_timer)), f_Bit2XsdString(encvalue(v_Off_network_maximum_duration)), f_Bit2XsdString(encvalue(v_Off_network_queue_usage)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_signalling_PPPP)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_media_PPPP)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_emergency_call_signalling_PPPP)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_emergency_call_media_PPPP)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_imminent_peril_call_signalling_PPPP)), f_Bit2XsdString(encvalue(v_Off_network_ProSe_imminent_peril_call_media_PPPP)), f_Bit2XsdString(encvalue(v_Multi_talker_control)), // @sic R5-213266 sic@ f_Bit2XsdString(encvalue(v_Max_number_simultaneous_talkers)) // @sic R5-213266 sic@ }; return v_Extension; }

## 2.3 Change 3

| Function name | fl_MCX_BuildSDP_TX |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-3: TTCN is not according to 36.579-1 Tables 5.5.3.1.2-1/2 |
| Summary of change | R5w220205 Proposal 2.3-3: c=line removed from session level in fl_MCX_BuildSDP_TX |
| TTCN module | MCX_SDP_MCPTT_MCVideo |

function fl_MCX_BuildSDP_TX(MCX_ServiceType_Type p_ServiceType, template (value) SDP_Origin p_Origin, template (value) SDP_media_desc p_MediaDescriptionAudio, template (omit) SDP_media_desc p_MediaDescriptionVideo, template (omit) SDP_media_desc p_MediaDescriptionMediaControl, boolean p_PRE_ESTABLISHED_SESSION) return template (value) SDP_Message { var charstring v_Sessionname := " "; // single empty space indicating no session name var template (value) SDP_connection v_Connection := cs_SDP_Connection(p_Origin.addr_type, p_Origin.addr); var template (value) SDP_bandwidth_list v_SessionBandwidthList; var template (value) SDP_media_desc_list v_SDP_Media_Desc_List; var template (value) SDP_Message v_SDP_Message; var integer i := 0; select (p_ServiceType) { case (MCPTT) { v_SessionBandwidthList := cs_SDP_Bandwidth_List_AS(38); } case (MCVideo) { v_SessionBandwidthList := cs_SDP_Bandwidth_List_AS(352); } } v_SDP_Media_Desc_List[0] := p_MediaDescriptionAudio; i := i + 1; if (isvalue(p_MediaDescriptionVideo)) { v_SDP_Media_Desc_List[i] := valueof(p_MediaDescriptionVideo); i := i + 1; } if (isvalue(p_MediaDescriptionMediaControl)) { v_SDP_Media_Desc_List[i] := valueof(p_MediaDescriptionMediaControl); i := i + 1; } v_SDP_Message := cs_SDP_Message_Common(p_Origin, v_Sessionname, -, v_SessionBandwidthList, -, v_SDP_Media_Desc_List); v_SDP_Message.attributes := f_SDP_Attributes_Add_TX(v_SDP_Message.attributes, crs_SDP_Attribute_ice_lite, p_PRE_ESTABLISHED_SESSION); // "a=ice-lite" session level attribute added for condition PRE_ESTABLISHED_SESSION return v_SDP_Message; }

## 2.4 Change 4

| Function name | f_MCX_Service_Info_Affiliation |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-4: f_MCX_Service_Info_Affiliation allows optional mcptt-called-party-id in the mcptt-info for affiliation, which is not according to the prose or to the core specs |
| Summary of change | R5w220205 Proposal 2.3-4: mcptt-called-party-id removed from mcptt-info in f_MCX_Service_Info_Affiliation |
| TTCN module | MCX_SIP_Affiliation |

function f_MCX_Service_Info_Affiliation(MCX_ServiceType_Type p_ServiceType, XSD.AnyURI p_TargetUserURI) return template (present) MCX_ServiceInfo_Type { var template (present) MCX_ServiceInfo_Type v_ServiceInfo := cs_MCX_Service_Info_Init; select (p_ServiceType) { case (MCPTT) { v_ServiceInfo.Mcptt := cr_MCPTT_Info_Base; v_ServiceInfo.Mcptt.mcptt_Params.mcptt_request_uri := cr_MCPTT_Content_URI(p_TargetUserURI); v_ServiceInfo.Mcptt.mcptt_Params.mcptt_calling_user_id := cr_MCPTT_Content_URI(px_MCPTT_ID_User_A) ifpresent; } case (MCVideo) { v_ServiceInfo.Mcvideo := cr_MCVideo_Info_Base; v_ServiceInfo.Mcvideo.mcvideo_Params.mcvideo_request_uri := cr_MCVideo_Content_URI(p_TargetUserURI); v_ServiceInfo.Mcvideo.mcvideo_Params.mcvideo_calling_user_id := cr_MCVideo_Content_URI(px_MCVideo_ID_User_A) ifpresent; } case (MCData) { v_ServiceInfo.Mcdata := cr_MCData_Info_Base; v_ServiceInfo.Mcdata.mcdata_Params.mcdata_request_uri := cr_MCData_Content_URI(p_TargetUserURI); v_ServiceInfo.Mcdata.mcdata_Params.mcdata_calling_user_id := cr_MCData_Content_URI(px_MCData_ID_User_A) ifpresent; } } return v_ServiceInfo; }

## 2.5 Change 5

| Function name | fl_MCX_IMS_GetNextDialogIndex |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-5: fl_MCX_IMS_GetNextDialogIndex has a workaround to avoid usage of the relatively new TTCN-3 build-in function int2enum |
| Summary of change | R5w220205 Proposal 2.3-5: int2enum used by fl_MCX_IMS_GetNextDialogIndex |
| TTCN module | MCX_SIP_Registration |

function fl_MCX_IMS_GetNextDialogIndex(IMS_PTC_DialogIndex_Type p_DialogIndex) return IMS_PTC_DialogIndex_Type { var IMS_PTC_DialogIndex_Type v_DialogIndex; int2enum(enum2int(p_DialogIndex) + 1, v_DialogIndex); return v_DialogIndex; }

## 2.6 Change 6

| Function name | a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-6: Template for Request-URI of the SUBSCRIBE for CONFIG and GROUPCONFIG shall not have matching pattern |
| Summary of change | R5w220205 Proposal 2.3-6b: cr_SipUri_HostPort replaced by f_SIP_ParseURI NOTE: Due to implementation of R5w220111 and the associated prose CR there will be further changes regarding handling of Request and Contact URIs |
| TTCN module | MCX_SIP_Registration |

altstep a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK(out MCX_DataRequestWithXmlBodies_Type p_DataRequestWithXmlBodies, MCX_ServiceType_Type p_ServiceType, MCX_Subscription_Type p_Subscription := INITIAL_SUBSCRIBE, IMS_PTC_DialogIndex_Type p_DialogIndex, charstring p_ServerName, template (present) ResourceLists_Type p_ResourceListsTemplate) runs on MCX_IMS_PTC { … var template (present) SipUrl v_RequestURI := f_SIP_ParseURI("sip:" & p_ServerName); … }

## 2.7 Change 7

| Function name | f_MCX_InviteRequest_MessageHeaderTX, f_MCX_CallSetupMO_Send18xResponse, f_MCX_SIP_LocationReportingConfiguration, f_MCData_MessageRequest_MessageHeaderTX, f_MCPTT_LocationInfo_SendMESSAGE_Receive200OK |
| --- | --- |
| Reason for change | R5w220205 Issue 2.3-9: Inconsistency between Prose and TTCN |
| Summary of change | R5w220205 Proposal 2.3-9: f_SIP_BuildSipUri_lr_TX replaced by f_SIP_BuildSipUri_TX; Change shown below for f_MCX_InviteRequest_MessageHeaderTX, the other functions are changed in the same way |
| TTCN module | MCX_SIP_CallControl_Common, MCX_SIP_CallControl_MCPTT_MCVideo, MCX_SIP_Registration, MCData_CommonProcedures, MCPTT_Location_MCX_IMS |

function f_MCX_InviteRequest_MessageHeaderTX(MCX_ServiceType_Type p_ServiceType, MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, MCX_CommencementMode_Type p_CommencementMode := NONE, template (omit) Contact p_Contact := omit, template (omit) ContentType p_ContentType, boolean p_IsReInvite := false) runs on IMS_PTC return template (value) MessageHeader { … v_MessageHeader_Invite.sessionExpires := cs_SessionExpires(1800); v_MessageHeader_Invite.pEarlyMedia := cs_PEarlyMedia({"inactive"}); v_MessageHeader_Invite.require := cs_Require(tsc_OptionTagList_sec_agree); v_MessageHeader_Invite.proxyRequire := cs_ProxyRequire(tsc_OptionTagList_sec_agree); v_MessageHeader_Invite.pAssertedID := cs_PAssertedId(f_SIP_BuildSipUri_TX(v_CallerSipAddrStr)); v_MessageHeader_Invite.contact := v_Contact; v_MessageHeader_Invite.accept := cs_Accept({cs_AcceptBody(c_sdpApplication), cs_AcceptBody(f_MCX_MimeType_mcs_info(p_ServiceType))}); v_MessageHeader_Invite.acceptContact := cs_AcceptContact_Common(f_MCX_AcceptContact_AcValues_Common_TX(v_AcceptContactParams)); v_MessageHeader_Invite.resourcePriority := v_ResourcePriority; v_MessageHeader_Invite.contentType := p_ContentType; … }