3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220985

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4685 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE emergency call testcase 11.2.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The functions f_IMS_EmergencyRegistration & f_IMS_EmergencyCallSetup (running in parallel) makes use of the same altstep (a_IMS_ReceiveRequest) to receive Subsequent REGISTER and INVITE respectively. Hence leading the TTCN to receive either one but not both. This may cause Runtime error when UE performs optional emergency registration during Step14-14L. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Both REGISTER & INVITE are received by same receive event and later checked for the content to perform either emergency registration or emergency call accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_11_2_7_IMS2 |
| --- | --- |
| Reason for change | The functions f_IMS_EmergencyRegistration & f_IMS_EmergencyCallSetup (running in parallel) makes use of the same altstep (a_IMS_ReceiveRequest) to receive Subsequent REGISTER and INVITE respectively. Hence leading the TTCN to receive either one but not both. This may cause Runtime error when UE performs optional emergency registration during Step14-14L. |
| Summary of change | Both REGISTER & INVITE are received by same receive event and later checked for the content to perform either emergency registration or emergency call accordingly. |
| TTCN module | IMS_EmergencyCall_IMS.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_11_2_7_IMS2() runs on IMS_PTC

{

var INVITE_Request v_InviteRequest;

var default v_RegistrationHandler := null;

f_IMS_PTC_Init(PDN_2);

// Emergency registration: 34.229-1 Annex C.20

f_IMS_EmergencyRegistration();

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup();

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

// 2nd emergency registration may be optional: default handler to deal with C.20 if happening

v_RegistrationHandler := activate(a_IMS_RcvAndHandleRegister(car_IMS_Register_Request(cr_REGISTER_Request())));  //@sic R5-191230 sic@

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup();

// deactivate default handler to deal with C.20 if happening

deactivate(v_RegistrationHandler);

// Inform EUTRA: IMS Call established.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//IMS call to be released.

f_IMS_IPCAN_WaitForTrigger(IPCAN);

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

}

/*

* @desc      Catches emergency registration according to 34.229-1 Annex C.20

* @param     p_IMS_DATA_REQ

* @status    APPROVED (LTE)

*/

altstep a_IMS_RcvAndHandleRegister(template (present) IMS_DATA_REQ p_IMS_DATA_REQ) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

[] IMS_Server.receive(p_IMS_DATA_REQ) -> value v_IMS_DATA_REQ

{

//Release Register info

f_IMS_GlobalRelease();

// Emergency registration: 34.229-1 Annex C.20

f_IMS_EmergencyRegistration(v_IMS_DATA_REQ);

}

}

After change:

function f_TC_11_2_7_IMS2() runs on IMS_PTC

{

var INVITE_Request v_InviteRequest;

var IMS_DATA_REQ v_IMS_DATA_REQ;

var default v_RetransmissionHandler := null;

f_IMS_PTC_Init(PDN_2);

// Emergency registration: 34.229-1 Annex C.20

f_IMS_EmergencyRegistration();

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup();

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

// Receive 2nd optional emergency registration / INVITE for emergency call

IMS_Server.receive(cr_IMS_DATA_REQ_Any) -> value v_IMS_DATA_REQ;

// Handle Retransmissions of REGISTER / INVITE that may be received by SS, due to overhead of following few functions post reception of above

v_RetransmissionHandler := activate(a_IMS_RetransmissionHandler(v_IMS_DATA_REQ));

if (ischosen(v_IMS_DATA_REQ.Request.Register)) {

//Release Register info

f_IMS_GlobalRelease();

// Emergency registration: 34.229-1 Annex C.20

f_IMS_EmergencyRegistration(v_IMS_DATA_REQ);

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup();

}

else if (ischosen(v_IMS_DATA_REQ.Request.Invite)) {

// Initialise global variables for SIP dialog

f_IMS_PTC_ImsInfo_DialogInit(dialogRemote, v_IMS_DATA_REQ.Request.Invite.msgHeader);

// Optional

f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), A_2_1_A7);

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup(-,-,-,v_IMS_DATA_REQ);

}

deactivate(v_RetransmissionHandler);

// Inform EUTRA: IMS Call established.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//IMS call to be released.

f_IMS_IPCAN_WaitForTrigger(IPCAN);

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

}

// REMOVED altstep a_IMS_RcvAndHandleRegister

## 2.2 Change 2

| Function name | f_IMS_EmergencyCallSetup |
| --- | --- |
| Reason for change | Required to fulfil Change 1 |
| Summary of change | Function is made to accept IMS_DATA_REQ as parameter and assign it to the variable v_IMS_DATA_REQ, if the parameter has a value. |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment |  |

Before change:

function f_IMS_EmergencyCallSetup(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7,

boolean p_GeolocationInfoAvailable := false,

template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request

{ /* p_Context   ..   A_2_1_A6 (INVITE for creating an emergency session in case of no registration) or

*                  A_2_1_A7 (INVITE for creating an emergency session within an emergency registration) */

/* @sic R5s130988 change 6/7: sic@

* in case of A_2_1_A6 there has been no registration beforehand and therefore we don't know whether the UE uses IPv4 or IPv6

* => we cannot setup any address information before we have received the INVITE

* => as the SDP message at step 1 contains address information we cannot build up the template in advance but need to match after the INVITE has been received */

/* @sic R5-140920: additional parameter p_OtherCalleeUri sic@ */

/* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */

/* @sic R5s210875: split function into step 1 and the rest sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ;

// Step 1. Receive INVITE for an Emergency call

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri);

return f_IMS_EmergencyCallSetup_Step2ToEnd(v_IMS_DATA_REQ, p_GeolocationInfoAvailable);

}

After change:

function f_IMS_EmergencyCallSetup(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7,

boolean p_GeolocationInfoAvailable := false,

template SipUrl p_OtherCalleeUri := omit,

template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit) runs on IMS_PTC return INVITE_Request

{ /* p_Context   ..   A_2_1_A6 (INVITE for creating an emergency session in case of no registration) or

*                  A_2_1_A7 (INVITE for creating an emergency session within an emergency registration) */

/* @sic R5s130988 change 6/7: sic@

* in case of A_2_1_A6 there has been no registration beforehand and therefore we don't know whether the UE uses IPv4 or IPv6

* => we cannot setup any address information before we have received the INVITE

* => as the SDP message at step 1 contains address information we cannot build up the template in advance but need to match after the INVITE has been received */

/* @sic R5-140920: additional parameter p_OtherCalleeUri sic@ */

/* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */

/* @sic R5s210875: split function into step 1 and the rest sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ;

if (not isvalue(p_IMS_DATA_REQ)) {

// Step 1. Receive INVITE for an Emergency call

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri);

} else {

v_IMS_DATA_REQ := valueof(p_IMS_DATA_REQ);

}

return f_IMS_EmergencyCallSetup_Step2ToEnd(v_IMS_DATA_REQ, p_GeolocationInfoAvailable);

}