3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221029

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2716 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG Testcase 6.5.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Similar to the change for TC 6.5.2.1 in R5s220857, TC 6.5.2.4 requires that the function f_NR_CellInfo_SetPLMN be called prior to F_NR_CellInfo_Set_SIB1_SetPNI_NPN_IdentityInfoList to properly configure the CagInfoList IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList f_NR_CellInfo_SetPLMN; Called: f_NR_CellInfo_SetPLMN f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_6_5_2_4_NR5GC() |
| --- | --- |
| Reason for change | Similar to the change for TC 6.5.2.1 in R5s220857, TC 6.5.2.4 requires that the function f_NR_CellInfo_SetPLMN be called prior to F_NR_CellInfo_Set_SIB1_SetPNI_NPN_IdentityInfoList to properly configure the CagInfoList IEs. |
| Summary of change | Instead of calling f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList f_NR_CellInfo_SetPLMN; Called: f_NR_CellInfo_SetPLMN f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList; |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_5_2_4_NR5GC() runs on NR5GC_PTC { // CAG / cell reselection / Within allowed CAG / non-CAG cell to CAG cell var boolean v_IsFR1 := true; var ReselectionThreshold v_ThreshX_HighP; var template(value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell; var template(value) SSB_MTC v_SSB_MTC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. f_NR5GC_Init(NR_4); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); if (v_IsFR1){ v_ThreshX_HighP := 10; // FR1 value }else{ v_ThreshX_HighP := 9; // FR2 value } // Table 6.5.2.4.3.1-1 Cell3 and Cell6 are CAG Cells. default CAG-ID value used for Cell3, value 2 used for Cell6 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell3); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell6, 2); // Table 6.5.2.4.3.3-2: SIB2 of NR Cell 6 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5); // Table 6.5.2.4.3.3-3 SIB4 of NR Cell 3 v_InterFreqCarrierFreqList_Cell := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList_Cell[1]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, v_ThreshX_HighP); // Table 6.5.2.4.3.3-3 SIB4 of NR Cell 1 v_InterFreqCarrierFreqList_Cell := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList_Cell[1], v_InterFreqCarrierFreqList_Cell[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5); // PeriodicityAndOffset with condtion INTER-FREQ_ODD v_SSB_MTC := f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell1, nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell1, 0, v_SSB_MTC.periodicityAndOffset); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_SSB_MTC.periodicityAndOffset); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell6, 0, v_SSB_MTC.periodicityAndOffset); // Table 6.5.2.4.3.1-1 PLMN f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell6, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78); //FR2 FFS // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //CAG Cell f_NR_CellConfig_Def(nr_Cell6); //CAG Cell // Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_CAGInfoList_twoEntries(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)),cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4))), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell3)))); //PLMN2 as EPLMN); //Anritsu TTCN CR R5s220768 MCC f_NR_TestBody_Set(true); fl_TC_6_5_2_4_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }

After Change:

function f_TC_6_5_2_4_NR5GC() runs on NR5GC_PTC { // CAG / cell reselection / Within allowed CAG / non-CAG cell to CAG cell var boolean v_IsFR1 := true; var ReselectionThreshold v_ThreshX_HighP; var template(value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell; var template(value) SSB_MTC v_SSB_MTC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. f_NR5GC_Init(NR_4); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); if (v_IsFR1){ v_ThreshX_HighP := 10; // FR1 value }else{ v_ThreshX_HighP := 9; // FR2 value } // Table 6.5.2.4.3.1-1 PLMN // MOVED f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell6, tsc_NR_HPLMN_002_11);//PLMN2 // Table 6.5.2.4.3.1-1 Cell3 and Cell6 are CAG Cells. default CAG-ID value used for Cell3, value 2 used for Cell6 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell3); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell6, 2); // Table 6.5.2.4.3.3-2: SIB2 of NR Cell 6 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5); // Table 6.5.2.4.3.3-3 SIB4 of NR Cell 3 v_InterFreqCarrierFreqList_Cell := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList_Cell[1]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, v_ThreshX_HighP); // Table 6.5.2.4.3.3-3 SIB4 of NR Cell 1 v_InterFreqCarrierFreqList_Cell := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList_Cell[1], v_InterFreqCarrierFreqList_Cell[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5); // PeriodicityAndOffset with condtion INTER-FREQ_ODD v_SSB_MTC := f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell1, nr_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell1, 0, v_SSB_MTC.periodicityAndOffset); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_SSB_MTC.periodicityAndOffset); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell6, 0, v_SSB_MTC.periodicityAndOffset); f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78); //FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78); //FR2 FFS // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //CAG Cell f_NR_CellConfig_Def(nr_Cell6); //CAG Cell // Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_CAGInfoList_twoEntries(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)),cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4))), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell3)))); //PLMN2 as EPLMN); //Anritsu TTCN CR R5s220768 MCC f_NR_TestBody_Set(true); fl_TC_6_5_2_4_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }