3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221099

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2751 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.4.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the function f_TC_8_1_4_2_1_2_NR5GC - When NR cell3 ( EN-DC cell ) is created using the f_NR_CellConfig_Def api call , 4th parameter has to be false , so that SRBs will not be configured ( please refer : R5s221087 ) In the current Implementation of the function fl_TC_8_1_4_2_1_2_TestBody - At step 0A, Pdu session modification command is sent to add DRB#a , as per prose table Table 8.1.4.2.1.2.3.3-0A , Eps Bearer Identity should be DRB#a + 4 . TTCN is not filling this correctly. This has to be corrected - Data sent at the step 6 is IPV6 ICMP packet . but when Pdu session Modification command is sent out , TFT is filled with bearer context as 1 , which maps to p_PacketFilterNumber = 1,2 which is not defining v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6 in the function fl_Get_508_PacketFilter. - It is proposed to change the bearer context as 6 , for which the p_PacketFilterNumber will be 6,7 which is defining v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6 in the function fl_Get_508_PacketFilter In the current implementation of the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA , Postamble trigger is called even before the Postamble is performed in EUTRA PTC . By doing so we are allowing Nr cell release to happen first, and UE will send SCG Failure with T310 timer expiry ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_4_2_1_2_NR5GC - When NR CELL 3 ( EN-DC cell ) is created using the f_NR_CellConfig_Def api call , 4th parameter is set as false , so that SRBs will not be configured ( please refer : R5s221087 ) In the function fl_TC_8_1_4_2_1_2_TestBody - Eps Bearer Identity is corrected to have DRB#a + 4 - TFT is filled with bearer context 6 instead of 1 In the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA, the postamble trigger is sent to NR after the EUTRA postamble is done , this avoids UE sending SCG Failure with T310 timer expiry |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function f_TC_8_1_4_2_1_2_NR5GC	3

2.2	Correction to the function fl_TC_8_1_4_2_1_2_TestBody	6

2.3	Correction to the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_TC_8_1_4_2_1_2_NR5GC

| Template name | f_TC_8_1_4_2_1_2_NR5GC |
| --- | --- |
| Reason for change | In the function f_TC_8_1_4_2_1_2_NR5GC - When NR cell3 ( EN-DC cell ) is created using the f_NR_CellConfig_Def api call , 4th parameter has to be false , so that SRBs will not be configured ( please refer : R5s221087 ) |
| Summary of change | In the function f_TC_8_1_4_2_1_2_NR5GC - When NR CELL 3 ( EN-DC cell ) is created using the f_NR_CellConfig_Def api call , 4th parameter is set as false , so that SRBs will not be configured ( please refer : R5s221087 ) |
| TTCN module | IRAT_Handover_NR5GC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_4_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT Handover / From NR to EN-DC / Success var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Use a specific init to initialise NR cells with ENDC band cobination and ENDC frequencies f_NR5GC_InitWithENDC(NR_6); fl_NR5GC_IRAT_InitSysinfo (NR_6); // Change cellGroupId of NR_Cell3 f_NR_CellInfo_Set_CellGroupId(nr_Cell3, tsc_NR_CellGroupId_SCG); f_NR_CellInfo_Set_ServingCellIndex(nr_Cell3, 1); //@sic R5s220853 sic@ f_NR_SetSysinfoCombinationToMIBOnly(nr_Cell3); //Create and configure cell // FFS Prose doesn't specify any power levels f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3, -, -, true); //No SRBs configured in EN-DC cells @sic R5s220853 sic@ //Reconfigure RACH of nr_Cell3 for ENDC RACH v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(f_NR_CellInfo_GetPhysicalParameters(nr_Cell3), -,-, tsc_NR_ENDC_Msg3Grant_Def); f_NR_SS_RachProcedureConfig(nr_Cell3, v_NR_RachProcedure); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); //Prepare nr_Cell3 f_NR_SetCellPower (nr_Cell3, -91, -91); // @sic R5-225246 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_4_2_1_2_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // @sic R5s220853 sic@ // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_1_4_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT Handover / From NR to EN-DC / Success var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Use a specific init to initialise NR cells with ENDC band cobination and ENDC frequencies f_NR5GC_InitWithENDC(NR_6); fl_NR5GC_IRAT_InitSysinfo (NR_6); // Change cellGroupId of NR_Cell3 f_NR_CellInfo_Set_CellGroupId(nr_Cell3, tsc_NR_CellGroupId_SCG); f_NR_CellInfo_Set_ServingCellIndex(nr_Cell3, 1); //@sic R5s220853 sic@ f_NR_SetSysinfoCombinationToMIBOnly(nr_Cell3); //Create and configure cell // FFS Prose doesn't specify any power levels f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3, -, -, false); //No SRBs configured in EN-DC cells @sic R5s220853 sic@ //WA#WI=1007036 //Reconfigure RACH of nr_Cell3 for ENDC RACH v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(f_NR_CellInfo_GetPhysicalParameters(nr_Cell3), -,-, tsc_NR_ENDC_Msg3Grant_Def); f_NR_SS_RachProcedureConfig(nr_Cell3, v_NR_RachProcedure); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); //Prepare nr_Cell3 f_NR_SetCellPower (nr_Cell3, -91, -91); // @sic R5-225246 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_4_2_1_2_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // @sic R5s220853 sic@ // UE now on EUTRA f_NR_ReleaseAllCells(); }

## Correction to the function fl_TC_8_1_4_2_1_2_TestBody

| Template name | fl_TC_8_1_4_2_1_2_TestBody |
| --- | --- |
| Reason for change | In the current Implementation of the function fl_TC_8_1_4_2_1_2_TestBody - At step 0A, Pdu session modification command is sent to add DRB#a , as per prose table Table 8.1.4.2.1.2.3.3-0A , Eps Bearer Identity should be DRB#a + 4 . TTCN is not filling this correctly. This has to be corrected - Data sent at the step 6 is IPV6 ICMP packet . but when Pdu session Modification command is sent out , TFT is filled with bearer context as 1 , which maps to p_PacketFilterNumber = 1,2 which is not defining v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6 in the function fl_Get_508_PacketFilter. - It is proposed to change the bearer context as 6 , for which the p_PacketFilterNumber will be 6,7 which is defining v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6 in the function fl_Get_508_PacketFilter |
| Summary of change | In the function fl_TC_8_1_4_2_1_2_TestBody - Eps Bearer Identity is corrected to have DRB#a + 4 - TFT is filled with bearer context 6 instead of 1 |
| TTCN module | IRAT_Handover_NR5GC_NR |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_2_1_2_TestBody() runs on NR5GC_PTC { var template (value) SDAP_Configuration_Type v_SDAP_Configuration; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_EUTRA_HOCmd; var NasCountInfo_Type v_NASCountInfo; var EUTRA_NR_PduSessionInfo_Type v_CoordMsg; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfoTemplate := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN); var GSM_MobilityInfo_Type v_PDUSessionInfo; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; var octetstring v_QoSParamOctets; var octetstring v_TFTParamOctets; var HalfOctet_Type v_EPSBearerId; var integer v_NewDRBId; var template (value) SDAP_Config v_SDAP_Config; var template (value) DRB_ToAddMod v_DRBToAddMod; var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell3, -, true); var template(value) CellGroupConfig v_SCGConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) RadioBearerConfig v_RadioBearerConfig; var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var octetstring v_SduTX; var NR_PDCP_SDUList_Type v_PDCP_Data; var template (value) NR_DRB_COMMON_REQ v_DRB_DL; var template (present) NR_DRB_COMMON_IND v_DRB_UL; // get info for only one PDU session as ENDC test body, and prose, only expects 1 PDN if (isvalue(v_PDUSessionInfoTemplate)) { v_PDUSessionInfo := valueof(v_PDUSessionInfoTemplate); } else { // @sic R5s220853 sic@ v_PDUSessionInfo := v_PDUSessionInfoList[0]; } v_CoordMsg.PDU_SessionId := oct2int(v_PDUSessionInfo.SessionId); v_NewDRBId := f_NR_GetNextDRBId(v_PDUSessionInfo, false); v_CoordMsg.EPS_Bearer := v_PDUSessionInfo.EPS_Bearer; // This is the default bearer v_CoordMsg.QFI := 0; // Don't need the QFI of the default bearer v_CoordMsg.PdnIndex := v_PDUSessionInfo.PdnIndex; v_CoordMsg.IsIMS := false; // Configure extra DRB on Cell 1, using PDU Session Modification // This will become Dedicated Bearer on EUTRA/SCG on Cell 3 @sic R5s220853 sic@ v_EPSBearerId := int2hex(valueof(v_PDUSessionInfo.EPS_Bearer), 1); v_QoSFlow := f_BuildDefaultQosFlow("5", true, hex2oct(v_EPSBearerId & '0'H)); v_QoSFlow.qfi := '000101'B; // QFI = 5 if (v_PDUSessionInfo.PDUType == IMS_DNN) { // @sic R5s220853 sic@ v_EPSParam := encvalue(f_Get_508_EPSQoS(DED_4)); v_QoSParamOctets := bit2oct(v_EPSParam); v_EPSParam := encvalue(f_EUTRA_NB_Get_508_TrafficFlowTemplate(4, v_EPSBearerId, v_PDUSessionInfo.SessionType, v_PDUSessionInfo.PdnIndex)); v_TFTParamOctets := bit2oct(v_EPSParam); } else { v_EPSParam := encvalue(f_Get_508_EPSQoS(DED_1)); v_QoSParamOctets := bit2oct(v_EPSParam); v_EPSParam := encvalue(f_EUTRA_NB_Get_508_TrafficFlowTemplate(1, v_EPSBearerId, v_PDUSessionInfo.SessionType, v_PDUSessionInfo.PdnIndex)); v_TFTParamOctets := bit2oct(v_EPSParam); << SKIPPED CODE >> }

After Change:

function fl_TC_8_1_4_2_1_2_TestBody() runs on NR5GC_PTC { var template (value) SDAP_Configuration_Type v_SDAP_Configuration; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_EUTRA_HOCmd; var NasCountInfo_Type v_NASCountInfo; var EUTRA_NR_PduSessionInfo_Type v_CoordMsg; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfoTemplate := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, Internet_DNN); var GSM_MobilityInfo_Type v_PDUSessionInfo; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; var octetstring v_QoSParamOctets; var octetstring v_TFTParamOctets; var HalfOctet_Type v_EPSBearerId; var integer v_NewDRBId; var template (value) SDAP_Config v_SDAP_Config; var template (value) DRB_ToAddMod v_DRBToAddMod; var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell3, -, true); var template(value) CellGroupConfig v_SCGConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) RadioBearerConfig v_RadioBearerConfig; var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var octetstring v_SduTX; var NR_PDCP_SDUList_Type v_PDCP_Data; var template (value) NR_DRB_COMMON_REQ v_DRB_DL; var template (present) NR_DRB_COMMON_IND v_DRB_UL; // get info for only one PDU session as ENDC test body, and prose, only expects 1 PDN if (isvalue(v_PDUSessionInfoTemplate)) { v_PDUSessionInfo := valueof(v_PDUSessionInfoTemplate); } else { // @sic R5s220853 sic@ v_PDUSessionInfo := v_PDUSessionInfoList[0]; } v_CoordMsg.PDU_SessionId := oct2int(v_PDUSessionInfo.SessionId); v_NewDRBId := f_NR_GetNextDRBId(v_PDUSessionInfo, false); v_CoordMsg.EPS_Bearer := v_PDUSessionInfo.EPS_Bearer; // This is the default bearer v_CoordMsg.QFI := 0; // Don't need the QFI of the default bearer v_CoordMsg.PdnIndex := v_PDUSessionInfo.PdnIndex; v_CoordMsg.IsIMS := false; // Configure extra DRB on Cell 1, using PDU Session Modification // This will become Dedicated Bearer on EUTRA/SCG on Cell 3 @sic R5s220853 sic@ v_EPSBearerId := int2hex(valueof(v_NewDRBId+4), 1); //WA#WI=1007036 v_QoSFlow := f_BuildDefaultQosFlow("5", true, hex2oct(v_EPSBearerId & '0'H)); v_QoSFlow.qfi := '000101'B; // QFI = 5 if (v_PDUSessionInfo.PDUType == IMS_DNN) { // @sic R5s220853 sic@ v_EPSParam := encvalue(f_Get_508_EPSQoS(DED_4)); v_QoSParamOctets := bit2oct(v_EPSParam); v_EPSParam := encvalue(f_EUTRA_NB_Get_508_TrafficFlowTemplate(4, v_EPSBearerId, v_PDUSessionInfo.SessionType, v_PDUSessionInfo.PdnIndex)); v_TFTParamOctets := bit2oct(v_EPSParam); } else { v_EPSParam := encvalue(f_Get_508_EPSQoS(DED_1)); v_QoSParamOctets := bit2oct(v_EPSParam); v_EPSParam := encvalue(f_EUTRA_NB_Get_508_TrafficFlowTemplate(6, v_EPSBearerId, v_PDUSessionInfo.SessionType, v_PDUSessionInfo.PdnIndex)); //WA#WI=1007036 v_TFTParamOctets := bit2oct(v_EPSParam); } << SKIPPED CODE >> }

## Correction to the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA

| Template name | f_TC_8_1_4_2_1_2_NR5GC_EUTRA |
| --- | --- |
| Reason for change | In the current implementation of the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA , Postamble trigger is called even before the Postamble is performed in EUTRA PTC . By doing so we are allowing Nr cell release to happen first, and UE will send SCG Failure with T310 timer expiry ) |
| Summary of change | In the function f_TC_8_1_4_2_1_2_NR5GC_EUTRA, the postamble trigger is sent to NR after the EUTRA postamble is done , this avoids UE sending SCG Failure with T310 timer expiry |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_4_2_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT Handover / From NR to EN-DC / Success f_EUTRA38_IRAT_InitWithENDC(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); // @sic R5s220853 sic@ f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change:

function f_TC_8_1_4_2_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT Handover / From NR to EN-DC / Success f_EUTRA38_IRAT_InitWithENDC(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); //f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); // @sic R5s220853 sic@ //WA#WI=1007036 f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); //WA#WI=1007036 }