3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221121

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2769 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR test case 6.1.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current implementation of the test case , at Step 0 . Modify System Information is done for Cell 3 , but cell 3 is still powered Off . It is proposed to make the Nr Cell 3 power level as Non Suitable Cell ( not to be powered off ) - As part of RRC system Information Request procedure as there is no Real Msg4 RRC involved , it is proposed to add the test case as part of f_GetTestcaseAttrib_EarlyContentionResolution function , so that this case will be sending early MSG4 CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR CELL 3 is made as NON-Suitable cell at Step 0 , before modifying the system Information on NR CELL 3 – An associated Prose CR will be raised for the same - Proposal to add 6.1.2.11 test case as part of the function f_GetTestcaseAttrib_EarlyContentionResolution |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_GetTestcaseAttrib_EarlyContentionResolution	2

2.2	Correction to the function f_TC_6_1_2_11_NR5GC_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_GetTestcaseAttrib_EarlyContentionResolution

| Template name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | As part of RRC system Information Request procedure as there is no Real Msg4 RRC involved , it is proposed to add the test case as part of f_GetTestcaseAttrib_EarlyContentionResolution function , so that this case will be sending early MSG4 CR |
| Summary of change | Proposal to add 6.1.2.11 test case as part of the function f_GetTestcaseAttrib_EarlyContentionResolution |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } << SKIPPED CODE >> }

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_6_1_2_11_NR5GC") { return true; } //WA#WI=1008478 case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } << SKIPPED CODE >> }

## Correction to the function f_TC_6_1_2_11_NR5GC_TestBody

| Template name | f_TC_6_1_2_11_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current implementation of the test case , at Step 0 . Modify System Information is done for Cell 3 , but cell 3 is still powered Off . It is proposed to make the Nr Cell 3 power level as Non Suitable Cell ( not to be powered off ) |
| Summary of change | NR CELL 3 is made as NON-Suitable cell at Step 0 , before modifying the system Information on NR CELL 3 |
| TTCN module | Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { //Power levels on FR2 are FFS //@sic R5-202603 R5-225373 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := -95; var integer v_T1_Cell3_RS_EPRE_FR1 := -80; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell3_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell6_RS_EPRE_FR1 := -80; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList := f_NR_InitInterFreqCarrierFreqNeighbourCell(nr_Cell1, nr_Cell3, nr_Cell6); var template SI_SchedulingInfo v_SI_SchedulingInfo; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; //@sic R5s200703 sic@ var template(value) SSB_MTC v_SSB_MTC; timer t_WaitTime := 5.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell3, v_T1_Cell3_RS_EPRE_FR1, v_T1_Cell3_RS_EPRE_FR1) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell3, v_T2_Cell3_RS_EPRE_FR1, v_T2_Cell3_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell6, v_T2_Cell6_RS_EPRE_FR1, v_T2_Cell6_RS_EPRE_FR1) }; // @siclog "Step 0" siclog@ // The SS changes SIB1 as specified as Table 6.1.2.11.3.3-1 in nr_Cell1 and nr_Cell3. // Table 6.1.2.11.3.3-1: SIB1 of NR cell 1 and NR cell 3 (All steps in Table 6.1.2.11.3.2-3) v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(nr_Cell1); v_SI_SchedulingInfo.schedulingInfoList[1] := cs_NR_SchedulingInfoDef(tsc_SIB4_Periodicity, {cs_NR_SIB_TypeInfoDef_AreaScope(sibType4)}, notBroadcasting); v_SI_SchedulingInfo.systemInformationAreaID := '000000000000000000000011'B; f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell1, valueof(v_SI_SchedulingInfo)); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell3, valueof(v_SI_SchedulingInfo)); f_NR_ModifySysinfo(nr_Cell3, false); << SKIPPED CODE >> }

After Change:

function f_TC_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { //Power levels on FR2 are FFS //@sic R5-202603 R5-225373 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := -95; var integer v_T1_Cell3_RS_EPRE_FR1 := -80; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell3_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell6_RS_EPRE_FR1 := -80; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList := f_NR_InitInterFreqCarrierFreqNeighbourCell(nr_Cell1, nr_Cell3, nr_Cell6); var template SI_SchedulingInfo v_SI_SchedulingInfo; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; //@sic R5s200703 sic@ var template(value) SSB_MTC v_SSB_MTC; timer t_WaitTime := 5.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell3, v_T1_Cell3_RS_EPRE_FR1, v_T1_Cell3_RS_EPRE_FR1) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell3, v_T2_Cell3_RS_EPRE_FR1, v_T2_Cell3_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell6, v_T2_Cell6_RS_EPRE_FR1, v_T2_Cell6_RS_EPRE_FR1) }; f_NR_SetCellPower(nr_Cell3, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //WA#WI=1008478 // @siclog "Step 0" siclog@ // The SS changes SIB1 as specified as Table 6.1.2.11.3.3-1 in nr_Cell1 and nr_Cell3. // Table 6.1.2.11.3.3-1: SIB1 of NR cell 1 and NR cell 3 (All steps in Table 6.1.2.11.3.2-3) v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(nr_Cell1); v_SI_SchedulingInfo.schedulingInfoList[1] := cs_NR_SchedulingInfoDef(tsc_SIB4_Periodicity, {cs_NR_SIB_TypeInfoDef_AreaScope(sibType4)}, notBroadcasting); v_SI_SchedulingInfo.systemInformationAreaID := '000000000000000000000011'B; f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell1, valueof(v_SI_SchedulingInfo)); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(nr_Cell3, valueof(v_SI_SchedulingInfo)); f_NR_ModifySysinfo(nr_Cell3, false); << SKIPPED CODE >> }