3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221143

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2785 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10.NR5GC, 6.2.3.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2 Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_3_10_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |
| Summary of change | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_3_10_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { …………….. //Modify SIB1 scheduling as per Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); // Add schedulingInfoList-v12j0 acc to Table 6.2.3.10.3.3-1 v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); //Modify SI list v_SIs := f_EUTRA_CellInfo_GetSysinfoSIs(eutra_Cell1); …………….. }

After Change:

function f_TC_6_2_3_10_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ……………….. //Modify SIB1 scheduling as per Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); // Add schedulingInfoList-v12j0 acc to Table 6.2.3.10.3.3-1 if(px_ePrimaryFrequencyBand >= maxFBI){ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(px_ePrimaryFrequencyBand, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } else{ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); //Modify SI list v_SIs := f_EUTRA_CellInfo_GetSysinfoSIs(eutra_Cell1); ………………. }

## 2.2 Change 2

| Function name | f_TC_6_2_3_11_NR5GC_EUTRA |
| --- | --- |
| Reason for change | If EUTRA band is greater than maxFBI then "freqBandIndicator-v9e0" should be included in the SIB1. |
| Summary of change | Include "freqBandIndicator-v9e0" in SIB1 forEUTRA bands greater than or equal to maxFBI. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ……………. v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit,cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); ……………. }

After Change

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { …………… v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); if(px_ePrimaryFrequencyBand >= maxFBI){ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(px_ePrimaryFrequencyBand, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } else{ v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit, cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); } f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); ……………………. }