3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221148

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0869 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS Conference test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 22wk23 ATS R5s220849 was submitted to allow the inclusion of “Require=Replaces” header in REFER. It was accepted and now that should be allow for Three Way Calls and not uniquely for 8.36 test case (Consultative Call Transfer). 22wk37 ATS has what seems some incomplete implementation and test cases 8.34, 8.35 and 8.36 are broken. This CR mostly recover proposed implementation in 22wk23 to fix the new issue. Removed unnecessary statemnents and allow UEs to include “Require=replaces” header when running any of the three test cases mentioned above. With proposed implementation Require header MUST be present only for 8.36. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed unnecessary statemnents and allow UEs to include “Require=replaces” header for Three Way Calls but requiring his presence for Consultative Call Transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.NR5GC, 8.35.NR5GC, 8.36.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s220849 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function fl_SIP_BuildReferToUri_RX	3

2.2	Correction to function a_IMS_REFER_ReceiveRequest	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_SIP_BuildReferToUri_RX

| Function name | fl_SIP_BuildReferToUri_RX |
| --- | --- |
| Reason for change | Removed unnecessary statemnents and allow UEs to include “Require=replaces” header when running any of the three test cases mentioned above. With proposed implementation Require header MUST be present only for 8.36. |
| Summary of change | Removed unnecessary statemnents and allow UEs to include “Require=replaces” header but requires his presence for Explicit Call Forwarding. *Code highlight in yellow is removed. |
| TTCN module | IMS\Common\NR5GC\IMS_Procedures_Conferencing.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function fl_SIP_BuildReferToUri_RX(charstring p_ReferToUriString, boolean p_ReplacesHeaderIsRequired, boolean p_RequireHeaderIsRequired) return template (present) SipUrl { // !!!! R5s220849_MCC160Comments: p_RequireHeaderIsRequired to be removed after 22wk37 delivery !!!! var template (present) SipUrl v_ReferToUri := f_SIP_BuildSipUri_RX(p_ReferToUriString); var template (present) UserInfo v_UserInfo; var template (present) UriComponents v_UriComponents; var template SemicolonParam_List v_Params; var template (present) AmpersandParam_List v_AllowedHeaders; if (p_ReplacesHeaderIsRequired) { if (ischosen(v_ReferToUri.components.tel)) { /* convert tel URI to SIP URI acc. to RFC 3261 clause 19.1.6: */ v_UserInfo := cr_UserInfo(v_ReferToUri.components.tel.subscriber, omit); v_Params := superset(cr_GenericParam_Common("user", cr_GenValueToken("phone"))); // @sic R5s160295; NOTE: per default f_SIP_BuildSipUri_RX initialises urlParameters with * sic@ v_UriComponents := cr_SipUriComponent(v_UserInfo, *, *); v_ReferToUri := cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); } if (false) { // !!!! R5s220849_MCC160Comments: to be restored after 22wk37 delivery !!!! v_AllowedHeaders := { cr_GenericParam_Common("Replaces", cr_GenValueToken(?)), cr_GenericParam("Require", "replaces") }; v_ReferToUri.headers := subset(all from v_AllowedHeaders); /* @sic R5s220849 change 4: whether a respective header is there, is checked explicitly in a_IMS_REFER_ReceiveRequest sic@ */ } else { // !!!! R5s220849_MCC160Comments: to be removed after 22wk37 delivery !!!! v_ReferToUri.headers[0] := cr_GenericParam_Common("Replaces", cr_GenValueToken(?)); if (p_RequireHeaderIsRequired) { /* @sic R5s220575 sic@ */ v_ReferToUri.headers[1] := cr_GenericParam("Require", "replaces"); } } } return v_ReferToUri; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_SIP_BuildReferToUri_RX(charstring p_ReferToUriString, boolean p_ReplacesHeaderIsRequired) return template (present) SipUrl { /* @sic R5s220575: p_RequireHeaderIsRequired sic@ */ var template (present) SipUrl v_ReferToUri := f_SIP_BuildSipUri_RX(p_ReferToUriString); var template (present) UserInfo v_UserInfo; var template (present) UriComponents v_UriComponents; var template SemicolonParam_List v_Params; var template (present) AmpersandParam_List v_AllowedHeaders; if (p_ReplacesHeaderIsRequired) { if (ischosen(v_ReferToUri.components.tel)) { /* convert tel URI to SIP URI acc. to RFC 3261 clause 19.1.6: */ v_UserInfo := cr_UserInfo(v_ReferToUri.components.tel.subscriber, omit); v_Params := superset(cr_GenericParam_Common("user", cr_GenValueToken("phone"))); // @sic R5s160295; NOTE: per default f_SIP_BuildSipUri_RX initialises urlParameters with * sic@ v_UriComponents := cr_SipUriComponent(v_UserInfo, *, *); v_ReferToUri := cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); } v_AllowedHeaders := { cr_GenericParam_Common("Replaces", cr_GenValueToken(?)), cr_GenericParam("Require", "replaces") }; v_ReferToUri.headers := subset(all from v_AllowedHeaders); } return v_ReferToUri; } <<SKIPPED CODE>>

## Correction to function a_IMS_REFER_ReceiveRequest

| Function name | a_IMS_REFER_ReceiveRequest |
| --- | --- |
| Reason for change | Removed unnecessary statemnents and allow UEs to include “Require=replaces” header when running any of the three test cases mentioned above. With proposed implementation Require header MUST be present only for 8.36. |
| Summary of change | Removed unnecessary statemnents and allow UEs to include “Require=replaces” header but requires his presence for Explicit Call Forwarding. *Code highlight in yellow is removed. |
| TTCN module | IMS\Common\NR5GC\IMS_Procedures_Conferencing.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> altstep a_IMS_REFER_ReceiveRequest(out IMS_DATA_REQ p_IMS_REFER_REQ, charstring p_RequestUriString, charstring p_ReferToUriString, template (omit) IMS_PTC_DialogIndex_Type p_ActiveSessionToBeReplaced, boolean p_ReferCreatingDialog := false, IMS_CallType_Type p_CallType := CONFERENCE_CALL) runs on IMS_PTC { /* @sic R5s141343 change 3.1: p_ReferCreatingDialog := false sic@ */ /* @sic R5-153765: new parameter p_ActiveSessionToBeReplaced sic@ * @sic R5s150909: p_ActiveSessionToBeReplaced changed to "template (omit) IMS_PTC_DialogIndex_Type" sic@ * NOTE: ispresent(p_ActiveSessionToBeReplaced) => there is an active session between the UE and a remote user which shall be replaced (e.g. Three way session creation; 15.121a); * (the remote user is referenced by p_ReferToUriString) * p_ActiveSessionToBeReplaced==omit => there is no active session with the remote user but the REFER is just to invite the remote user */ /* @sic R5s160045 p_CSeqValue removed sic@ @sic R5s180548: p_CallType added sic@ */ var IMS_DATA_REQ v_IMS_REFER_REQ_ByRef; var template (present) SipUrl v_RequestUriRefer := f_SIP_BuildSipUri_RX(p_RequestUriString); var boolean v_ReplacesHeaderIsRequired := ispresent(p_ActiveSessionToBeReplaced); var boolean v_RequireHeaderIsRequired := v_ReplacesHeaderIsRequired and (p_CallType == NORMAL_CALL); var template (present) SipUrl v_ReferToUri := fl_SIP_BuildReferToUri_RX(p_ReferToUriString, v_ReplacesHeaderIsRequired, v_RequireHeaderIsRequired); /* @sic R5s150382 change 3; R5s160043, R5-160796 MCC160 implementation sic@ !!!! R5s220849_MCC160Comments: v_RequireHeaderIsRequiredto be removed after 22wk37 delivery !!!! */ var template (present) MessageHeader v_MessageHeader_Refer := f_IMS_ReferRequest_MessageHeaderRX(v_RequestUriRefer, v_ReferToUri, p_ReferCreatingDialog, -, p_CallType); var SipUrl v_ReferToUrl; var template (omit) GenericParam v_Replaces; var template (value) CallId v_CallId; var charstring v_FromTag; var charstring v_ToTag; var CharStringList_Type v_ReceivedDialogId; var template (present) CharStringList_Type v_ExpectedDialogId; [] a_IMS_ReceiveRequest(car_IMS_Refer_Request(cr_REFER_Request(v_RequestUriRefer, v_MessageHeader_Refer)), v_IMS_REFER_REQ_ByRef) { f_IMS_A_2_10_Refer_CommonChecks(v_IMS_REFER_REQ_ByRef, p_ReferCreatingDialog); if (not p_ReferCreatingDialog) { /* @sic R5s160045: CSeq value to be incremented and checked for in-dialog REFER sic@ */ f_IMS_RequestInDialog_IncrAndCheckCSeqValue(v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader); } v_ReferToUrl := v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader.referTo.nameAddr.addrSpec; if (v_ReplacesHeaderIsRequired) { /* @sic R5-153765, R5s150909 sic@ */ if (false) { // !!!! R5s220849_MCC160Comments: to be restored after 22wk37 delivery !!!! v_Replaces := f_SIP_AmpersandParamList_GetParam(v_ReferToUrl.headers, "Replaces"); if (not isvalue(v_Replaces)) { // @sic R5s220849 change 4 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "'Replaces' header missing"); } if (v_RequireHeaderIsRequired and not isvalue(f_SIP_AmpersandParamList_GetParam(v_ReferToUrl.headers, "Require"))) { // @sic R5s220849 change 4 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "'Require' header missing"); } } v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(p_ActiveSessionToBeReplaced); v_FromTag := valueof(f_IMS_PTC_ImsInfo_DialogGetRemoteTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ToTag := valueof(f_IMS_PTC_ImsInfo_DialogGetLocalTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ExpectedDialogId := { v_CallId.callid, permutation("to-tag=" & v_ToTag, "from-tag=" & v_FromTag, *) }; v_ReceivedDialogId := f_StringSplit(f_URL_Decoding(valueof(v_Replaces.paramValue.tokenOrHost)), {";"}); // see RFC 3891 6.1 if (not match(v_ReceivedDialogId, v_ExpectedDialogId)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid dialog id in Replaces header of Refer-To's SipUri"); } } p_IMS_REFER_REQ := v_IMS_REFER_REQ_ByRef; // return message by ref } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> altstep a_IMS_REFER_ReceiveRequest(out IMS_DATA_REQ p_IMS_REFER_REQ, charstring p_RequestUriString, charstring p_ReferToUriString, template (omit) IMS_PTC_DialogIndex_Type p_ActiveSessionToBeReplaced, boolean p_ReferCreatingDialog := false, IMS_CallType_Type p_CallType := CONFERENCE_CALL) runs on IMS_PTC { /* @sic R5s141343 change 3.1: p_ReferCreatingDialog := false sic@ */ /* @sic R5-153765: new parameter p_ActiveSessionToBeReplaced sic@ * @sic R5s150909: p_ActiveSessionToBeReplaced changed to "template (omit) IMS_PTC_DialogIndex_Type" sic@ * NOTE: ispresent(p_ActiveSessionToBeReplaced) => there is an active session between the UE and a remote user which shall be replaced (e.g. Three way session creation; 15.121a); * (the remote user is referenced by p_ReferToUriString) * p_ActiveSessionToBeReplaced==omit => there is no active session with the remote user but the REFER is just to invite the remote user */ /* @sic R5s160045 p_CSeqValue removed sic@ @sic R5s180548: p_CallType added sic@ */ var IMS_DATA_REQ v_IMS_REFER_REQ_ByRef; var template (present) SipUrl v_RequestUriRefer := f_SIP_BuildSipUri_RX(p_RequestUriString); var boolean v_ReplacesHeaderIsRequired := ispresent(p_ActiveSessionToBeReplaced); var boolean v_RequireHeaderIsRequired := v_ReplacesHeaderIsRequired and (p_CallType == NORMAL_CALL); var template (present) SipUrl v_ReferToUri := fl_SIP_BuildReferToUri_RX(p_ReferToUriString, v_ReplacesHeaderIsRequired); /* @sic R5s150382 change 3; R5s160043, R5-160796 MCC160 implementation, R5s220575: v_RequireHeaderIsRequired sic@ */ var template (present) MessageHeader v_MessageHeader_Refer := f_IMS_ReferRequest_MessageHeaderRX(v_RequestUriRefer, v_ReferToUri, p_ReferCreatingDialog, -, p_CallType); var SipUrl v_ReferToUrl; var template (omit) GenericParam v_Replaces; var template (value) CallId v_CallId; var charstring v_FromTag; var charstring v_ToTag; var CharStringList_Type v_ReceivedDialogId; var template (present) CharStringList_Type v_ExpectedDialogId; [] a_IMS_ReceiveRequest(car_IMS_Refer_Request(cr_REFER_Request(v_RequestUriRefer, v_MessageHeader_Refer)), v_IMS_REFER_REQ_ByRef) { f_IMS_A_2_10_Refer_CommonChecks(v_IMS_REFER_REQ_ByRef, p_ReferCreatingDialog); if (not p_ReferCreatingDialog) { /* @sic R5s160045: CSeq value to be incremented and checked for in-dialog REFER sic@ */ f_IMS_RequestInDialog_IncrAndCheckCSeqValue(v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader); } v_ReferToUrl := v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader.referTo.nameAddr.addrSpec; if (v_ReplacesHeaderIsRequired) { /* @sic R5-153765, R5s150909 sic@ */ v_Replaces := f_SIP_AmpersandParamList_GetParam(v_ReferToUrl.headers, "Replaces"); if (not isvalue(v_Replaces)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "'Replaces' header missing"); } if (v_RequireHeaderIsRequired and not isvalue(f_SIP_AmpersandParamList_GetParam(v_ReferToUrl.headers, "Require"))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "'Require' header missing"); } v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(p_ActiveSessionToBeReplaced); v_FromTag := valueof(f_IMS_PTC_ImsInfo_DialogGetRemoteTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ToTag := valueof(f_IMS_PTC_ImsInfo_DialogGetLocalTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ExpectedDialogId := { v_CallId.callid, permutation("to-tag=" & v_ToTag, "from-tag=" & v_FromTag, *) }; v_ReceivedDialogId := f_StringSplit(f_URL_Decoding(v_Replaces.paramValue.tokenOrHost), {";"}); // see RFC 3891 6.1 if (not match(v_ReceivedDialogId, v_ExpectedDialogId)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid dialog id in Replaces header of Refer-To's SipUri"); } } p_IMS_REFER_REQ := v_IMS_REFER_REQ_ByRef; // return message by ref } } <<SKIPPED CODE>>