3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221167

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2798 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC 5GSM test case 10.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The preamble condition “The UE is in state 3N-A on NGC Cell A with at least one PDU session for Internet active” is currently not enforced by TTCN. If the UE only creates an IMS PDU session in the preamble, then the PDU session established at step 0A can then be selected for release at step 1, causing the test case to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Trigger establishment of internet type PDU session after preamble if one is not alredy established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_10_1_2_2_NR5GC() |
| --- | --- |
| Reason for change | The preamble condition “The UE is in state 3N-A on NGC Cell A with at least one PDU session for Internet active” is currently not enforced by TTCN. If the UE only creates an IMS PDU session in the preamble, then the PDU session established at step 0A can then be selected for release at step 1. |
| Summary of change | Trigger establishment of internet type PDU session after preamble if one is not alredy established. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_10_1_2_2_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE / Invalid PDU session identity var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_2_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change:

function f_TC_10_1_2_2_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE / Invalid PDU session identity var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; var template (omit) NR_DRBInfo_Type v_DRBInfo; f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A); v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId) if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); } f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_2_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }