3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221174

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2803 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 9.1.10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 17 and 53 the SRB is hardcoded to tsc_NR_RbId_SRB2. At Step 17 this doesn’t impact much because a conformant UE wont go in that path, but this fails a conformant UE at Step 53 because after 50 SRB2 is not yet configured and the PDU Session Establishment is expected on SRB2. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced tsc_NR_RbId_SRB2 with v_SRB at Steps 17 and 53. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Correction to fl_TC_9_1_10_3_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Correction to fl_TC_9_1_10_3_TestBody

| Function name | fl_TC_9_1_10_3_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 17 and 53 the SRB is hardcoded to tsc_NR_RbId_SRB2. At Step 17 this doesn’t impact much because a conformant UE wont go in that path, but this fails a conformant UE at Step 53 because after 50 SRB2 is not yet configured and the PDU Session Establishment is expected on SRB2. This needs to be addressed. |
| Summary of change | Replaced tsc_NR_RbId_SRB2 with v_SRB at Steps 17 and 53. Please see below. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NG_NAS_UL_Message_Type v_GSMMsg; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; //@sic R5s220948 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '02'O, //@sic R5-224165 sic@ rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '01'O, omit)}}; //@sic R5-222008 R5-223429 R5-224165 sic@ var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '06'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit), cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} //@sic R5-223429 sic@ }; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_NAS_DL_Pdu_Type v_MToSend; var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI(omit, '01'O, '01'O, omit)); var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) GPRS_Timer3 v_T3512 := cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B); timer t_Wait := 5.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ //The SS transmits a REGISTRATION ACCEPT message including Rejected S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, //@sic R5-222008 sic@ v_RejectedNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 15" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 17" siclog@ //Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message within 5 seconds? t_Wait.start; //@sic R5-222008 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Switch off procedure in RRC_CONNECTED specified in TS 38.508-1 subclause 4.9.6.3 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A, v_SRB); //@sic R5s220948 sic@ //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 20" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 21" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 22" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 23" siclog@ Prose CR required //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } //@siclog "Step 24-32" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 33" siclog@ v_RejectedNSSAI := { iei := '11'O, iel := '02'O, //@sic R5-224165 sic@ rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '02'O, omit)}}; //@sic R5-222008 R5-223429 R5-224165 sic@ v_AllowedNSSAI:= { iei := '15'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} //@sic R5-223429 sic@ }; v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, v_RejectedNSSAI, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); //@sic R5-222008 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 34" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 35a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 36" siclog@ //Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 36"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 36"); } //@siclog "Step 37" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); if(pc_USIM_Removal){ //@siclog "Step 38a1" siclog@ f_UT_USIM_Remove (UT); //@siclog "Step 38a2" siclog@ //@sic R5s220948 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); v_ReceivedMsg := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(v_NGC_NASCellA, v_ReceivedMsg); // @sic R5-204389 sic@ f_NR_RRCRelease(v_NGC_NASCellA); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } //@siclog "Step 38a3" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 38a4" siclog@ f_UT_USIM_Insert(UT, omit); //@sic R5s220948 sic@ //@siclog "Step 38a5" siclog@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38a5"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38a5"); } //@siclog "Step 38a6" siclog@ //The SS configures NGC Cell A as the "Serving cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } else { //@siclog "Step 38b1" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5-222008 sic@ //@siclog "Step 38b2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5-222008 sic@ } //@siclog "Step 39" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40-48" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 49" siclog@ //The SS transmits a REGISTRATION ACCEPT message not in including Pending S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, v_T3512, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '0'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 50" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 51a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI('22'O, //@sic R5s220948 sic@ '01'O, '01'O, omit) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@sic R5s220535 sic@ //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, //@sic R5s220948 sic@ -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body

After Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NG_NAS_UL_Message_Type v_GSMMsg; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; //@sic R5s220948 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '02'O, //@sic R5-224165 sic@ rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '01'O, omit)}}; //@sic R5-222008 R5-223429 R5-224165 sic@ var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '06'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit), cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} //@sic R5-223429 sic@ }; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_NAS_DL_Pdu_Type v_MToSend; var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI(omit, '01'O, '01'O, omit)); var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) GPRS_Timer3 v_T3512 := cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B); timer t_Wait := 5.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ //The SS transmits a REGISTRATION ACCEPT message including Rejected S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, //@sic R5-222008 sic@ v_RejectedNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 15" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 17" siclog@ //Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message within 5 seconds? t_Wait.start; //@sic R5-222008 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB /*WA#9_1_10_3 tsc_NR_RbId_SRB2*/, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Switch off procedure in RRC_CONNECTED specified in TS 38.508-1 subclause 4.9.6.3 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A, v_SRB); //@sic R5s220948 sic@ //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 20" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 21" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 22" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 23" siclog@ Prose CR required //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } //@siclog "Step 24-32" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 33" siclog@ v_RejectedNSSAI := { iei := '11'O, iel := '02'O, //@sic R5-224165 sic@ rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '02'O, omit)}}; //@sic R5-222008 R5-223429 R5-224165 sic@ v_AllowedNSSAI:= { iei := '15'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} //@sic R5-223429 sic@ }; v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, v_RejectedNSSAI, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); //@sic R5-222008 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 34" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 35a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 36" siclog@ //Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 36"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 36"); } //@siclog "Step 37" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); if(pc_USIM_Removal) { //@siclog "Step 38a1" siclog@ f_UT_USIM_Remove (UT); //@siclog "Step 38a2" siclog@ //@sic R5s220948 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); v_ReceivedMsg := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(v_NGC_NASCellA, v_ReceivedMsg); // @sic R5-204389 sic@ f_NR_RRCRelease(v_NGC_NASCellA); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } //@siclog "Step 38a3" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 38a4" siclog@ f_UT_USIM_Insert(UT, omit); //@sic R5s220948 sic@ //@siclog "Step 38a5" siclog@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38a5"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38a5"); } //@siclog "Step 38a6" siclog@ //The SS configures NGC Cell A as the "Serving cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } else { //@siclog "Step 38b1" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5-222008 sic@ //@siclog "Step 38b2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5-222008 sic@ } //@siclog "Step 39" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40-48" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 49" siclog@ //The SS transmits a REGISTRATION ACCEPT message not in including Pending S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, v_T3512, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '0'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 50" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 51a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB /*WA#9_1_10_3 tsc_NR_RbId_SRB2*/, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI('22'O, //@sic R5s220948 sic@ '01'O, '01'O, omit) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@sic R5s220535 sic@ //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, //@sic R5s220948 sic@ -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body