3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221195

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2813 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC testcase 7.1.1.6.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add ENDC test case 7.1.1.6.1 in FR1 to the IWD_22wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to ENDC test case 7.1.1.6.1 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	NOTE: Common TTCN changes are covered in the NR5GC submission CR	3

2.2	f_TC_7_1_1_6_1_ENDC_EUTRA	3

2.3	f_TC_7_1_1_6_1_ENDC_NR	4

2.4	f_GetTestcaseAttrib_ShortDCI	8

3	Branches executed	9

4	Execution Log Files	9

4.1	MediaTek	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of ENDC test case 7.1.1.6.1 in FR1 which is part of the ENDC test suite in iWD_TTCN3-B2020-09_D22wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.6.1.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## NOTE: Common TTCN changes are covered in the NR5GC submission CR

## f_TC_7_1_1_6_1_ENDC_EUTRA

| Function name | f_TC_7_1_1_6_1_ENDC_EUTRA |
| --- | --- |
| Reason for change | UM DRB is used to transmit and receive Data, Hence, in the call to function f_EUTRA38_ENDC_ConfigAM_UM, the third parameter should be set to 1 |
| Summary of change | Third parameter controlling UM DRB establishment is set to 1 |
| TTCN module | MAC_SPS_ENDC_EUTRA |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; // Logical channel prioritization handling f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE in state 2A f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_Only, RRC_IDLE); f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 1, 0, TEST_LOOPModeA_ON, -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, tsc_DRB2)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; // Logical channel prioritization handling f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE in state 2A f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_Only, RRC_IDLE); f_EUTRA38_ENDC_ConfigAM_UM (eutra_Cell1, 0, 1, TEST_LOOPModeA_ON, -, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, tsc_DRB2)); //WA# <<SKIPPED CODE>>

## f_TC_7_1_1_6_1_ENDC_NR

| Function name | f_TC_7_1_1_6_1_ENDC_NR |
| --- | --- |
| Reason for change | - The initial configuration of MAC should be in Normal mode for RRC signalling to take place and to establish the UM DRB - The information to be sent to the UE in the RRC Connection Reconfiguration message should first be got from the function f_NR_GetServinCellConfig to be then set via the variable v_SpCellConfig - The addition of IEs reconfigurationWithSync and rlf_TimersAndConstants in the RRC Connection Reconfiguration message at Step 1 is not required as that information has already been communicated to the UE previously, specifically, the addition of IE reconfigurationWithSync would trigger an un-necessary PRACH procedure from the UE - The cellGroup Id should be set to SCG as the E-UTRA PTC is the MCG in this case - It is recommended to perform the local end configuration after the peer signalling message has been sent to EUTRA PTC. - MAC should be put in Transparent mode before the Testbody is called and should be set appropriately for the rest of the test case. |
| Summary of change | - Corrected the variable v_SS_Drb_ConfigList to put MAC in Normal mode - Function f_NR_GetServinCellConfig called to retrieve the necessary information. - Set IEs reconfigurationWithSync and rlf_TimersAndConstants to omit - cellGroup Id should be set to SCG - Local end configuration done after sending the peer signalling message to EUTRA PTC. - Set MAC mode appropriately to handle RRC Signalling and Transparent mode operations. |
| TTCN module | MAC_SPS_ENDC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 0A-0B logsic@ v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); //@siclog steps 29-30 logsic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //@siclog steps 31-32 logsic@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); };

After Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; //WA# var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; var template (omit) ServingCellConfig v_ServingCellConfig; //WA# //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 0A-0B logsic@ //WA# v_ServingCellConfig := f_NR_GetServinCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; //WA# v_SpCellConfig.reconfigurationWithSync := omit; v_SpCellConfig.rlf_TimersAndConstants := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //WA# omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //WA# f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@siclog steps 29-30 logsic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG ,//WA# omit, omit, omit, omit, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@siclog steps 31-32 logsic@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //WA# v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); };

## f_GetTestcaseAttrib_ShortDCI

| Function name | f_GetTestcaseAttrib_ShortDCI |
| --- | --- |
| Reason for change | In Rel-15, the PDCCH [for DL SPS explicit release] has to be done using DCI 0_0. Moreover, the DL SPS configuration is also being done using DCI 0_0 format. Hence, it is recommended to start the testcase configuration in Short DCI otherwise it would lead to an implementation with a mixture of DCI x_0 and DCI x_1 formats |
| Summary of change | TC 7.1.1.6.1 added to Short DCI list. |
| TTCN module | TestCaseProperties |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_ShortDCI(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_7_1_1_1_2_ENDC") { return true; } case ("TC_7_1_1_3_3_ENDC") { return true; } case ("TC_7_1_1_4_1_1_ENDC") { return true; } case ("TC_7_1_1_4_2_1_ENDC") { return true; } case ("TC_7_1_1_4_2_5_ENDC") { return true; } case ("TC_7_1_1_4_2_6_ENDC") { return true; } case ("TC_7_1_1_6_3_ENDC") { return true; } } return false; }

After Change:

function f_GetTestcaseAttrib_ShortDCI(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_7_1_1_1_2_ENDC") { return true; } case ("TC_7_1_1_3_3_ENDC") { return true; } case ("TC_7_1_1_4_1_1_ENDC") { return true; } case ("TC_7_1_1_4_2_1_ENDC") { return true; } case ("TC_7_1_1_4_2_5_ENDC") { return true; } case ("TC_7_1_1_4_2_6_ENDC") { return true; } case ("TC_7_1_1_6_1_ENDC") { return true; } //WA# case ("TC_7_1_1_6_3_ENDC") { return true; } } return false; }

# Branches executed

## This ENDC TC was executed in Band combination “DC_3A_N78A” using EEA1/EIA1 algorithms”.

# Execution Log Files

## MediaTek

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_7_1_1_6_1_ENDC_FR1_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221196: Supporting information for addition of ENDC TC 7.1.1.6.1 in FR1. |
| --- | --- |