3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221241

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0088 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Sep'22 baseline upgrade for MCX Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop the Rel-17 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to September 2022 in Rel-17 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 36.579-5 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the September 2022 Rel-17. These changes of the type definitions in tabular format and XSD have been taken from 3GPP specs, RFCs and OMA specs as listed below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 36.579-5 TTCN-3 ATSs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Baseline Moving – Required Changes	3

2.1	SIP/SDP types	3

2.2	XSD types	3

2.2.1	XSD Modules	3

2.2.2	TTCN Changes	5

2.2.2.1	cs_MCPTT_UE_InitialConfiguration_OnNetwork	5

2.2.2.2	cs_MCVideo_ServiceConfiguration_OnNetwork	6

2.3	TTCN type definitions	7

2.3.1	MIKEY Messages (RFC 6509 and RFC 6043)	7

2.3.2	MIKEY general extension payload (33.180 E.6)	7

2.3.3	MCPTT Floor Control and Preestablished session control (24.380 clause 8)	7

2.3.3.1	FloorControl_ListOfSSRCs	7

2.3.3.2	FloorControl_Ack	8

2.3.3.3	PreestablishedSessionControl_Disconnect	9

2.3.3.4	New Floor Control Messages	10

2.3.4	MCVideo media plane control (24.581 clause 9)	10

2.3.4.1	New Fields for Functional Alias and Reception Mode	10

2.3.4.2	MCVideo_TransmissionControl_TransmissionRequest	11

2.3.4.3	MCVideo_TransmissionControl_MediaTransmissionNotification	12

2.3.4.4	MCVideo_TransmissionControl_ReceiveMediaRequest	13

2.3.4.5	MCVideo_TransmissionControl_MediaReceptionNotification	14

2.3.5	MCData messages (24.282 clause 15)	14

2.3.5.1	MCData_SDS_SIGNALLING_PAYLOAD, MCData_FD_SIGNALLING_PAYLOAD	14

2.3.5.2	MCData_DEFERRED_DATA_RESPONSE	17

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2020-09_D22wk37.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – Required Changes

# SIP/SDP types

No changes.

# XSD types

## XSD Modules

| IANA-resource-lists.xsd |  | RFC 4826 clause 3.2 (no changes) |
| --- | --- | --- |
| RFC4354-poc-settings.xsd |  | RFC 4354 clause 6.1 (no changes) |
| RFC4661-SimpleFilter.xsd |  | RFC 4661 clause 7 (no changes) |
| RFC5261-patch-ops.xsd |  | RFC 5261 clause 8 (no changes) |
| RFC5874-xcap-diff.xsd |  | RFC 5874 clause 4 (no changes) |
| poc_listService-v1_0.xsd |  | OMA - poc_listService-v1_0: "List Service" (no changes) |
| xdm_extensions-v1_0.xsd |  | OMA - xdm_extensions-v1_0: "XDM - XDM2 - Extensions" (no changes) |
| xdm_rsrclst_uriusage-v1_0.xsd |  | OMA - xdm_commonPolicy-V1_0: "XDM - Common Policy" (no changes) |
| xenc-schema.xsd |  | http://www.w3.org/2001/04/xmlenc#' (no changes) |
| xmldsig-core-schema.xsd |  | http://www.w3.org/2000/09/xmldsig# (no changes) |
| TS24281_mcvideoinfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24281_mcvideolocationinfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24281_mcvideoPresInfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24282_mcdatainfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24282_mcdatalocationinfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24282_mcdataPresInfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24379_mcpttaff.xsd | V17.8.0 | No changes |
| TS24379_mcpttinfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24379_mcpttlocationinfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24379_mcpttPresInfo.xsd | V17.8.0 | Updated; no impact on TTCN |
| TS24379_mcpttPresInfoFA.xsd | V17.8.0 | No changes |
| TS24379_mcpttsigneddoc.xsd | V17.8.0 | No changes |
| TS24379_poc-settings.xsd | V17.8.0 | No changes |
| TS24481-mcptt-group.xsd | V17.5.0 | Updated; no impact on TTCN |
| TS24481-mcpttGKTP.xsd | V17.5.0 | No changes |
| TS24481-mcpttGMOP.xsd | V17.5.0 | No changes |
| TS24484-mcdata-service-config.xsd | V17.7.0 | Updated; no impact on TTCN |
| TS24484-mcdata-ue-config.xsd | V17.7.0 | No changes |
| TS24484-mcdata-user-profile.xsd | V17.7.0 | Updated; no impact on TTCN |
| TS24484-mcptt-service-config.xsd | V17.7.0 | Updated; no impact on TTCN |
| TS24484-mcptt-ue-config.xsd | V17.7.0 | No changes |
| TS24484-mcptt-user-profile.xsd | V17.7.0 | Updated; no impact on TTCN |
| TS24484-mcvideo-service-config.xsd | V17.7.0 | Updated; requires TTCN changes as shown in 2.3.2 |
| TS24484-mcvideo-ue-config.xsd | V17.7.0 | No changes |
| TS24484-mcvideo-user-profile.xsd | V17.7.0 | Updated; no impact on TTCN |
| TS24484-ue-init-config.xsd | V17.7.0 | Updated; requires TTCN changes as shown in 2.3.1 |
| TS33180-mcsecKMSInterface.xsd | V17.7.0 | No changes |
| TS33180-mcsecKMSKRR.xsd | V17.7.0 | No changes |

## TTCN Changes

## cs_MCPTT_UE_InitialConfiguration_OnNetwork

| Reason for change | New optional fields in On-networkType |
| --- | --- |
| Summary of change | New optional fields added to cs_MCPTT_UE_InitialConfiguration_OnNetwork |
| TTCN module | MCX_XML_Configuration_Initial |

template (value) MCPTT_UE_InitialConfiguration_OnNetwork_Type cs_MCPTT_UE_InitialConfiguration_OnNetwork(charstring p_PLMN) := { /* @status APPROVED (MCX) */ index := omit, attr := omit, timers := { t100 := 2, t101 := 2, t103 := 5, t104 := 2, t132 := 3, anyExt := omit, elem_list := {} }, hPLMN := { // @sic R5s201409 BASELINE MOVING 2020 hPLM -> hPLMN sic@ pLMN := p_PLMN, service := cs_MCPTT_UE_InitialConfiguration_Service, vPLMN_list := {}, // @sic R5s201409 BASELINE MOVING 2020 vPLM -> vPLMN sic@ @sic R5-217985: no VPLMN sic@ anyExt := omit, elem_list := {} }, app_Server_Info := { idms_auth_endpoint := f_MCX_AbsoluteServerURI(-, px_MCX_IdMS_auth_IPAddress, px_MCX_IdMS_auth_Port, tsc_MCX_IdMS_auth_UriPath), // @sic R5-200385 sic@ idms_token_endpoint := f_MCX_AbsoluteServerURI(-, px_MCX_IdMS_token_IPAddress, px_MCX_IdMS_token_Port, tsc_MCX_IdMS_token_UriPath), // @sic R5-200385 sic@ http_proxy := f_MCX_AbsoluteServerURI(-, px_MCX_HTTP_Proxy_IPAddress, px_MCX_HTTP_Proxy_Port), gms := tsc_MCX_GMS_Hostname, // @sic R5-200385 sic@ cms := tsc_MCX_CMS_Hostname, // @sic R5-200385 sic@ kms := tsc_MCX_KMS_Hostname, // @sic R5-200385 sic@ tls_tunnel_auth_method := { mutual_authentication := false, x509 := omit, key := omit, anyExt := omit, elem_list := {} }, anyExt := omit, elem_list := {} }, gMS_URI := tsc_MCX_GMSURI, group_creation_XUI := px_MCX_GroupCreationXUI, gMS_XCAP_root_URI := tsc_MCX_GMSXCAPRootURI, cMS_XCAP_root_URI := tsc_MCX_CMSXCAPRootURI, integrity_protection_enabled := tsc_MCX_XML_IntegrityProtection, confidentiality_protection_enabled := tsc_MCX_XML_ConfidentialityProtection, anyExt := omit, elem_list := {} };

## cs_MCVideo_ServiceConfiguration_OnNetwork

| Reason for change | In Rel-17 new mandatory fields for configuration of the resource priorities have been added to the on-networkType in the MCVideo service configuration. |
| --- | --- |
| Summary of change | Addition of resource priorities to cs_MCVideo_ServiceConfiguration_OnNetwork. NOTE: A prose CR will be raised at TSGR5#97 to add the MCVideo resource priorities to 36.579-1 Table 5.5.8.8-1 |
| TTCN modules | MCX_XML_Configuration_MCVideo, MCX_CommonDefs |

const charstring tsc_MCVideo_EmergencyResourcePriorityNamespace := "mcpttp"; const charstring tsc_MCVideo_EmergencyResourcePriorityValue := "7"; const charstring tsc_MCVideo_ImminentPerilResourcePriorityNamespace := "mcpttp"; const charstring tsc_MCVideo_ImminentPerilResourcePriorityValue := "4"; const charstring tsc_MCVideo_NormalResourcePriorityNamespace := "mcpttp"; const charstring tsc_MCVideo_NormalResourcePriorityValue := "0";

template (value) urn_3gpp_ns_mcvideoServiceConfig_1_0.Resource_priorityType cs_MCVideo_ResourcePriority(charstring p_ResourcePriorityNamespace, charstring p_ResourcePriorityValue) := { attr := omit, resource_priority_namespace := p_ResourcePriorityNamespace, resource_priority_priority := p_ResourcePriorityValue, anyExt := omit, elem_list := {} };

template (value) MCVideo_ServiceConfiguration_OnNetwork_Type cs_MCVideo_ServiceConfiguration_OnNetwork := { attr := omit, signalling_protection := cs_MCVideo_ServiceConfiguration_Signalling_protection, protection_between_mcvideo_servers := cs_MCVideo_ServiceConfiguration_Server_protection, emergency_resource_priority := cs_MCVideo_ResourcePriority(tsc_MCVideo_EmergencyResourcePriorityNamespace, tsc_MCVideo_EmergencyResourcePriorityValue), imminent_peril_resource_priority := cs_MCVideo_ResourcePriority(tsc_MCVideo_ImminentPerilResourcePriorityNamespace, tsc_MCVideo_ImminentPerilResourcePriorityValue), normal_resource_priority := cs_MCVideo_ResourcePriority(tsc_MCVideo_NormalResourcePriorityNamespace, tsc_MCVideo_NormalResourcePriorityValue), anyExt := omit, elem_list := {} };

# TTCN type definitions

## MIKEY Messages (RFC 6509 and RFC 6043)

No changes in MIKEY_TypeDefs.

## MIKEY general extension payload (33.180 V17.7.0 E.6)

No changes in MIKEY_MCX_Extension_TypeDefs.

## MCPTT Floor Control and Preestablished session control (24.380 V17.7.0 clause 8)

## FloorControl_ListOfSSRCs

| Reason for change | Change of 24.380 Table 8.2.3.18-1 |
| --- | --- |
| Summary of change | Update of TTCN Definitions NOTE 1: The change of FloorControl_ListOfSSRCs is non-backward compatible as 'lgth' now is the number of octets (i.e. four times the number of SSRCs); this change has not been done for Rel-17 only but also for previous releases (e.g.Rel-15) NOTE 2: The 'lgth' field is not specified for any IE of 36.579-1 clause 5.5.6 but needs to be considered by the codec. |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type record FloorControl_ListOfSSRCs { // Rel-15 O1_Type id, O1_Type lgth, O1_Type noOfSSRCs, O1_Type spare, RTP_SSRC_List list };

template (value) FloorControl_ListOfSSRCs cs_FloorControl_ListOfSSRCs(RTP_SSRC_List p_SSRC_List) := { id := '10'O, lgth := int2oct(lengthof(p_SSRC_List) * 4, 1), noOfSSRCs := int2oct(lengthof(p_SSRC_List), 1), spare := '00'O, list := p_SSRC_List };

## FloorControl_Ack

| Reason for change | Changes of 24.380 Table 8.2.13-1 for Rel-16/17 |
| --- | --- |
| Summary of change | Update of TTCN Definitions NOTE: Change of cs_FloorCtrlMsg_Ack and cr_FloorCtrlMsg_Ack is subject to RAN5 approval. |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type record FloorControl_Ack { FloorControl_CommonHdr commonHdr, FloorControl_Source source, FloorControl_MsgType msgType, FloorControl_TrackInfo trackInfo optional, FloorControl_Location location optional };

template (value) FloorControlMsg_Type cs_FloorCtrlMsg_Ack(O1_Type p_MsgType) := { ackMessage := { commonHdr := cs_FloorControl_CommonHdr('01010'B, tsc_MCX_SSRC_NW), // ACK subtype 24.380 clause 8.2.2.1 source := cs_FloorControl_Source('0002'O), msgType := cs_FloorControl_MsgType(p_MsgType), trackInfo := omit, location := omit } };

template (present) FloorControlMsg_Type cr_FloorCtrlMsg_Ack(template (present) RTP_SSRC p_SSRC := ?, O1_Type p_MsgType) := { ackMessage := { commonHdr := cr_FloorControl_CommonHdr('01010'B, p_SSRC), // ACK subtype 24.380 clause 8.2.2.1 source := cs_FloorControl_Source('0000'O), msgType := cr_FloorControl_MsgType(p_MsgType), trackInfo := omit, location := cr_FloorControl_Location(crs_FloorControl_LocationValue_NotProvided) ifpresent } };

## PreestablishedSessionControl_Disconnect

| Reason for change | Changes of 24.380 Table 8.3.5-1 for Rel-17 |
| --- | --- |
| Summary of change | Update of TTCN Definitions NOTE: 24.380 does not specify when the Reason cause shall be provided to the client and what the client shall do with it. |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type record PreestablishedSessionControl_ReasonCause { O1_Type id, O1_Type lgth, O2_Type val };

type record PreestablishedSessionControl_Disconnect { PreestablishedSessionControl_CommonHdr commonHdr, PreestablishedSessionControl_SessionIdentity sessionIdentity optional, PreestablishedSessionControl_ReasonCause reasonCause optional };

template (value) PreestablishedSessionControlMsg_Type cs_PreestablishedSessionControl_Disconnect(template (value) PreestablishedSessionControl_SessionIdentity p_SessionIdentity, MCX_MediaControl_AckRequest_Type p_AckRequest) := { disconnectMessage := { commonHdr := cs_PreestablishedSessionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00001'B, p_AckRequest), tsc_MCX_SSRC_NW_P), sessionIdentity := p_SessionIdentity, reasonCause := omit } };

## New Floor Control Messages

New Rel-17 type definitions are added to MCPTT_MediaControl_TypeDefs:

- FloorControl_QueuedFloorRequestsPurpose
(24.380 clause 8.2.3.23)

- FloorControl_ListOfQueuedUsers
(24.380 clause 8.2.3.24)

- FloorControl_QueuedFloorRequestsResult
(24.380 clause 8.2.3.25)

- FloorControl_MediaFlowControlIndicator
(24.380 clause 8.2.3.26)

- FloorControl_QueuedFloorRequests
(24.380 clause 8.2.15)

- FloorControl_
(24.380 clause 8.2.16)

## MCVideo media plane control (24.581 V.17.2.0 clause 9)

## New Fields for Functional Alias and Reception Mode

| Reason for change | New clauses 9.2.3.21 and 9.2.3.22 in 24.581 for Rel-17 |
| --- | --- |
| Summary of change | New types TransmissionControl_FunctionalAlias, TransmissionControl_ReceptionMode |
| TTCN module | MCVideo_TransmissionControl_TypeDefs |

type record TransmissionControl_FunctionalAlias { O1_Type id, O1_Type lgth, octetstring val, octetstring padding };

type record TransmissionControl_ReceptionMode { O1_Type id, O1_Type lgth, O2_Type val };

## MCVideo_TransmissionControl_TransmissionRequest

| Reason for change | New field 'Functional Alias' in 24.581 Table 9.2.4-1 for Rel-17 |
| --- | --- |
| Summary of change | functionalAlias added to MCVideo_TransmissionControl_TransmissionRequest NOTE: the field is in 36.579-1 Table 5.5.11.1.1-1 already |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type record MCVideo_TransmissionControl_TransmissionRequest { TransmissionControl_CommonHdr commonHdr, TransmissionControl_TransmissionPriority transmissionPriority optional, TransmissionControl_UserID userId optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_FunctionalAlias functionalAlias optional }

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_TransmissionRequest(template (present) RTP_SSRC p_SSRC := ?, template (present) B16_Type p_TransmissionInd) := { // 36.579-1 Table 5.5.11.1.1-1 transmissionRequest := { commonHdr := cr_TransmissionControl_CommonHdr('?0000'B, p_SSRC, tsc_MCVideo_MediaControl_MCV0), transmissionPriority := cr_TransmissionControl_TransmissionPriority ifpresent, userId := omit, transmissionIndicator := cr_TransmissionControl_TransmissionIndicator(p_TransmissionInd), functionalAlias := omit }

## MCVideo_TransmissionControl_MediaTransmissionNotification

| Reason for change | New field 'Functional Alias' in 24.581 Table 9.2.13-1 for Rel-17 and new field 'Reception Mode' for Rel-15/16/17 |
| --- | --- |
| Summary of change | functionalAlias and receptionMode added to MCVideo_TransmissionControl_MediaTransmissionNotification NOTE 1: the field 'Functional Alias' is in 36.579-1 Table 5.5.11.2.7-1 already NOTE 2: the field 'Reception Mode' has been added down to Rel-15 of 24.581 and is mentioned even in Rel-14; it seems to be a non-backward compatible change as according to clauses 6.3.6.3.3 and 6.3.6.4.10 the server shall include a Reception Mode field set either to '0' or to '1' It is assumed that in general the field shall be included in the Media Transmission Notification. |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type record MCVideo_TransmissionControl_MediaTransmissionNotification { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserID userId optional, TransmissionControl_SSRC ssrcTransmitter optional, TransmissionControl_PermissionToRequest permissionToRequest optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_TrackInfo trackInfo optional, TransmissionControl_FunctionalAlias functionalAlias optional, // Rel-17 TransmissionControl_ReceptionMode receptionMode optional }

template (value) TransmissionControl_ReceptionMode cs_TransmissionControl_ReceptionMode(O2_Type p_Val) := { id := '16'O, lgth := '02'O, val := p_Val };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaTransmissionNotification(B16_Type p_TransmissionInd, charstring p_UserID_Transmitter, O4_Type p_SSRC_Transmitter, O2_Type p_PermissionToRequestTransmissionVal, O2_Type p_ReceptionModeVal, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.7-1 mediaTransmissionNotification := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00110'B, p_AckRequest), tsc_MCX_SSRC_NW, tsc_MCVideo_MediaControl_MCV1), userId := fl_TransmissionControl_UserID_TX(p_UserID_Transmitter), ssrcTransmitter := cs_TransmissionControl_SSRC(p_SSRC_Transmitter), permissionToRequest := cs_TransmissionControl_PermissionToRequestTransmission(p_PermissionToRequestTransmissionVal), transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd), trackInfo := omit, functionalAlias := omit, receptionMode := cs_TransmissionControl_ReceptionMode(p_ReceptionModeVal) } };

## MCVideo_TransmissionControl_ReceiveMediaRequest

| Reason for change | New field 'Functional Alias' in 24.581 Table 9.2.14-1 for Rel-17 |
| --- | --- |
| Summary of change | functionalAlias added to MCVideo_TransmissionControl_ReceiveMediaRequest NOTE: the field is in 36.579-1 Table 5.5.11.1.4-1 already |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type record MCVideo_TransmissionControl_ReceiveMediaRequest { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserID userId optional, TransmissionControl_SSRC ssrcTransmitter optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_ReceptionPriority receptionPriority optional, TransmissionControl_TrackInfo trackInfo optional, TransmissionControl_FunctionalAlias functionalAlias optional // Rel-17 }

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_ReceiveMediaRequest(template (present) RTP_SSRC p_SSRC, charstring p_UserID_Receiver, RTP_SSRC p_SSRC_Transmitter, template (present) B16_Type p_TransmissionInd) := { // 36.579-1 Table 5.5.11.1.4-1 receiveMediaRequest := { commonHdr := cr_TransmissionControl_CommonHdr('?0100'B, p_SSRC, tsc_MCVideo_MediaControl_MCV0), userId := fl_TransmissionControl_UserID_RX(p_UserID_Receiver), ssrcTransmitter := cr_TransmissionControl_SSRC(p_SSRC_Transmitter), transmissionIndicator := cr_TransmissionControl_TransmissionIndicator(p_TransmissionInd), receptionPriority := cr_TransmissionControl_ReceptionPriority ifpresent, trackInfo := omit, functionalAlias := omit } };

## MCVideo_TransmissionControl_MediaReceptionNotification

| Reason for change | New field 'Functional Alias' in 24.581 Table 9.2.16-1 for Rel-17 |
| --- | --- |
| Summary of change | functionalAlias added to MCVideo_TransmissionControl_MediaReceptionNotification NOTE: the field is in 36.579-1 Table 5.5.11.2.9-1 already |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type record MCVideo_TransmissionControl_MediaReceptionNotification { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserID userId optional, TransmissionControl_FunctionalAlias functionalAlias optional // Rel-17 }

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaReceptionNotification(MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED, charstring p_UserID_Receiver := px_MCVideo_ID_User_B) := { // 36.579-1 Table 5.5.11.2.9-1 mediaReceptionNotification := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('01000'B, p_AckRequest), tsc_MCX_SSRC_NW, tsc_MCVideo_MediaControl_MCV1), userId := fl_TransmissionControl_UserID_TX(p_UserID_Receiver), functionalAlias := omit } }

## MCData messages (24.282 V17.8.0 clause 15)

## MCData_SDS_SIGNALLING_PAYLOAD, MCData_FD_SIGNALLING_PAYLOAD

| Reason for change | Change of 24.282 Table 15.1.2.1-1 and Table 15.1.3.1-1 |
| --- | --- |
| Summary of change | Update of TTCN Definitions NOTE: Change of cs_MCData_SDS_SIGNALLING_PAYLOAD, cr_MCData_SDS_SIGNALLING_PAYLOAD, cs_MCData_FD_SIGNALLING_PAYLOAD and cr_MCData_FD_SIGNALLING_PAYLOAD is subject to RAN5 approval. |
| TTCN module | MCDataMessage_TypeDefs, MCData_MCDataMessages |

type record MCData_SDS_SIGNALLING_PAYLOAD { // Table 15.1.2.1-1 MCData_MessageType messageType, MCData_DateAndTime dateAndTime, MCData_ConversationID conversationID, MCData_MessageID messageID, MCData_InReplyToMessageID inReplyTo optional, MCData_ApplicationID applicationID optional, MCData_SDS_DispositionRequestType dispositionRequestType optional, MCData_ExtendedApplicationID extendedApplicationID optional, MCData_UserLocation userLocation optional, MCData_UserID senderUserID optional, MCData_ApplicationMetadata applicationMetadata optional };

type record MCData_FD_SIGNALLING_PAYLOAD { // Table 15.1.3.1-1 MCData_MessageType messageType, MCData_DateAndTime dateAndTime, MCData_ConversationID conversationID, MCData_MessageID messageID, MCData_InReplyToMessageID inReplyTo optional, MCData_ApplicationID applicationID optional, MCData_FD_DispositionRequestType dispositionRequestType optional, MCData_MandatoryDownload mandatoryDownload optional, MCData_Payload payload optional, MCData_Metadata metadata optional, MCData_ExtendedApplicationID extendedApplicationID optional, MCData_UserID senderUserID optional, MCData_ApplicationMetadata applicationMetadata optional };

type record MCData_ApplicationMetadata { // 24.282 clause 15.2.28 O1_Type iei optional, // '53'O O2_Type iel, octetstring content };

template (present) MCData_ApplicationMetadata cr_MCData_ApplicationMetadata(O1_Type p_IEI := '53'O) := { iei := p_IEI, iel := ?, content := ? };

template (value) MCData_Message cs_MCData_SDS_SIGNALLING_PAYLOAD(integer p_TimestampSeconds, MCData_ConversationID p_ConversationID, MCData_MessageID p_MessageID, H1_Type p_DispositionRequestType) := { SDS_SIGNALLING_PAYLOAD := { messageType := oct2bit('01'O), dateAndTime := int2oct(p_TimestampSeconds, 5), conversationID := p_ConversationID, messageID := p_MessageID, inReplyTo := omit, applicationID := omit, dispositionRequestType := cs_MCData_SDS_DispositionRequestType(p_DispositionRequestType), extendedApplicationID := omit, userLocation := omit, senderUserID := omit, applicationMetadata := omit } };

template (present) MCData_Message cr_MCData_SDS_SIGNALLING_PAYLOAD(template (present) H1_Type p_DispositionRequestType) := { SDS_SIGNALLING_PAYLOAD := { messageType := oct2bit('01'O), dateAndTime := ?, conversationID := ?, messageID := ?, inReplyTo := omit, applicationID := omit, dispositionRequestType := cr_MCData_SDS_DispositionRequestType(p_DispositionRequestType), extendedApplicationID := omit, userLocation := *, senderUserID := omit, applicationMetadata := cr_MCData_ApplicationMetadata ifpresent } };

template (value) MCData_Message cs_MCData_FD_SIGNALLING_PAYLOAD(integer p_TimestampSeconds, MCData_ConversationID p_ConversationID, MCData_MessageID p_MessageID, template (omit) MCData_FD_DispositionRequestType p_DispositionRequestType, template (omit) MCData_MandatoryDownload p_MandatoryDownload := omit, template (omit) MCData_Payload p_Payload := omit, template (omit) MCData_Metadata p_Metadata := omit) := { FD_SIGNALLING_PAYLOAD := { messageType := oct2bit('02'O), dateAndTime := int2oct(p_TimestampSeconds, 5), conversationID := p_ConversationID, messageID := p_MessageID, inReplyTo := omit, applicationID := omit, dispositionRequestType := p_DispositionRequestType, mandatoryDownload := p_MandatoryDownload, payload := p_Payload, metadata := p_Metadata, extendedApplicationID := omit, senderUserID := omit, applicationMetadata := omit } };

template (present) MCData_Message cr_MCData_FD_SIGNALLING_PAYLOAD(template MCData_FD_DispositionRequestType p_DispositionRequestType := cr_MCData_FD_DispositionRequestType, template MCData_MandatoryDownload p_MandatoryDownload := omit, template MCData_Payload p_Payload := omit, template MCData_Metadata p_Metadata := omit) := { FD_SIGNALLING_PAYLOAD := { messageType := oct2bit('02'O), dateAndTime := ?, conversationID := ?, messageID := ?, inReplyTo := omit, applicationID := omit, dispositionRequestType := p_DispositionRequestType, mandatoryDownload := p_MandatoryDownload, payload := p_Payload, metadata := p_Metadata, extendedApplicationID := omit, senderUserID := omit, applicationMetadata := cr_MCData_ApplicationMetadata ifpresent }

## MCData_DEFERRED_DATA_RESPONSE

| Reason for change | Change of 24.282 Table 15.1.12.1-1 |
| --- | --- |
| Summary of change | Update of TTCN Definitions NOTE: Change of cs_MCData_DEFERRED_DATA_RESPONSE is subject to RAN5 approval. |
| TTCN module | MCDataMessage_TypeDefs, MCData_MCDataMessages |
| MCC160 Comment |  |

type record MCData_DEFERRED_DATA_RESPONSE { // Table 15.1.12.1-1 MCData_MessageType messageType, MCData_NumberOfPayloads numberOfPayloads, MCData_ProtectedPayloadMessageList protectedPayloads, MCData_PayloadList payloads, MCData_GroupID groupID optional, MCData_DeferredPayloadList deferredPayloads };

type record MCData_DeferredPayload { // 24.282 clause 15.2.27 O1_Type iei optional, // '52'O O2_Type iel, octetstring content };

type record of MCData_DeferredPayload MCData_DeferredPayloadList;

template (value) MCData_Message cs_MCData_DEFERRED_DATA_RESPONSE(template (value) MCData_Payload p_Payload) := { DEFERRED_DATA_RESPONSE := { messageType := oct2bit('0C'O), numberOfPayloads := '01'O, protectedPayloads := {}, payloads := { p_Payload }, groupID := omit deferredPayloads := {} } };