3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221309

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2741 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The timing at Step 7 results in the same timing as in Step 3 as a result of the change proposed in TTCN CR R5s220470 for Step 3. Due to this, the Data at Step 7 does not get sent out. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The timing at Step 7 should take the timing calculated at Step 5 as reference to calculate the timing which satisfies the condition [(SFN * 10) + subframe number] modulo drx-LongCycle) = (drx-StartOffset) modulo (drx-LongCycle)) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcase 7.1.1.5.5	2

2.1	Correction to f_TC_7_1_1_5_5_NR_TestBody	2

2.2	Correction to f_NR_GetNextSendOccasion_7_1_1_5_X	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.5.5

## Correction to f_TC_7_1_1_5_5_NR_TestBody

| Function name | f_TC_7_1_1_5_5_NR_TestBody() |
| --- | --- |
| Reason for change | The timing at Step 7 results in the same timing as in Step 3 as a result of the change proposed in TTCN CR R5s220470 for Step 3. Due to this, the Data at Step 7 does not get sent out. |
| Summary of change | The timing at Step 7 should take the timing calculated at Step 5 as reference to calculate the timing which satisfies the condition [(SFN * 10) + subframe number] modulo drx-LongCycle) = (drx-StartOffset) modulo (drx-LongCycle)) |
| TTCN module | DRX_MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID_SCG_Bearer, v_EncodedRlcPdu1) };//@sic R5s211665 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); //@sic R5s220740 sic@ // The other things in Step 3 automatically taken care by SS //@sic R5s210582 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ // 5 ms later configuere SS for normal CRC error mode // In the Last PDCCH occasion befor on duration timer expiry v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInSubframe * (tsc_DrxOnDurationTimer - 1)); //@sic R5s201495 R5s210677 sic@ v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) }; // v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit, cs_NR_MAC_Padding_SubPDU('000000'O)) }; // 2 byte sub headers +3byte Padding header= 40 bits DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2));//@sic R5s210677 sic@ //@siclog "Step 7" siclog@ v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL));//@sic R5s210677 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID_SCG_Bearer, v_EncodedRlcPdu1) };//@sic R5s211665 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); //@sic R5s220740 sic@ // The other things in Step 3 automatically taken care by SS //@sic R5s210582 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ // 5 ms later configuere SS for normal CRC error mode // In the Last PDCCH occasion befor on duration timer expiry v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInSubframe * (tsc_DrxOnDurationTimer - 1)); //@sic R5s201495 R5s210677 sic@ v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) }; // v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit, cs_NR_MAC_Padding_SubPDU('000000'O)) }; // 2 byte sub headers +3byte Padding header= 40 bits DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2));//@sic R5s210677 sic@ //@siclog "Step 7" siclog@ //WA# v_Timing := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset, -, v_Timing); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL));//@sic R5s210677 sic@ <<SKIPPED CODE>>

## Correction to f_NR_GetNextSendOccasion_7_1_1_5_X

| Function name | f_NR_GetNextSendOccasion_7_1_1_5_X () |
| --- | --- |
| Reason for change | The timing at Step 7 results in the same timing as in Step 3 as a result of the change proposed in TTCN CR R5s220470 for Step 3. Due to this, the Data at Step 7 does not get sent out. |
| Summary of change | The timing at Step 7 should take the timing calculated at Step 5 as reference to calculate the timing which satisfies the condition [(SFN * 10) + subframe number] modulo drx-LongCycle) = (drx-StartOffset) modulo (drx-LongCycle)) |
| TTCN module | DRX_MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_NR_GetNextSendOccasion_7_1_1_5_X(NR_CellId_Type p_NR_CellId, integer p_DrxCycle, integer p_DrxStartOffset, integer p_CountY :=0 // in slots ) runs on NR_BASE_PTC return SubFrameTiming_Type { // Function does not consider drx-SlotOffset as it is always set to 0 in all 7.1.1.5.x test cases <<SKIPPED CODE>> select (v_SCS){ case (kHz15) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology0;} case (kHz30) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology1(p_CountY mod v_SlotsInSubframe);} case (kHz60) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology2(p_CountY mod v_SlotsInSubframe); } case (kHz120){v_SlotTimingInfo := cs_SlotTimingInfo_Numerology3(p_CountY mod v_SlotsInSubframe); } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } // 2 cycles ahead is selected to provide enough time for SS v_DrxNumberofCycle := (v_SubFrameInfo.SFN.Number * 10 + v_SubFrameInfo.Subframe.Number + (p_CountY / v_SlotsInSubframe))/p_DrxCycle; // 2 cycles ahead is selected to provide enough time for SS v_DRXnumberofSubframe := p_DrxCycle * (v_DrxNumberofCycle + 2) + p_DrxStartOffset; v_SubframeNumber := v_DRXnumberofSubframe mod 10 ; v_FrameNumber := v_DRXnumberofSubframe / 10; //v_FrameNumber := (((v_SubFrameInfo.SFN.Number / v_DRXFrame) + 2) * v_DRXFrame) ; //v_FrameNumber := (v_FrameNumber + (p_DrxStartOffset / 10)+ p_CountY/(10 * v_SlotsInSubframe)); v_HSFN := (v_SubFrameInfo.HSFN.Number + (v_FrameNumber / 1024)) mod 1024; v_FrameNumber := (v_FrameNumber) mod 1024; v_Timing := valueof(cs_SubFrameTiming(v_FrameNumber, v_SubframeNumber, v_HSFN)); v_Timing.Slot := valueof(v_SlotTimingInfo); return v_Timing; };

After Change:

function f_NR_GetNextSendOccasion_7_1_1_5_X(NR_CellId_Type p_NR_CellId, integer p_DrxCycle, integer p_DrxStartOffset, integer p_CountY :=0, // in slots template (omit) SubFrameTiming_Type p_Timing := omit ) runs on NR_BASE_PTC return SubFrameTiming_Type { // Function does not consider drx-SlotOffset as it is always set to 0 in all 7.1.1.5.x test cases <<SKIPPED CODE>> select (v_SCS){ case (kHz15) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology0;} case (kHz30) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology1(p_CountY mod v_SlotsInSubframe);} case (kHz60) {v_SlotTimingInfo := cs_SlotTimingInfo_Numerology2(p_CountY mod v_SlotsInSubframe); } case (kHz120){v_SlotTimingInfo := cs_SlotTimingInfo_Numerology3(p_CountY mod v_SlotsInSubframe); } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } //WA# if(ispresent(p_Timing)){ v_SubFrameInfo := valueof(p_Timing); } // 2 cycles ahead is selected to provide enough time for SS v_DrxNumberofCycle := (v_SubFrameInfo.SFN.Number * 10 + v_SubFrameInfo.Subframe.Number + (p_CountY / v_SlotsInSubframe))/p_DrxCycle; // 2 cycles ahead is selected to provide enough time for SS v_DRXnumberofSubframe := p_DrxCycle * (v_DrxNumberofCycle + 2) + p_DrxStartOffset; v_SubframeNumber := v_DRXnumberofSubframe mod 10 ; v_FrameNumber := v_DRXnumberofSubframe / 10; //v_FrameNumber := (((v_SubFrameInfo.SFN.Number / v_DRXFrame) + 2) * v_DRXFrame) ; //v_FrameNumber := (v_FrameNumber + (p_DrxStartOffset / 10)+ p_CountY/(10 * v_SlotsInSubframe)); v_HSFN := (v_SubFrameInfo.HSFN.Number + (v_FrameNumber / 1024)) mod 1024; v_FrameNumber := (v_FrameNumber) mod 1024; v_Timing := valueof(cs_SubFrameTiming(v_FrameNumber, v_SubframeNumber, v_HSFN)); v_Timing.Slot := valueof(v_SlotTimingInfo); return v_Timing; };