3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221315

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2854 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 8.2.2.1.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.2.2.1.2 to the IWD_22wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.2.1.2 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_NR_ReconfigureAddSRB3_NRDC ()	4

2.2	Function f_TC_8_2_2_1_2_NR5GC_TestBody ()	5

3	Branches executed	9

4	Execution Log Files	9

4.1	Mediatek MT6879 UE	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.2.2.1.2  which is part of the NR5GC test suite in iwd-TTCN3-B2020-09_D22wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.2.2.1.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Corrections required

## Function f_NR_ReconfigureAddSRB3_NRDC ()

| Function name | f_NR_ReconfigureAddSRB3_NRDC () |
| --- | --- |
| Reason for change | In the current implementation , while configuring SRB3 , SDAP is set to Omit ; but according to 38523-03 , Omit is applicable only for EN-DC. Hence it is proposed to have SDAP as cs_SDAP_Configuration_None |
| Summary of change | SDAP configuration is changed from omit to cs_SDAP_Configuration_None |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ReconfigureAddSRB3_NRDC(NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR5GC_PTC { var template (value) RadioBearerConfig v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, omit, omit, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); var template (value) CellGroupConfig v_CellGroupConfigSCG := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_SCG_CellId),{cs_38508_RLC_BearerConfig_SRB(tsc_SRB3)}); var template (value) NR_RadioBearer_Type v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority) ,omit); << SKIPPED CODE >> }

After Change:

function f_NR_ReconfigureAddSRB3_NRDC(NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR5GC_PTC { var template (value) RadioBearerConfig v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, omit, omit, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); var template (value) CellGroupConfig v_CellGroupConfigSCG := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_SCG_CellId),{cs_38508_RLC_BearerConfig_SRB(tsc_SRB3)}); var template (value) NR_RadioBearer_Type v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority) ,cs_SDAP_Configuration_None); //WA#WI=1003889 << SKIPPED CODE >> }

## Function f_TC_8_2_2_1_2_NR5GC_TestBody ()

| Function name | f_TC_8_2_2_1_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | In the current Implementation , at step 5 when there is reconfiguration message is sent to reconfigure Pdcp , release and reconfigure of SRB3 is not required at SS side as it will reset the SN |
| Summary of change | SS configurations to release and reconfigure the SRB3 are removed at step 5 |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_2_1_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) CellGroupConfig v_SCGConfig; var template (omit) octetstring v_EncodedSCG_Config; var template (omit) RadioBearerConfig v_RadioBearerConfig; var template (omit) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RadioBearer_Type v_SS_ConfigSCG; var template (value) DRX_Config v_DRX_Config; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(nr_Cell10); var SubFrameTiming_Type v_Timing_NextSendOccasion; var SubFrameTiming_Type v_Timing; //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message via SRB1 to establish SRB3. //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB1? f_NR_ReconfigureAddSRB3_NRDC(nr_Cell1, nr_Cell10); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure NR MAC. if (f_NR_IsBandFR1(v_FreqBandIndicatorNR)) { // FR1 v_DRX_Config := cs_38508_DRX_Config_FR1; } else { // FR2 v_DRX_Config := cs_38508_DRX_Config_FR2; } v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(cs_NR_DRX_Config_Setup(v_DRX_Config))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config))); //Reconfigure SS with mac_CellGroupConfig f_NR_CellInfo_SetDrxCtrl(nr_Cell10, valueof(cs_NR_DrxCtrl_Config(v_DRX_Config))); f_NR_SS_ServingCellConfig(nr_Cell10, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_SCGConfig.mac_CellGroupConfig, omit))); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure PDCP. v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell10); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB3)}); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, cs_TimingInfo_NR(v_Timing_NextSendOccasion), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig))); //Release and reconfigure SRB3 in SS v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB3); f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {v_SS_ConfigSCG},cs_TimingInfo_NR(v_Timing)); v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 10); v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority), omit); f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {v_SS_ConfigSCG}, cs_TimingInfo_NR(v_Timing)); //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); << SKIPPED CODE >> }

After Change:

function f_TC_8_2_2_1_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) CellGroupConfig v_SCGConfig; var template (omit) octetstring v_EncodedSCG_Config; var template (omit) RadioBearerConfig v_RadioBearerConfig; var template (omit) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RadioBearer_Type v_SS_ConfigSCG; var template (value) DRX_Config v_DRX_Config; var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(nr_Cell10); var SubFrameTiming_Type v_Timing_NextSendOccasion; var SubFrameTiming_Type v_Timing; //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message via SRB1 to establish SRB3. //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB1? f_NR_ReconfigureAddSRB3_NRDC(nr_Cell1, nr_Cell10); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure NR MAC. if (f_NR_IsBandFR1(v_FreqBandIndicatorNR)) { // FR1 v_DRX_Config := cs_38508_DRX_Config_FR1; } else { // FR2 v_DRX_Config := cs_38508_DRX_Config_FR2; } v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(cs_NR_DRX_Config_Setup(v_DRX_Config))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config))); //Reconfigure SS with mac_CellGroupConfig f_NR_CellInfo_SetDrxCtrl(nr_Cell10, valueof(cs_NR_DrxCtrl_Config(v_DRX_Config))); f_NR_SS_ServingCellConfig(nr_Cell10, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_SCGConfig.mac_CellGroupConfig, omit))); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure PDCP. v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell10); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB3)}); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, cs_TimingInfo_NR(v_Timing_NextSendOccasion), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig))); //Release and reconfigure SRB3 in SS //Can be removed //WA#WI=1003889 /* v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB3); f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {v_SS_ConfigSCG},cs_TimingInfo_NR(v_Timing)); v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 10); v_SS_ConfigSCG := cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority), cs_SDAP_Configuration_None); //WA#WI=1003889 f_NR_SS_CommonRadioBearerConfig(nr_Cell10, {v_SS_ConfigSCG}, cs_TimingInfo_NR(v_Timing));*/ //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); << SKIPPED CODE >> }

# Branches executed

## This NR TC was executed in NR-DC FR1+FR2 Band Combination DC_n78A_n257A with EEA1/EIA1

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_2_2_1_2_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221316: Supporting information for addition of NR-DC test case 8.2.2.1.2 in FR1+FR2 |
| --- | --- |