3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221317

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0882 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS Call Control test case 7.33 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 34.229-5 Table 7.33.3.3-4, 200OK at Steps 22 and 25 should include SDP message. Current implementation does not expect the 200OK sent by the UE to include an SDP message causing the test case to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added SDP body to 200OK expected template at steps 22 and 25. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.33.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	New function f_IMS_CallWithSessionTimer_Send200OK_SDP	3

2.2	Correction to function f_IMS_CallWithSessionTimer_MORefresh	3

2.3	Correction to function f_TC_7_33_NR5GC_IMS1	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## New function f_IMS_CallWithSessionTimer_Send200OK_SDP

| Function name | f_IMS_CallWithSessionTimer_Send200OK_SDP |
| --- | --- |
| Reason for change | Current implementation does not expect the 200OK sent by the UE to include a SDP message causing the test case to fail. |
| Summary of change | Added SDP body to 200OK expected template at steps 22 and 25. |
| TTCN module | IMS\Common\IMS_CC_SessionTimerCommon.ttcn |
| MCC160 Comment |  |

New function:

<<SKIPPED CODE>> function f_IMS_CallWithSessionTimer_Send200OK_SDP(INVITE_Request p_Invite, template (value) IMS_RoutingInfo_Type p_RoutingInfo_DL, template (omit) Allow p_Allow := omit, template (omit) Require p_Require := omit, template (omit) Supported p_Supported := omit, template (omit) SessionExpires p_SessionExpires := omit, template (omit) MinSE p_MinSE := omit, charstring p_ContactUri := px_IMS_CalleeContactUri) runs on IMS_PTC { /* @sic R5s200334: p_ContactUri sic@ */ /* @sic R5-201447: p_Allow sic@ */ var template (value) MessageHeader v_MessageHeader_200; var template (value) MessageBody v_MessageBody; // Send 200 OK v_MessageHeader_200 := f_IMS_OtherResponse_200_MessageHeaderTX(p_Invite.msgHeader, cs_ContentTypeSDP, -, p_ContactUri, true); v_MessageHeader_200.allow := p_Allow; v_MessageHeader_200.require := p_Require; v_MessageHeader_200.supported := p_Supported; v_MessageHeader_200.sessionExpires := p_SessionExpires; v_MessageHeader_200.minSE := p_MinSE; IMS_Server.send(cas_IMS_DATA_RSP(p_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_200, p_Invite.messageBody))); } <<SKIPPED CODE>>

## Correction to function f_IMS_CallWithSessionTimer_MORefresh

| Function name | f_IMS_CallWithSessionTimer_MORefresh |
| --- | --- |
| Reason for change | CR R5s220782 was raised to address an issue for test case 7.33 which states that 200OK at steps 21-26 should include SDP message. This CR was raised in 22wk23 but it has not been included in 22wk37. Present CR is submitted to include again the changes proposed in R5s220782. |
| Summary of change | Added changes proposed in R5s220782. |
| TTCN module | \IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_CallWithSessionTimer_MORefresh(SipUrl p_ContactUrl, template (omit) Require p_Require := omit, template (omit) Supported p_Supported := omit, integer p_DeltaSecValue, charstring p_RefresherValue) runs on IMS_PTC { /* @sic R5-210124: 200 OK and ACK added to this function sic@ */ /* @sic R5s210678 change 2: p_Require, p_Supported sic@ */ var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderExpected; var INVITE_Request v_InviteRequest; var template (omit) SDP_Message v_SDP_Message; v_MessageHeaderExpected := f_IMS_InviteRequest_MessageHeaderRX(p_ContactUrl, tsc_OptionTagList_Empty, A_2_1_A5); // @sic R5-214531, R5-214553, R5-214450: tsc_OptionTagList_Empty sic@ v_MessageHeaderExpected.sessionExpires := cr_SessionExpires; // Receive INVITE v_IMS_DATA_REQ := f_IMS_CallWithSessionTimer_ReceiveRefreshRequest(p_DeltaSecValue, car_IMS_Invite_Request(cr_INVITE_Request(p_ContactUrl, v_MessageHeaderExpected, cr_MessageBody_SDP))); v_RoutingInfo_DL := v_IMS_DATA_REQ.RoutingInfo; f_IMS_A_2_1_Invite_CommonChecks(v_IMS_DATA_REQ, A_2_1_A5); f_IMS_SIP_DecodeMatchAndCheckSDP(v_IMS_DATA_REQ.Request.Invite.messageBody.sdpMessageBody, cr_SDP_Message_Common, sameOriginAsPreviousSDP); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; f_IMS_CallWithSessionTimer_CheckSessionExpires(v_InviteRequest.msgHeader.sessionExpires, p_DeltaSecValue, p_RefresherValue); // Send 200 OK v_SDP_Message := f_IMS_PTC_ImsInfo_DialogGetPrevSdpMessageTX(); // @sic R5-225427, R5-225258 sic@ f_IMS_CallWithSessionTimer_Send200OK(v_InviteRequest.msgHeader, v_RoutingInfo_DL, -, p_Require, p_Supported, cs_SessionExpires(p_DeltaSecValue, "uac"), -, f_SIP_SipUrl2Str(p_ContactUrl), v_SDP_Message); // Receive ACK f_IMS_CallWithSessionTimer_ReceiveACK(v_InviteRequest, p_ContactUrl); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_CallWithSessionTimer_MORefresh(SipUrl p_ContactUrl, template (omit) Require p_Require := omit, template (omit) Supported p_Supported := omit, integer p_DeltaSecValue, charstring p_RefresherValue, boolean p_SDPRequired := false) runs on IMS_PTC { /* @sic R5-210124: 200 OK and ACK added to this function sic@ */ /* @sic R5s210678 change 2: p_Require, p_Supported sic@ */ var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderExpected; var INVITE_Request v_InviteRequest; var template (omit) SDP_Message v_SDP_Message; v_MessageHeaderExpected := f_IMS_InviteRequest_MessageHeaderRX(p_ContactUrl, tsc_OptionTagList_Empty, A_2_1_A5); // @sic R5-214531, R5-214553, R5-214450: tsc_OptionTagList_Empty sic@ v_MessageHeaderExpected.sessionExpires := cr_SessionExpires; // Receive INVITE v_IMS_DATA_REQ := f_IMS_CallWithSessionTimer_ReceiveRefreshRequest(p_DeltaSecValue, car_IMS_Invite_Request(cr_INVITE_Request(p_ContactUrl, v_MessageHeaderExpected, cr_MessageBody_SDP))); v_RoutingInfo_DL := v_IMS_DATA_REQ.RoutingInfo; f_IMS_A_2_1_Invite_CommonChecks(v_IMS_DATA_REQ, A_2_1_A5); f_IMS_SIP_DecodeMatchAndCheckSDP(v_IMS_DATA_REQ.Request.Invite.messageBody.sdpMessageBody, cr_SDP_Message_Common, sameOriginAsPreviousSDP); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; f_IMS_CallWithSessionTimer_CheckSessionExpires(v_InviteRequest.msgHeader.sessionExpires, p_DeltaSecValue, p_RefresherValue); // Send 200 OK if(p_SDPRequired) { f_IMS_CallWithSessionTimer_Send200OK_SDP(v_InviteRequest, v_RoutingInfo_DL, -, p_Require, p_Supported, cs_SessionExpires(p_DeltaSecValue, "uac"), -, f_SIP_SipUrl2Str(p_ContactUrl)); } else { f_IMS_CallWithSessionTimer_Send200OK(v_InviteRequest.msgHeader, v_RoutingInfo_DL, -, p_Require, p_Supported, cs_SessionExpires(p_DeltaSecValue, "uac"), -, f_SIP_SipUrl2Str(p_ContactUrl)); } // Receive ACK f_IMS_CallWithSessionTimer_ReceiveACK(v_InviteRequest, p_ContactUrl); } <<SKIPPED CODE>>

## Correction to function f_TC_7_33_NR5GC_IMS1

| Function name | f_TC_7_33_NR5GC_IMS1 |
| --- | --- |
| Reason for change | CR R5s220782 was raised to address an issue for test case 7.33 which states that 200OK at steps 21-26 should include SDP message. This CR was raised in 22wk23 but it has not been included in 22wk37. Present CR is submitted to include again the changes proposed in R5s220782. |
| Summary of change | Added changes proposed in R5s220782. |
| TTCN module | \IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"}; var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var integer v_DeltaSec := 1800; var integer i; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9" siclog@ @sic R5-213545: A.5.2 instead of A.5.1 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions); v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas"); v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); // @siclog "Step 10 - 16" IPCAN siclog@ f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp)); // @sic R5-213545: A.5.2 instead of A.5.1 sic@ // @siclog "Steps 17" siclog@ f_UT_AnswerCall(MMI); // @siclog "Steps 18" Void siclog@ // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), -, -, v_DeltaSec, "uas"); /* @sic R5s210678 change 1: cr_Supported_timer instead of cr_Require_timer sic@ @sic R5-214450: cr_Supported_timer removed sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Steps 21 - 26" siclog@ for (i := 0; i < 2; i := i + 1) { f_IMS_CallWithSessionTimer_MORefresh(v_ContactUrlSS, cs_Require_timer, -, v_DeltaSec, "uac"); /* @sic R5-210124: common function sic@ @sic R5s210678 change 2: cs_Require_timer as parameter sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21/24"); } // @siclog "Step 20 - 23" siclog@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end chooses UE as refresher / 5GS var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"}; var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var SipUrl v_ContactUrlSS; var integer v_DeltaSec := 1800; var integer i; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9" siclog@ @sic R5-213545: A.5.2 instead of A.5.1 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions); v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas"); v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); // @siclog "Step 10 - 16" IPCAN siclog@ f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp)); // @sic R5-213545: A.5.2 instead of A.5.1 sic@ // @siclog "Steps 17" siclog@ f_UT_AnswerCall(MMI); // @siclog "Steps 18" Void siclog@ // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), -, -, v_DeltaSec, "uas"); /* @sic R5s210678 change 1: cr_Supported_timer instead of cr_Require_timer sic@ @sic R5-214450: cr_Supported_timer removed sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Steps 21 - 26" siclog@ for (i := 0; i < 2; i := i + 1) { f_IMS_CallWithSessionTimer_MORefresh(v_ContactUrlSS, cs_Require_timer, -, v_DeltaSec, "uac", true); /* @sic R5-210124: common function sic@ @sic R5s210678 change 2: cs_Require_timer as parameter sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21/24"); } // @siclog "Step 20 - 23" siclog@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); } <<SKIPPED CODE>>