3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221319

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0883 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS registration test cases 6.2 and 6.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For subsequent initial registration attempts, a UE operating in IPv4v6 mode may switch between use of IPv4 and IPv6 addresses. The current TTCN implementation does not always update the routing info for the response which can caused some SIP responses to be sent on the wrong IP address (IPv4 instead of IPv6 or vice versa) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refresh v_RoutingInfoDL on receiving each initial REGISTER request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.NR5GC, 6.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_NR5GC_IMS1() |
| --- | --- |
| Reason for change | For subsequent initial registration attempts, a UE operating in IPv4v6 mode may switch between use of IPv4 and IPv6 addresses. The current TTCN implementation does not always update the routing info for the response which can caused some SIP responses to be sent on the wrong IP address (IPv4 instead of IPv6 or vice versa) |
| Summary of change | Refresh v_RoutingInfoDL on receiving each initial REGISTER request |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2), var integer v_NumOfIPAddressesToAllocate := 2; // @sic R5s210720 sic@ var charstring v_RegisterExpiration; var integer v_Cseq; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); /* @sic R5-213534: support of 2nd P-CSCF, R5-217830 sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); t_Timer.start(f_IMS_SetTimerToleranceMin(10.0)); /* @sic R5s210720: timer to be started before RRC connection release sic@ */ f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ t_Timer.timeout; /* @sic R5s210720: t_Timer.start/stop instead of f_Delay sic@ */ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

After Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2), var integer v_NumOfIPAddressesToAllocate := 2; // @sic R5s210720 sic@ var charstring v_RegisterExpiration; var integer v_Cseq; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); /* @sic R5-213534: support of 2nd P-CSCF, R5-217830 sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); t_Timer.start(f_IMS_SetTimerToleranceMin(10.0)); /* @sic R5s210720: timer to be started before RRC connection release sic@ */ f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ t_Timer.timeout; /* @sic R5s210720: t_Timer.start/stop instead of f_Delay sic@ */ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

## Change 2

| Function name | f_TC_6_8_NR5GC_IMS1() |
| --- | --- |
| Reason for change | For subsequent initial registration attempts, a UE operating in IPv4v6 mode may switch between use of IPv4 and IPv6 addresses. The current TTCN implementation does not always update the routing info for the response which can caused some SIP responses to be sent on the wrong IP address (IPv4 instead of IPv6 or vice versa) |
| Summary of change | Refresh v_RoutingInfoDL on receiving each initial REGISTER request |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_8_NR5GC_IMS1() runs on IMS_PTC { var charstring v_InvalidNonce := f_IMS_Authentication_GenerateInvalidNonce(sqnFailure); var IMS_DATA_REQ v_IMS_REGISTER_REQ; var SecurityClientParamsList_Type v_SecurityClientParamsList; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var REGISTER_Request v_RegisterReq; var charstring v_CallId_Step2; var charstring v_CallId_Step4; var template (value) MessageHeader v_MessageHeader_RegisterResponse_401; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_CallId_Step2 := v_RegisterReq.msgHeader.callId.callid; // @siclog "Step 3" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, -, false); v_MessageHeader_RegisterResponse_401 := f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq); v_MessageHeader_RegisterResponse_401.securityServer := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, v_MessageHeader_RegisterResponse_401))); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_CallId_Step4 := v_RegisterReq.msgHeader.callId.callid; if (v_CallId_Step4 == v_CallId_Step2) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "same Call-ID as at step 2"); } v_SecurityClientParamsList[0] := f_IMS_Register_GetSecurityClientParams(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, -, false); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq, v_InvalidNonce)))); // @siclog "Step 6" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_RequestWithSpecificAuth(v_CallId_Step4, cr_Authorization_SqnOutOfRange(v_InvalidNonce)); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_SecurityClientParamsList := f_IMS_Authentication_CheckSecurityClientParams(v_SecurityClientParamsList, v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 9" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10 - 13" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_InitialRegistration); }

After Change:

function f_TC_6_8_NR5GC_IMS1() runs on IMS_PTC { var charstring v_InvalidNonce := f_IMS_Authentication_GenerateInvalidNonce(sqnFailure); var IMS_DATA_REQ v_IMS_REGISTER_REQ; var SecurityClientParamsList_Type v_SecurityClientParamsList; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var REGISTER_Request v_RegisterReq; var charstring v_CallId_Step2; var charstring v_CallId_Step4; var template (value) MessageHeader v_MessageHeader_RegisterResponse_401; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_CallId_Step2 := v_RegisterReq.msgHeader.callId.callid; // @siclog "Step 3" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, -, false); v_MessageHeader_RegisterResponse_401 := f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq); v_MessageHeader_RegisterResponse_401.securityServer := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, v_MessageHeader_RegisterResponse_401))); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_CallId_Step4 := v_RegisterReq.msgHeader.callId.callid; if (v_CallId_Step4 == v_CallId_Step2) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "same Call-ID as at step 2"); } v_SecurityClientParamsList[0] := f_IMS_Register_GetSecurityClientParams(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq, -, false); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq, v_InvalidNonce)))); // @siclog "Step 6" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_RequestWithSpecificAuth(v_CallId_Step4, cr_Authorization_SqnOutOfRange(v_InvalidNonce)); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_SecurityClientParamsList := f_IMS_Authentication_CheckSecurityClientParams(v_SecurityClientParamsList, v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 9" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10 - 13" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_InitialRegistration); }