3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221331

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2863 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC testcase 8.2.3.16.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.2.3.16.2 to the IWD_22wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.3.16.2 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.16.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function fl_TC_8_2_3_16_2_NR5GC_TestBody ()	4

2.2	Template cr_38508_MeasResults()	7

2.3	Function f_NR_ServCellIndex()	9

3	Branches executed	10

4	Execution Log Files	10

4.1	Mediatek MT6879 UE	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.2.3.16.2  which is part of the NR5GC test suite in iwd-TTCN3-B2020-09_D22wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.2.3.16.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Corrections required

## Function fl_TC_8_2_3_16_2_NR5GC_TestBody ()

| Function name | fl_TC_8_2_3_16_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | In the current implementation of the test case at step 5, when TTCN is expecting the measurement report containing measured RSRP values of NR Cell 10 ( SCG Cell ) , serving cell index has to be filled as 1 in the template cr_38508_MeasResults |
| Summary of change | To fill serving cell index as 1 in the template cr_38508_MeasResults , boolean parameter p_NRDC_SCG_Cell is used |
| TTCN module | RRC_Measurement_NRDC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_3_16_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (omit) octetstring v_EncodedSCG_Config := omit; var PhysCellId v_PhysCellIdServing; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; // @siclog "Step 1 and Step2" siclog@ // The SS transmits an RRCReconfiguration message via SRB1 to establish SRB3 // The UE transmits an RRCReconfigurationComplete message via SRB1 f_NR_ReconfigureAddSRB3_NRDC(nr_Cell1, nr_Cell10); // @siclog "Step 3" siclog@ // The SS transmits an RRCReconfiguration message from Cell10 to setup periodical intra-NR measurements and reporting via SRB3 v_SCGConfig := cs_NR_CellGroupConfigMeas(tsc_NR_CellGroupId_SCG, cs_NR_SpCellConfigDef(tsc_NR_CellGroupId_SCG, -, -, cs_NR_ServingCellConfigMeas(tsc_NR_MeasObjectId1))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell10)}; v_ReportConfigList := cs_NR_ReportConfigToAddModList_1Entry(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical); v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config, v_MeasConfig))); // @siclog "Step 4" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ // Check: Does the UE transmits at least 10 MeasurementReport messges to periodically report the measured RSRP value for NR Cell 10 // via SRB3 within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell10); v_MeasResults := cr_38508_MeasResults(tsc_NR_MeasId1, v_PhysCellIdServing); f_NR_ReceivePeriodicalMeasurementReports(nr_Cell10, 30.0, 10, cr_NR_MeasurementReport(v_MeasResults), false); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); activate(a_NR_AddDefault_IgnorePeriodicalMeasurmentReport_SRB3(nr_Cell10, cr_NR_MeasurementReport(v_MeasResults))); } // End of fl_TC_8_2_3_16_2_TestBody

After Change:

function fl_TC_8_2_3_16_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (omit) octetstring v_EncodedSCG_Config := omit; var PhysCellId v_PhysCellIdServing; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; var boolean p_NRDC_SCG_Cell := true; //WA#WI=1028438 // @siclog "Step 1 and Step2" siclog@ // The SS transmits an RRCReconfiguration message via SRB1 to establish SRB3 // The UE transmits an RRCReconfigurationComplete message via SRB1 f_NR_ReconfigureAddSRB3_NRDC(nr_Cell1, nr_Cell10); // @siclog "Step 3" siclog@ // The SS transmits an RRCReconfiguration message from Cell10 to setup periodical intra-NR measurements and reporting via SRB3 v_SCGConfig := cs_NR_CellGroupConfigMeas(tsc_NR_CellGroupId_SCG, cs_NR_SpCellConfigDef(tsc_NR_CellGroupId_SCG, -, -, cs_NR_ServingCellConfigMeas(tsc_NR_MeasObjectId1))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell10)}; v_ReportConfigList := cs_NR_ReportConfigToAddModList_1Entry(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical); v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config, v_MeasConfig))); // @siclog "Step 4" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message via SRB3? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell10, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ // Check: Does the UE transmits at least 10 MeasurementReport messges to periodically report the measured RSRP value for NR Cell 10 // via SRB3 within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell10); v_MeasResults := cr_38508_MeasResults(tsc_NR_MeasId1, v_PhysCellIdServing,-,-,p_NRDC_SCG_Cell); //WA#WI=1028438 f_NR_ReceivePeriodicalMeasurementReports(nr_Cell10, 30.0, 10, cr_NR_MeasurementReport(v_MeasResults), false); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); activate(a_NR_AddDefault_IgnorePeriodicalMeasurmentReport_SRB3(nr_Cell10, cr_NR_MeasurementReport(v_MeasResults))); } // End of fl_TC_8_2_3_16_2_TestBody

## Template cr_38508_MeasResults()

| Template name | cr_38508_MeasResults() |
| --- | --- |
| Reason for change | at step 5, when TTCN is expecting the measurement report containing measured RSRP values of NR Cell 10 ( SCG Cell ) , serving cell index has to be filled as 1 in the template cr_38508_MeasResults |
| Summary of change | To fill serving cell index as 1 in the template cr_38508_MeasResults , boolean parameter p_NRDC_SCG_Cell is used |
| TTCN module | NR_Measurement_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (present) MeasResults cr_38508_MeasResults(MeasId p_MeasId := tsc_NR_MeasId1, PhysCellId p_PhysCellId, template MeasResults.measResultNeighCells p_MeasResultNeighCells := omit, template (present) MeasQuantityResults p_MeasQuantityResults := f_NR_Get_MeasQuantityResults() ) := { // @sic R5-186068, R5-194847, R5s190200, R5s190144, R5-194849, R5-201097 sic@ /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ measId := p_MeasId, measResultServingMOList := { { //1 entry servCellId := f_NR_ServCellIndex(), //@sic R5-192274 sic@ measResultServingCell := cr_NR_MeasResultNR_CellResults(p_PhysCellId, p_MeasQuantityResults), measResultBestNeighCell := omit } }, measResultNeighCells := p_MeasResultNeighCells, measResultServFreqListEUTRA_SCG := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultServFreqListNR_SCG := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_EUTRA := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_NR := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultCellListSFTD_NR := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ measResultForRSSI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ locationInfo_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ ul_PDCP_DelayValueResultList_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultsSL_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultCLI_r16 := omit /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ };

After Change:

template (present) MeasResults cr_38508_MeasResults(MeasId p_MeasId := tsc_NR_MeasId1, PhysCellId p_PhysCellId, template MeasResults.measResultNeighCells p_MeasResultNeighCells := omit, template (present) MeasQuantityResults p_MeasQuantityResults := f_NR_Get_MeasQuantityResults(), boolean p_NRDC_SCG_Cell := false //WA#WI=1028438 ) := { // @sic R5-186068, R5-194847, R5s190200, R5s190144, R5-194849, R5-201097 sic@ /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ measId := p_MeasId, measResultServingMOList := { { //1 entry servCellId := f_NR_ServCellIndex(-,p_NRDC_SCG_Cell), //@sic R5-192274 sic@ //WA#WI=1028438 measResultServingCell := cr_NR_MeasResultNR_CellResults(p_PhysCellId, p_MeasQuantityResults), measResultBestNeighCell := omit } }, measResultNeighCells := p_MeasResultNeighCells, measResultServFreqListEUTRA_SCG := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultServFreqListNR_SCG := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_EUTRA := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_NR := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultCellListSFTD_NR := *, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ measResultForRSSI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ locationInfo_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ ul_PDCP_DelayValueResultList_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultsSL_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultCLI_r16 := omit /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ };

## Function f_NR_ServCellIndex()

| Function name | f_NR_ServCellIndex() |
| --- | --- |
| Reason for change | at step 5, when TTCN is expecting the measurement report containing measured RSRP values of NR Cell 10 ( SCG Cell ) , serving cell index has to be filled as 1 in the template cr_38508_MeasResults |
| Summary of change | To fill serving cell index filled as 1 in the template cr_38508_MeasResults , boolean parameter p_NRDC_SCG_Cell is used |
| TTCN module | NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ServCellIndex(boolean p_ENDC_SpecificCell := false) return ServCellIndex { var ServCellIndex v_ServCellIndex := 0; if (f_GetTestcaseAttrib_ENDC(testcasename())or p_ENDC_SpecificCell) { // @sic R5-192274 R5s220981 sic@ v_ServCellIndex := 1; } return v_ServCellIndex; }

After Change:

function f_NR_ServCellIndex(boolean p_ENDC_SpecificCell := false,boolean p_NRDC_SCG_Cell :=false ) return ServCellIndex { var ServCellIndex v_ServCellIndex := 0; if (f_GetTestcaseAttrib_ENDC(testcasename())or p_ENDC_SpecificCell or p_NRDC_SCG_Cell) { // @sic R5-192274 R5s220981 sic@ v_ServCellIndex := 1; } return v_ServCellIndex; }

# Branches executed

## This NR TC was executed in NR-DC FR1+FR2 Band Combination DC_n78A_n257A

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_2_3_16_2_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221332: Supporting information for addition of NR-DC testcase 8.2.3.16.2 in FR1+FR2 |
| --- | --- |