3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221355

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2869 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDCP Test Case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Step 1, 2C, 3, and 7 all state that: “The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell.” Current implementation always transmits PDCP Data PDU on NR Cell 1. However for NR-DC, the PSCell configured for SCG is NR Cell 10. This function needs to be aligned with the prose and handle different PSCells depending on the test case configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use new input parameter p_PSCellId to determine which cell to transmit and receive on. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC, 7.1.3.5.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	8

3	References	10

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath() |
| --- | --- |
| Reason for change | Step 1, 2C, 3, and 7 all state that: “The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell.” Current implementation always transmits PDCP Data PDU on NR Cell 1. For NR-DC, the PSCell configured for SCG is NR Cell 10. This function needs to be aligned with the prose and handle different PSCells depending on the test case configuration. |
| Summary of change | Use new input parameter p_PSCellId to determine which cell to transmit and receive on. Default value of NR Cell 1 is chosen to handle the EN-DC configuration without changes to PDCP_ENDC_NR.ttcn. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(inout NR_PDCP_SS_State_Type p_NR_PDCP_State,// @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, charstring p_TestStep) runs on NR_BASE_PTC {// @sic R5-220641 sic@ var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); //@siclog "Step 1 or 2C" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_SCG_RoutingRlcRouting); v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]); t_WaitForPdu.start; alt{ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) { //@siclog "Step 2 or 2D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)? f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; t_WaitForPdu.stop; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } [] t_WaitForPdu.timeout{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } } //@sic R5s220534 sic@ }

After Change:

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(inout NR_PDCP_SS_State_Type p_NR_PDCP_State,// @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, charstring p_TestStep, NR_CellId_Type p_PSCellId := nr_Cell1) runs on NR_BASE_PTC {// @sic R5-220641 sic@ var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); //@siclog "Step 1 or 2C" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,p_PSCellId,v_SduList[0], p_SCG_RoutingRlcRouting); v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]); t_WaitForPdu.start; alt{ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) { //@siclog "Step 2 or 2D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)? f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; t_WaitForPdu.stop; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } [] t_WaitForPdu.timeout{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } } //@sic R5s220534 sic@ }

## Change 2

| Function name | function f_TC_7_1_3_5_2_Steps3to4b2() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_2_Steps3to4b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, // @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var boolean v_ReceivedPdu := false; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {164}); //@siclog "Step 3" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU=164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath); //Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT,v_SduList[0]); t_WaitForPdu.start; alt{ //@siclog "Step 4a1-4b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4a1"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) //@sic R5s190401 sic@ { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4b2"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){//@sic R5s210065 sic@ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1-4b2"); } } } }

After Change:

function f_TC_7_1_3_5_2_Steps3to4b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, // @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, NR_CellId_Type p_PSCellId := nr_Cell1 ) runs on NR_BASE_PTC { var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var boolean v_ReceivedPdu := false; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {164}); //@siclog "Step 3" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU=164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,p_PSCellId,v_SduList[0], p_PrimaryPath); //Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT,v_SduList[0]); t_WaitForPdu.start; alt{ //@siclog "Step 4a1-4b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4a1"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) //@sic R5s190401 sic@ { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4b2"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){//@sic R5s210065 sic@ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1-4b2"); } } } }

## Change 3

| Function name | function f_TC_7_1_3_5_2_Steps7to8b2() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Same as Change 1 |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_5_2_Steps7to8b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, // @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var boolean v_ReceivedPdu := false; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64}); //@siclog "Step 7" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes @sic R5s210065 sic@ f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath);//Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); t_WaitForPdu.start; alt{ //@siclog "Step 8a1-8b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8a1"); v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity ? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8b2"); v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1-8b2"); } } } }

After Change:

function f_TC_7_1_3_5_2_Steps7to8b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, // @sic R5s220567 sic@ template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, NR_CellId_Type p_PSCellId := nr_Cell1 ) runs on NR_BASE_PTC { var boolean v_ReceivedPdu := false; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64}); //@siclog "Step 7" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes @sic R5s210065 sic@ f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,p_PSCellId,v_SduList[0], p_PrimaryPath);//Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); t_WaitForPdu.start; alt{ //@siclog "Step 8a1-8b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8a1"); v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCellId, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity ? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8b2"); v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1-8b2"); } } } }

## Change 4

| Function name | function fl_TC_7_1_3_5_2_NR_TestBody() |
| --- | --- |
| Reason for change | The PSCell for NR-DC for Step 1, 2C, 3, and 7 is NR Cell 10. |
| Summary of change | Pass the PSCell (NR Cell 10) as input to the corresponding data loopback check functions of each step mentioned. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ // SCG NR Cell10, Primary path NR Cell 1, Secondary path NR Cell 10 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell10), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ //SCG NR Cell10, Primary path NR Cell 10, Secondary path NR Cell 1 //@siclog "Step 2C-2D" siclog@ f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), "Step2D"); f_TC_7_1_3_5_2_Steps3to4b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,b0,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ f_TC_7_1_3_5_2_Steps7to8b2(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1)); }

After Change:

function fl_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on NR_BASE_PTC { var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ // SCG NR Cell10, Primary path NR Cell 1, Secondary path NR Cell 10 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell10), "Step2", nr_Cell10); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,-,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ //SCG NR Cell10, Primary path NR Cell 10, Secondary path NR Cell 1 //@siclog "Step 2C-2D" siclog@ f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), "Step2D", nr_Cell10); f_TC_7_1_3_5_2_Steps3to4b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), nr_Cell10); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_SCG,b0,- ,-, tsc_NR_DRB3))}; //@sic R5s211471 sic@ v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ f_TC_7_1_3_5_2_Steps7to8b2(p_NR_PDCP_State,cs_RlcBearerRouting_NR(nr_Cell10), cs_RlcBearerRouting_NR(nr_Cell1), nr_Cell10); }

# References