3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220230

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2361 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The PDU session ID included in PDU SESSION AUTHENTICATION COMMAND at step 1 may not match the id of the PDU session established at step 0, because there is a hardcoded index of 1 used when retrieving the 5GSM mobility information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace [1] with the index of the last PDU session created. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_1_2_Body() |
| --- | --- |
| Reason for change | The PDU session ID included in PDU SESSION AUTHENTICATION COMMAND at step 1 may not match the id of the PDU session established at step 0, because there is a hardcoded index of 1 used when retrieving the 5GSM mobilitiy information. |
| Summary of change | Replace [1] with the index of the last PDU session created |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment | Rejected: The Prose does not indicate that PDU Session established at Step 0 is the one that should be authenticated. In case this chane is needed, a Prose CR should be raised. |

Before Change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC { ………………. //@siclog "Steps 0A - 0B" siclog@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5-203785, Rs201230, R5s210148 sic@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[1]; //@siclog "Steps 1 - 2" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); ……………… }

After Change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC { ………………… //@siclog "Steps 0A - 0B" siclog@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5-203785, Rs201230, R5s210148 sic@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[f_NR5GC_MobileInfo_GetNoOfSessions()-1]; //@siclog "Steps 1 - 2" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); ………………………… }