3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220448

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2432 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC EPS fallback test cases 11.1.1 and 11.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During these test cases a UE may establish PDU sessions on NR5GC for IMS and internet. These will be mapped to EPS bearer IDs 5 and 7. When this information is transferred to EUTRA PTC, the TTCN will establish DRBs 1 and 3 to map to these. During the execution of the function f_EUTRA38_MultiPDN_ActivateDedicated() for the IMS speech call, the TTCN will allocate DRB ID 4 and EPS bearer ID 8 for the new dedicated EPS bearer. At step 29-33, the test case can then fail because current TTCN uses the hardcoded value tsc_EpsDedicatedBearerId=6 when deactivating the dedicated EPS bearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Recieve the ACTIVATE DEDICVATED EPS BEARER CONTEXT REQUEST message back from the function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(), so that the EPS bearer ID can be passed to f_EUTRA_DeactivateEPS_BearerContext() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE which creates a multiple PDU sessions can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.NR5GC, 11.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_1_1_NR5GC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | During this test case a UE may establish PDU sessions on NR5GC for IMS and internet. These will be mapped to EPS bearer IDs 5 and 7. When this information is transferred to EUTRA PTC, the TTCN will establish DRBs 1 and 3 to map to these. During the execution of the function f_EUTRA38_MultiPDN_ActivateDedicated() for the IMS speech call, the TTCN will allocate DRB ID 4 and EPS bearer ID 8 for the new dedicated EPS bearer. At step 29-33, the test case can then fail because current TTCN uses the hardcoded value tsc_EpsDedicatedBearerId=6 when deactivating the dedicated EPS bearer. |
| Summary of change | Recieve the ACTIVATE DEDICVATED EPS BEARER CONTEXT REQUEST message back from the function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(), so that the EPS bearer ID can be passed to f_EUTRA_DeactivateEPS_BearerContext() |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted in principle, but received only the EPS Bearer Id. Also implemented in 1.1.2, 11.1.3, 11.1.8, 11.1.9 |

Before Change:

function fl_TC_11_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EUTRA_NR_CoOrd_SysInfo_Type v_SysInfoNR; var NasCount_Type v_NasCount; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); …………………….. f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @siclog "Step 29 - 33" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, tsc_EpsDedicatedBearerId);

After Change:

function fl_TC_11_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EUTRA_NR_CoOrd_SysInfo_Type v_SysInfoNR; var NasCount_Type v_NasCount; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReq; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); ………… v_ActDedicatedEPSBearerCxtReq := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @siclog "Step 29 - 33" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, v_ActDedicatedEPSBearerCxtReq.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.epsBearerId); …………….. }

### MCC160 Implementation:

function fl_TC_11_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EUTRA_NR_CoOrd_SysInfo_Type v_SysInfoNR; var NasCount_Type v_NasCount; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var integer v_EPSBearerId; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); …………………….. v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @siclog "Step 29 - 33" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); …………….. }

## Change 2

| Function name | fl_TC_11_1_3_NR5GC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | During this test case a UE may establish PDU sessions on NR5GC for IMS and internet. These will be mapped to EPS bearer IDs 5 and 7. When this information is transferred to EUTRA PTC, the TTCN will establish DRBs 1 and 3 to map to these. During the execution of the function f_EUTRA38_MultiPDN_ActivateDedicated() for the IMS speech call, the TTCN will allocate DRB ID 4 and EPS bearer ID 8 for the new dedicated EPS bearer. At step 29-33, the test case can then fail because current TTCN uses the hardcoded value tsc_EpsDedicatedBearerId=6 when deactivating the dedicated EPS bearer. |
| Summary of change | Recieve the ACTIVATE DEDICVATED EPS BEARER CONTEXT REQUEST message back from the function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(), so that the EPS bearer ID can be passed to f_EUTRA_DeactivateEPS_BearerContext() |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | See comment for change 1 above |

Before Change:

function fl_TC_11_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var SRB_COMMON_IND v_ReceivedMsg; var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template (value) MobileIdentity v_MappedGUTI; ……. // @siclog "Step 24 - 26" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // wait for Trigger for Step 27 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 27" siclog@ f_Delay(1.0); // @siclog "Step 28a" siclog@ //Void @sic R5-211407 sic@ //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @sic R5s201237 sic@ f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, tsc_EpsDedicatedBearerId); …………. }

After Change:

function fl_TC_11_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var SRB_COMMON_IND v_ReceivedMsg; var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template (value) MobileIdentity v_MappedGUTI; var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReq; /**** Skiping code ***********/ // @siclog "Step 24 - 26" siclog@ v_ActDedicatedEPSBearerCxtReq := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // wait for Trigger for Step 27 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 27" siclog@ f_Delay(1.0); // @siclog "Step 28a" siclog@ //Void @sic R5-211407 sic@ //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @sic R5s201237 sic@ f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, v_ActDedicatedEPSBearerCxtReq.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.epsBearerId); …………….. }

## Change 3

| Function name | f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech() |
| --- | --- |
| Reason for change | During this test case a UE may establish PDU sessions on NR5GC for IMS and internet. These will be mapped to EPS bearer IDs 5 and 7. When this information is transferred to EUTRA PTC, the TTCN will establish DRBs 1 and 3 to map to these. During the execution of the function f_EUTRA38_MultiPDN_ActivateDedicated() for the IMS speech call, the TTCN will allocate DRB ID 4 and EPS bearer ID 8 for the new dedicated EPS bearer. At step 29-33, the test case can then fail because current TTCN uses the hardcoded value tsc_EpsDedicatedBearerId=6 when deactivating the dedicated EPS bearer. |
| Summary of change | Return ACTIVATE DEDICATED EPS BEARER REQUEST message so that the EPS bearer ID can be used for deactivation |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.TTCN |
| MCC160 Comment | Accepted in principle, but simply returned the v_DefaultBearerId |

Before Change:

function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_DNNType := IMS_DNN, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC { var integer v_DefaultBearerId := valueof(f_Get_EPSBearerIdForAPNType(p_DNNType)); var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (p_DNNType); f_EUTRA38_MultiPDN_ActivateDedicated(p_CellId, tsc_RRC_TI_Def, int2hex(v_DefaultBearerId, 1), 0, 1, IPCAN_SpeechCall, v_PdnIndex, p_IsSRVCC); }

After Change:

function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_DNNType := IMS_DNN, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC return template (value) NAS_DL_Message_Type { var integer v_DefaultBearerId := valueof(f_Get_EPSBearerIdForAPNType(p_DNNType)); var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (p_DNNType); var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReq; v_ActDedicatedEPSBearerCxtReq := f_EUTRA38_MultiPDN_ActivateDedicated(p_CellId, tsc_RRC_TI_Def, int2hex(v_DefaultBearerId, 1), 0, 1, IPCAN_SpeechCall, v_PdnIndex, p_IsSRVCC); return v_ActDedicatedEPSBearerCxtReq; }

### MCC160 Implementation:

function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId,

PDU_PDN_DNN_Type p_DNNType := IMS_DNN,

boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC return integer

{

var integer v_DefaultBearerId := valueof(f_Get_EPSBearerIdForAPNType(p_DNNType));

var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (p_DNNType);

f_EUTRA38_MultiPDN_ActivateDedicated(p_CellId, tsc_RRC_TI_Def, int2hex(v_DefaultBearerId, 1), 0, 1, IPCAN_SpeechCall, v_PdnIndex, p_IsSRVCC);

return v_DefaultBearerId;

}

## Change 4

| Function name | f_EUTRA38_MultiPDN_ActivateDedicated() |
| --- | --- |
| Reason for change | During this test case a UE may establish PDU sessions on NR5GC for IMS and internet. These will be mapped to EPS bearer IDs 5 and 7. When this information is transferred to EUTRA PTC, the TTCN will establish DRBs 1 and 3 to map to these. During the execution of the function f_EUTRA38_MultiPDN_ActivateDedicated() for the IMS speech call, the TTCN will allocate DRB ID 4 and EPS bearer ID 8 for the new dedicated EPS bearer. At step 29-33, the test case can then fail because current TTCN uses the hardcoded value tsc_EpsDedicatedBearerId=6 when deactivating the dedicated EPS bearer. |
| Summary of change | Return ACTIVATE DEDICATED EPS BEARER REQUEST message so that the EPS bearer ID can be used for deactivation |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.TTCN |
| MCC160 Comment | Not needed |

Before Change:

function f_EUTRA38_MultiPDN_ActivateDedicated(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, EPS_BearerIdentity p_EpsDefaultBearerId, UInt_Type p_NoOfAddDRBs_AM, UInt_Type p_NoOfAddDRBs_UM, IMS_TestConfiguration_Type p_CallType, // @sic R5-216150 sic@ PDN_Index_Type p_PdnIndex := PDN_1, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC { var template (value) RadioBearerList_Type v_DrbConfigList; var template (value) EUTRA38_DRBInfoList_Type v_NewDRBs := {}; var EUTRA38_DRBInfoList_Type v_NewDRBValues := {}; var template (value) EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); /******** Skipped code***************/ f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId, p_RRC_TI, p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM); }

After Change:

function f_EUTRA38_MultiPDN_ActivateDedicated(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, EPS_BearerIdentity p_EpsDefaultBearerId, UInt_Type p_NoOfAddDRBs_AM, UInt_Type p_NoOfAddDRBs_UM, IMS_TestConfiguration_Type p_CallType, // @sic R5-216150 sic@ PDN_Index_Type p_PdnIndex := PDN_1, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC return template (value) NAS_DL_Message_Type { var template (value) RadioBearerList_Type v_DrbConfigList; var template (value) EUTRA38_DRBInfoList_Type v_NewDRBs := {}; var EUTRA38_DRBInfoList_Type v_NewDRBValues := {}; var template (value) EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); /******** Skipped code***************/ f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId, p_RRC_TI, p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM); return v_ActDedicatedEPSBearerCxtReq; }