3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230209

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2884 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.4.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of wk49, NR Cell4 Target cell configurations for handover in between the Rach indication reception of NR Cell2 prior to T304 timer expiry is not appropriate as it leads to test case failure due to conflicts with CNF receptions of other ASPs call in function. This change was already accepted in R5s221145 CR but just missing in 22WK49 delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted f_NR5GC_CHO_RRCReconfiguration_SSConfig() call as it needs to be called from T304 timeout body only and which is already there. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function fl_TC_8_1_4_4_3_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_TC_8_1_4_4_3_TestBody

| Function name | fl_TC_8_1_4_4_3_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of wk49, NR Cell4 Target cell configurations for handover in between the Rach indication reception of NR Cell2 prior to T304 timer expiry is not appropriate as it leads to test case failure due to conflicts with CNF receptions of other ASPs call in function. This change was already accepted in R5s221145 CR but just missing in 22WK49 delivery. |
| Summary of change | Deleted f_NR5GC_CHO_RRCReconfiguration_SSConfig() call as it needs to be called from T304 timeout body only and which is already there. |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT4; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT5; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_NonSuitable_FR1; var RSRP_Range v_Rsrp; var integer v_DRBId; var template (omit) MeasConfig v_MeasConfig; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_11; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList; var boolean v_Rcvd_RachPreambleInd := false; // @sic R5-225171 sic@ var boolean v_AlrySetPowerLevel := false; // @sic R5-225171 sic@ var boolean v_T304_Timeout := false; // @sic R5-225171 sic@ timer t_T304Min; // @sic R5-225171 sic@ timer t_Watchdog := 10.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ---------------------------------------------- * T0 ¦ -85 ¦ -113 ¦ -113 * ---------------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ---------------------------------------------- * T2 ¦ -113 ¦ -113 ¦ -79 * ---------------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 * ---------------------------------------------- * T4 ¦ -113 ¦ -113 ¦ -113 * ---------------------------------------------- * T5 ¦ -113 ¦ -79 ¦ -113 */ //v_CellPower_Serving_FR1 := -85; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPower_NonSuitable_FR1 := -113; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; } else{ v_Rsrp := 2; //FFS dummy value FR2 } //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells. v_MeasConfig := f_NR_GenerateMeasurementConfigNR( {cs_MeasObjectId1(nr_Cell1)}, {cs_NR_ReportConfigCondTrigger(tsc_NR_IdReportConfigId1, v_Rsrp)}, {cs_NR_MeasId_Config_id1_obj1_conf1}, -,-,-,-); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_1))); //@siclog "Step 2" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 1. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable));// @sic R5s221145 sic@ f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ //@siclog "Step 3" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 4-5" siclog@ //- EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. //The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. //The SS does not respond. //t_T304Min := f_NR_SetTimerToleranceMin(nr_Cell2, rrcTimer, 1.0); t_T304Min.start(f_NR_SetTimerToleranceMin(nr_Cell2, rrcTimer, 1.0)); // @sic R5-225171 sic@ while (not v_T304_Timeout) { // @sic R5-225171 sic@ alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)) { v_Rcvd_RachPreambleInd := true; if (not v_AlrySetPowerLevel){ //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_AlrySetPowerLevel := true; //Target cell configuration for handover // @sic R5s220375 sic@ f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList,-,-,-,-,-,-,-); } } [] t_T304Min.timeout { v_T304_Timeout := true; f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s221145 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s221145 sic@ //Target cell configuration for handover // @sic R5s221145 sic@ f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList,-,-,-,-,-,-,-); // @sic R5s221145 sic@ } } } if (not v_Rcvd_RachPreambleInd){ // @sic R5-225171 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE did not send PRACH preamble"); } //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". //f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220375, R5s220537 sic@ //@siclog "Steps 8-9" siclog@ //The SS transmits Randzom Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell4, -); //@siclog "Step 10" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); //@siclog "Step 11" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells. v_ConditionalReconfiguration_r16_Step_11 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell4, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_11))); //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ f_NR_SS_RachProcedureConfig(nr_Cell4, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220694 sic@ //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220537 sic@ if (not fl_NR_SysIND_Rach(nr_Cell1)){ // @sic R5s220537 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE did not send PRACH preamble"); } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220537 sic@ //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); f_NR_SS_RachProcedureConfig_Def(nr_Cell2); // @sic R5s220537 sic@ //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell2, // @sic R5s220537 sic@ -,-,-,-,-); // @sic R5s220375 sic@ }// End of fl_TC_8_1_4_4_3_TestBody

After Change:

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT4; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT5; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_NonSuitable_FR1; var RSRP_Range v_Rsrp; var integer v_DRBId; var template (omit) MeasConfig v_MeasConfig; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_11; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList; var boolean v_Rcvd_RachPreambleInd := false; // @sic R5-225171 sic@ var boolean v_AlrySetPowerLevel := false; // @sic R5-225171 sic@ var boolean v_T304_Timeout := false; // @sic R5-225171 sic@ timer t_T304Min; // @sic R5-225171 sic@ timer t_Watchdog := 10.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ---------------------------------------------- * T0 ¦ -85 ¦ -113 ¦ -113 * ---------------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ---------------------------------------------- * T2 ¦ -113 ¦ -113 ¦ -79 * ---------------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 * ---------------------------------------------- * T4 ¦ -113 ¦ -113 ¦ -113 * ---------------------------------------------- * T5 ¦ -113 ¦ -79 ¦ -113 */ //v_CellPower_Serving_FR1 := -85; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPower_NonSuitable_FR1 := -113; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; } else{ v_Rsrp := 2; //FFS dummy value FR2 } //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells. v_MeasConfig := f_NR_GenerateMeasurementConfigNR( {cs_MeasObjectId1(nr_Cell1)}, {cs_NR_ReportConfigCondTrigger(tsc_NR_IdReportConfigId1, v_Rsrp)}, {cs_NR_MeasId_Config_id1_obj1_conf1}, -,-,-,-); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_1))); //@siclog "Step 2" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 1. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable));// @sic R5s221145 sic@ f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ //@siclog "Step 3" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 4-5" siclog@ //- EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. //The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. //The SS does not respond. //t_T304Min := f_NR_SetTimerToleranceMin(nr_Cell2, rrcTimer, 1.0); t_T304Min.start(f_NR_SetTimerToleranceMin(nr_Cell2, rrcTimer, 1.0)); // @sic R5-225171 sic@ while (not v_T304_Timeout) { // @sic R5-225171 sic@ alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)) { v_Rcvd_RachPreambleInd := true; if (not v_AlrySetPowerLevel){ //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_AlrySetPowerLevel := true; <<Deleted below Call>> //Target cell configuration for handover // @sic R5s220375 sic@ // f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList,-,-,-,-,-,-,-); } } [] t_T304Min.timeout { v_T304_Timeout := true; f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s221145 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s221145 sic@ //Target cell configuration for handover // @sic R5s221145 sic@ f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList,-,-,-,-,-,-,-); // @sic R5s221145 sic@ } } } if (not v_Rcvd_RachPreambleInd){ // @sic R5-225171 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE did not send PRACH preamble"); } //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". //f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220375, R5s220537 sic@ //@siclog "Steps 8-9" siclog@ //The SS transmits Randzom Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell4, -); //@siclog "Step 10" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); //@siclog "Step 11" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells. v_ConditionalReconfiguration_r16_Step_11 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell4, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_11))); //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@ f_NR_SS_RachProcedureConfig(nr_Cell4, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220694 sic@ //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220537 sic@ if (not fl_NR_SysIND_Rach(nr_Cell1)){ // @sic R5s220537 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE did not send PRACH preamble"); } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220537 sic@ //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); f_NR_SS_RachProcedureConfig_Def(nr_Cell2); // @sic R5s220537 sic@ //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell2, // @sic R5s220537 sic@ -,-,-,-,-); // @sic R5s220375 sic@ }// End of fl_TC_8_1_4_4_3_TestBody