3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230292

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3028 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.1.3.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per Table 4.6.3-185: SSB-MTC from 38508-1 , SSB-MTC -> sf20 is set as 10 for INTER-FREQ_ODD cell. In the test case 8.1.1.3.7a, NR Cell 3 is iinter-frequencycell for NR Cell 1 . Hence the function f_NR_GetSSB_MTC_InterFreqOdd sets v_SSB_MTC.periodicityAndOffset.sf20 := 10 in SIB4 of NR cell 1 corresponding to Nr cell 3 frequency. But SIB24 information of EUTRA Cell 1 is having periodicityAndOffset_->sf20_r15 = (0) corresponding to NR Cell 3 Frequenc . As per the test , at step 4 , UEis camped on EUTRA cell 1 and expected to Camp on NR Cell 3 at Step 6 after T325 expiry. In order for UE to detect NR cell 3, SIB24 information of EUTRA cell 1 has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.7a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_8_1_1_3_7a_NR5GC_EUTRA	3

2.2	New Template cs_MTC_SSB_NR_FR1_8_1_1_3_7a	5

2.3	New Function f_EUTRA_CellInfo_SetSIB24_SetmeasTimingConfig_r15_periodicityAndOffset_r15	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_8_1_1_3_7a_NR5GC_EUTRA

| Function name | f_TC_8_1_1_3_7a_NR5GC_EUTRA |
| --- | --- |
| Reason for change | As per Table 4.6.3-185: SSB-MTC from 38508-1 , SSB-MTC -> sf20 is set as 10 for INTER-FREQ_ODD cell. In the test case 8.1.1.3.7a , NR Cell 3 is inter frequency cell for NR Cell 1 . hence the function f_NR_GetSSB_MTC_InterFreqOdd sets v_SSB_MTC.periodicityAndOffset.sf20 := 10 in SIB4 of NR cell 1 corresponding to Nr cell 3 frequency. But SIB24 information of EUTRA cell 1 is having periodicityAndOffset_->sf20_r15 = (0) corresponding to NR Cell 3 Frequency . As per the test case , at step 4 , UE is camped on EUTRA cell 1 and expected to Camp on NR Cell 3 at Step 6 after T325 expiry. In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |
| Summary of change | In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) . |
| TTCN module | ttcn/develop/NR5GC_IRAT/8_1_1/RRC_ConnRel_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_1_3_7a_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //RRC connection release / Success / Deprioritisation / NR / T325 expiry f_EUTRA38_IRAT_Init(c31); f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell1, -75); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); //Take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

After Change:

function f_TC_8_1_1_3_7a_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //RRC connection release / Success / Deprioritisation / NR / T325 expiry f_EUTRA38_IRAT_Init(c31); //sib4->smtc.periodicityAndOffset.sf20 = (10) is set in NR cell 1 for NR cell 3 frequency , same has to be replicated on EUTRA side for NR Cell 3 Frequency f_EUTRA_CellInfo_SetSIB24_SetmeasTimingConfig_r15_periodicityAndOffset_r15(eutra_Cell1,cs_MTC_SSB_NR_FR1_8_1_1_3_7a,1); //WA#WI=1078616 f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell1, -75); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_8_1_1_3_7a_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); //Take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

## New Template cs_MTC_SSB_NR_FR1_8_1_1_3_7a

| Template name | cs_MTC_SSB_NR_FR1_8_1_1_3_7a |
| --- | --- |
| Reason for change | As per Table 4.6.3-185: SSB-MTC from 38508-1 , SSB-MTC -> sf20 is set as 10 for INTER-FREQ_ODD cell. In the test case 8.1.1.3.7a , NR Cell 3 is inter frequency cell for NR Cell 1 . hence the function f_NR_GetSSB_MTC_InterFreqOdd sets v_SSB_MTC.periodicityAndOffset.sf20 := 10 in SIB4 of NR cell 1 corresponding to Nr cell 3 frequency. But SIB24 information of EUTRA cell 1 is having periodicityAndOffset_->sf20_r15 = (0) corresponding to NR Cell 3 Frequency . As per the test case , at step 4 , UE is camped on EUTRA cell 1 and expected to Camp on NR Cell 3 at Step 6 after T325 expiry. In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |
| Summary of change | In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) . |
| TTCN module | ttcn/develop/NR5GC_IRAT/Common/EUTRA38_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Template definition:

template(value) MTC_SSB_NR_r15 cs_MTC_SSB_NR_FR1_8_1_1_3_7a := //WA#WI=1078616 { /* @status APPROVED (NR5GC_IRAT) */ periodicityAndOffset_r15 := { sf20_r15 := 10 }, ssb_Duration_r15 := sf2 };

## New Function f_EUTRA_CellInfo_SetSIB24_SetmeasTimingConfig_r15_periodicityAndOffset_r15

| Function name | f_EUTRA_CellInfo_SetSIB24_SetmeasTimingConfig_r15_periodicityAndOffset_r15 |
| --- | --- |
| Reason for change | As per Table 4.6.3-185: SSB-MTC from 38508-1 , SSB-MTC -> sf20 is set as 10 for INTER-FREQ_ODD cell. In the test case 8.1.1.3.7a , NR Cell 3 is inter frequency cell for NR Cell 1 . hence the function f_NR_GetSSB_MTC_InterFreqOdd sets v_SSB_MTC.periodicityAndOffset.sf20 := 10 in SIB4 of NR cell 1 corresponding to Nr cell 3 frequency. But SIB24 information of EUTRA cell 1 is having periodicityAndOffset_->sf20_r15 = (0) corresponding to NR Cell 3 Frequency . As per the test case , at step 4 , UE is camped on EUTRA cell 1 and expected to Camp on NR Cell 3 at Step 6 after T325 expiry. In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |
| Summary of change | In order for UE to detect NR cell 3 , SIB24 information of EUTRA cell 1 is has to be set accordingly periodicityAndOffset_->sf20_r15 = (10) |
| TTCN module | ttcn/develop/NR5GC_IRAT/Common/EUTRA38_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Function definition:

function f_EUTRA_CellInfo_SetSIB24_SetmeasTimingConfig_r15_periodicityAndOffset_r15(EUTRA_CellId_Type p_CellId, template(value) MTC_SSB_NR_r15 p_MTC_SSB_NR_r15, integer i := 0) runs on EUTRA_PTC { var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); // SIB24 is only present in combination c31 select (valueof(v_CellInfo.Sysinfo.Combination)) { case (c31) { v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530.carrierFreqListNR_r15[i].measTimingConfig_r15 := p_MTC_SSB_NR_r15; f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); f_EUTRA_SetSIs_ValueTag(p_CellId); } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } }