3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230313

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3044 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of ENDC MAC BWP test case 7.1.1.8.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU EMEA LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.8.1 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to ENDC test case 7.1.1.8.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	6

4	Execution Log Files	6

4.1	Qualcomm Snapdragon X70 UE	6

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.8.1 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1.1.8.1

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_7_1_1_8_1_DedicatedBwpListUL |
| --- | --- |
| Reason for change | The fl_TC_7_1_1_8_1_DedicatedBwpListUL is common for both SA and ENDC variants of the test, however the function f_NR_GetSIB1PUSCH_TimeDomainResourceAllocationList cannot be used to retrieve the TDRA from SIB1 in case of ENDC variant of the test, since the Sis are transmitted only in LTE and not in NR. |
| Summary of change | v_PUSCH_TimeDomainResourceAllocationList is populated from a template and not retrieved from the NR SIB1 |
| TTCN module | MAC_BWP_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_1_8_1_DedicatedBwpListUL(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template (value) UplinkConfig.uplinkBWP_ToAddModList { //@sic R5-227417 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP_Default := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, 0); //@sic R5s230220 change 3 sic@ var SSB_Index v_SSB_Index := f_NR_CellInfo_GetSSB_Index(p_NR_CellId); var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId); var template (value) BWP_UplinkCommon v_BWP_UplinkCommon; var template (value) PUCCH_Config v_PUCCH_Config; var template (value) PUSCH_Config v_PUSCH_Config := v_NR_UplinkBWP_Default.Dedicated.R15.pusch_Config.setup; var template (value) BWP_UplinkDedicated.srs_Config v_SRS_Config; var template (value) BWP_UplinkDedicated v_BWP_UplinkDedicated; var template (value) UplinkConfig.uplinkBWP_ToAddModList v_DedicatedBwpListUL; var BWP_Id v_BwpId; var PRB_Id v_StartingPRB; var integer i; var integer v_MaxBWP := 3; //@sic R5-213459 sic@ var integer v_C_SRS := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var PUSCH_TimeDomainResourceAllocationList v_PUSCH_TimeDomainResourceAllocationList := f_NR_GetSIB1PUSCH_TimeDomainResourceAllocationList(p_NR_CellId); //@sic R5s230220 change 3 sic@ if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } //@sic R5s230220 change 3 sic@ if (f_NR_CellInfo_GetIsFR1(p_NR_CellId)) { v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR1(v_C_SRS, v_SSB_Index)); } else { v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR2(v_C_SRS, v_SSB_Index)); } v_PUSCH_Config.pusch_TimeDomainAllocationList.setup := v_PUSCH_TimeDomainResourceAllocationList; //@sic R5s230220 change 3 sic@ for (i := 0; i < v_MaxBWP; i := i + 1) { // Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N v_BwpId := i + 1; v_StartingPRB := 0; // @sic R5-205578 sic@ //@sic R5-213459 sic@ // Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N v_BWP_UplinkCommon := cs_BWP_UplinkCommon_Def(fl_TC_7_1_1_8_1_BWP_Generic(p_NR_CellId, v_BwpId), omit, cs_Pusch_ConfigCommon_Setup(cs_38508_PUSCH_ConfigCommon(omit)), cs_Pucch_ConfigCommon_Setup(cs_TC_7_1_1_8_1_PUCCH_ConfigCommon)); v_PUCCH_Config := cs_38508_PUCCH_Config(v_SubcarrierSpacing, -, -, v_SSB_Index, cs_38508_DL_DataToUL_ACK); // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_PUCCH_Config.resourceSetToAddModList := { cs_TC_7_1_1_8_1_PUCCH_ResourceSet }; // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_PUCCH_Config.resourceToAddModList := { cs_38508_PUCCH_ResourceDef(0, v_StartingPRB, omit, cs_38508_ResourceFormat0(0, 2, 10), omit) }; // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_BWP_UplinkDedicated := cs_38508_BWP_UplinkDedicated(v_PUCCH_Config, v_PUSCH_Config, v_SRS_Config); v_DedicatedBwpListUL[i] := cs_38508_BWP_Uplink(v_BwpId, v_BWP_UplinkCommon, v_BWP_UplinkDedicated); } return v_DedicatedBwpListUL; }

After Change

function fl_TC_7_1_1_8_1_DedicatedBwpListUL(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template (value) UplinkConfig.uplinkBWP_ToAddModList { //@sic R5-227417 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP_Default := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, 0); //@sic R5s230220 change 3 sic@ var SSB_Index v_SSB_Index := f_NR_CellInfo_GetSSB_Index(p_NR_CellId); var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId); var PUSCH_TimeDomainResourceAllocation.k2 v_K2forMsg3; var template (value) BWP_UplinkCommon v_BWP_UplinkCommon; var template (value) PUCCH_Config v_PUCCH_Config; var template (value) PUSCH_Config v_PUSCH_Config := v_NR_UplinkBWP_Default.Dedicated.R15.pusch_Config.setup; var template (value) BWP_UplinkDedicated.srs_Config v_SRS_Config; var template (value) BWP_UplinkDedicated v_BWP_UplinkDedicated; var template (value) UplinkConfig.uplinkBWP_ToAddModList v_DedicatedBwpListUL; var BWP_Id v_BwpId; var PRB_Id v_StartingPRB; var integer i; var integer v_MaxBWP := 3; //@sic R5-213459 sic@ var integer v_C_SRS := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var PUSCH_TimeDomainResourceAllocationList v_PUSCH_TimeDomainResourceAllocationList; if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } //@sic R5s230220 change 3 sic@ if (f_NR_CellInfo_GetIsFR1(p_NR_CellId)) { select (v_SubcarrierSpacing) { case (kHz15) {v_K2forMsg3 := 2;} case (kHz30) {v_K2forMsg3 := 6;} } v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR1(v_C_SRS, v_SSB_Index)); } else { v_SRS_Config := cs_NR_Srs_Config_Setup(cs_38508_SRS_Config_FR2(v_C_SRS, v_SSB_Index)); } v_PUSCH_TimeDomainResourceAllocationList := cs_38508_PUSCH_TimeDomainResourceAllocationList(v_K2forMsg3); v_PUSCH_Config.pusch_TimeDomainAllocationList.setup := v_PUSCH_TimeDomainResourceAllocationList; //@sic R5s230220 change 3 sic@ for (i := 0; i < v_MaxBWP; i := i + 1) { // Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N v_BwpId := i + 1; v_StartingPRB := 0; // @sic R5-205578 sic@ //@sic R5-213459 sic@ // Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N v_BWP_UplinkCommon := cs_BWP_UplinkCommon_Def(fl_TC_7_1_1_8_1_BWP_Generic(p_NR_CellId, v_BwpId), omit, cs_Pusch_ConfigCommon_Setup(cs_38508_PUSCH_ConfigCommon(omit)), cs_Pucch_ConfigCommon_Setup(cs_TC_7_1_1_8_1_PUCCH_ConfigCommon)); v_PUCCH_Config := cs_38508_PUCCH_Config(v_SubcarrierSpacing, -, -, v_SSB_Index, cs_38508_DL_DataToUL_ACK); // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_PUCCH_Config.resourceSetToAddModList := { cs_TC_7_1_1_8_1_PUCCH_ResourceSet }; // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_PUCCH_Config.resourceToAddModList := { cs_38508_PUCCH_ResourceDef(0, v_StartingPRB, omit, cs_38508_ResourceFormat0(0, 2, 10), omit) }; // Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N v_BWP_UplinkDedicated := cs_38508_BWP_UplinkDedicated(v_PUCCH_Config, v_PUSCH_Config, v_SRS_Config); v_DedicatedBwpListUL[i] := cs_38508_BWP_Uplink(v_BwpId, v_BWP_UplinkCommon, v_BWP_UplinkDedicated); } return v_DedicatedBwpListUL; }

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Qualcomm Snapdragon X70 UE

The Qualcomm Snapdragon X70 5G Modem-RF System UEpassed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_8_1_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_8_1_PIXIT.xml

# References

R5s230314: Supporting information for re-verification of ENDC MAC BWP test case 7.1.1.8.1 in FR1