3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230398

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4718 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_1_6_4_EUTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TDD Mode, at Step 9, the return value of the function f_DRX_NormalSF would be negative due to parameter p_CountY being set to 0, while it is expected to be an integer (0…infinity). This will cause runtime error. In TDD Mode, at Step 9, parameter p_CountY should be set to 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the second input parameter from 0 1 in function f_DRX_NormalSF at step 9 (for TDD mode) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	function f_TC_7_1_6_4_EUTRA()	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## function f_TC_7_1_6_4_EUTRA()

| Function name | f_TC_7_1_6_4_EUTRA() |
| --- | --- |
| Reason for change | In TDD Mode, at Step 9, the return value of the function f_DRX_NormalSF would be negative due to parameter p_CountY being set to 0, while it is expected to be an integer (0…infinity). This will cause runtime error. In TDD Mode, at Step 9, parameter p_CountY should be set to 1. |
| Summary of change | Changed the second input parameter from 0 1 in function f_DRX_NormalSF at step 9 (for TDD mode) |
| TTCN module | LTE\7_1\MAC_716.ttcn |

Before Change:

function f_TC_7_1_6_4_EUTRA() runs on EUTRA_PTC { /* L2/MAC * 7.1.6.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception */ const integer tsc_LongDRXCycle := 640; const integer tsc_ShortDRXCycle := 64; const integer tsc_DrxStartOffset := 4; const integer tsc_OnDurationTimer := 20; const integer tsc_ShortDRXCycleTimer := 10; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; const integer tsc_SubFrameY := 2; const integer tsc_SubFrameZ := 6; <CODE SKIPPED> //@siclog "Step 9" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); if (v_EUTRA_FDD_TDD_Mode == FDD) { v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode); } v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing4), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), cs_HarqProcessAssignment_Specific(4))); <CODE SKIPPED>

After Change:

function f_TC_7_1_6_4_EUTRA() runs on EUTRA_PTC { /* L2/MAC * 7.1.6.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception */ const integer tsc_LongDRXCycle := 640; const integer tsc_ShortDRXCycle := 64; const integer tsc_DrxStartOffset := 4; const integer tsc_OnDurationTimer := 20; const integer tsc_ShortDRXCycleTimer := 10; const integer tsc_DRXRetranTimer := 6; const integer tsc_SubFrameX := 6; const integer tsc_SubFrameY := 2; const integer tsc_SubFrameZ := 6; <CODE SKIPPED> //@siclog "Step 9" siclog@ v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer); if (v_EUTRA_FDD_TDD_Mode == FDD) { v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode); } else if (v_EUTRA_FDD_TDD_Mode == TDD) { v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode); } v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF); DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing4), cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), cs_HarqProcessAssignment_Specific(4))); <CODE SKIPPED>