3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230419

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3005 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR Idle mode testcase 6.4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - AT step 4, T0 power event will take place on which EUTRA cell should be turned off so EUTRA cell will not impact cell from step 6 to 7I which UE should be exchanging messages on NR. - After Release from NR cell at step 7I T2 power event should take place. - At step 8, T2 should take place after release from NR cell, So Need to inform EUTRA component to update cell power level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added function f_EUTRA38_SetCellPower to change power level of EUTRA cells. - Added co-ordination message for wait from NR. - Added co-ordination message for EUTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2………………………………………………………………………………………………………..4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | - AT step 4, T0 power event will take place on which EUTRA cell should be turned off so EUTRA cell will not impact cell from step 6 to 7I which UE should be exchanging messages on NR. - After Release from NR cell at step 7I T2 power event should take place. |
| Summary of change | - Added function f_EUTRA38_SetCellPower to change power level of EUTRA cells. - Added co-ordination message for wait from NR. |
| TTCN module | Inactive_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-200366 R5s200817 R5-203745 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := -78; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NonSuitableOffCellRS_EPRE; var integer v_T2_Cell3_RS_EPRE_FR1 := -76; //@siclog "Step 1" siclog@ // The SS changes E-UTRA Cell 1 SSS level according to the row "T1" in table 6.4.3.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1); //@siclog "Step 2 - Void" siclog@ //@siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 indicate that the UE is camped on E-UTRA Cell 1? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // let NR change power levels on Step4 //Change of power level of EUTRA cell for T0 here //@siclog "Step 6" siclog@ // Steps of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1. f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); // @sic R5-204417 sic@ //Added co-ordination msg here //@siclog "Step 8" siclog@ // The SS changes NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 SSS level according to the row "T2" in table 6.4.3.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1); f_EUTRA38_SetCellPower(eutra_Cell3, v_T2_Cell3_RS_EPRE_FR1, v_T2_Cell3_RS_EPRE_FR1); //@siclog "Step 9" - Void" siclog@ //@siclog "Step 10" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 indicate that the UE is camped on E-UTRA Cell 3 with condition 'connected without release'? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell3, tsc_RRC_TI_Def, eutra_Cell1, -, noRrcConnectionRelease, -, -, -, false); // @sic R5s201004 sic@ } // fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody

After Change:

function fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @sic R5-200366 R5s200817 R5-203745 sic@ var integer v_T1_Cell1_RS_EPRE_FR1 := -78; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NonSuitableOffCellRS_EPRE; var integer v_T2_Cell3_RS_EPRE_FR1 := -76; var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NonSuitableOffCellRS_EPRE; var integer v_T0_Cell3_RS_EPRE_FR1 := tsc_NonSuitableOffCellRS_EPRE; //@siclog "Step 1" siclog@ // The SS changes E-UTRA Cell 1 SSS level according to the row "T1" in table 6.4.3.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1); //@siclog "Step 2 - Void" siclog@ //@siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 indicate that the UE is camped on E-UTRA Cell 1? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // let NR change power levels on Step4 // The SS changes NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 SSS level according to the row "T0" in table 6.4.3.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR1); f_EUTRA38_SetCellPower(eutra_Cell3, v_T0_Cell3_RS_EPRE_FR1, v_T0_Cell3_RS_EPRE_FR1); //@siclog "Step 6" siclog@ // Steps of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1. f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); // @sic R5-204417 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Step 8"); //@siclog "Step 8" siclog@ // The SS changes NR Cell 1, E-UTRA Cell 1 and E-UTRA Cell 3 SSS level according to the row "T2" in table 6.4.3.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1); f_EUTRA38_SetCellPower(eutra_Cell3, v_T2_Cell3_RS_EPRE_FR1, v_T2_Cell3_RS_EPRE_FR1); //@siclog "Step 9" - Void" siclog@ //@siclog "Step 10" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 indicate that the UE is camped on E-UTRA Cell 3 with condition 'connected without release'? f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell3, tsc_RRC_TI_Def, eutra_Cell1, -, noRrcConnectionRelease, -, -, -, false); // @sic R5s201004 sic@ } // fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody

## Change 2

| Function name | fl_TC_6_4_3_1_TestBody () |
| --- | --- |
| Reason for change | At step 8 T2 should take place after release from NR cell, So Need to inform EUTRA component to update cell power level. |
| Summary of change | Added co-ordination message for EUTRA. |
| TTCN module | Inactive_CellReSelection_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_4_3_1_TestBody() runs on NR5GC_PTC { // @sic R5s200566 R5s200817 R5s200944 R5-203745 R5-210610 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // Rx context for Step4 from EUTRA v_NR_RachProcedure := f_NR_InitialiseRachProcedure(nr_Cell1); // @sic R5s191148 Ch. 3, R5s201380 sic@ f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.3.1.3.2-1 f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 5 - Void" siclog@ //@siclog "Step 6" siclog@ // Steps of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1,noRrcConnectionRelease,false); //@siclog "Step 7" siclog@ // The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 7A-7H" siclog@ // Steps 1-8 as defined in TS 38.508-1 Table 4.5.4.2-3 f_NR5GC_RRCIdleToConnected(nr_Cell1); //@siclog "Step 7I" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell1); //Added co-ordination msg here. //@siclog "Step 8" siclog@ // The SS changes NR Cell 1, E-UTRA Cell 12 and E-UTRA Cell 13 SSS level according to the row "T2" in table 6.4.3.1.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -98, -98); // Send both authentication and NR sysinfo parameters to EUTRA for f_EUTRA38_TrackingAreaUpdateFromN1 f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); } // fl_TC_6_4_3_1_TestBody

After Change:

function fl_TC_6_4_3_1_TestBody() runs on NR5GC_PTC { // @sic R5s200566 R5s200817 R5s200944 R5-203745 R5-210610 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // Rx context for Step4 from EUTRA v_NR_RachProcedure := f_NR_InitialiseRachProcedure(nr_Cell1); // @sic R5s191148 Ch. 3, R5s201380 sic@ f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.3.1.3.2-1 f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 5 - Void" siclog@ //@siclog "Step 6" siclog@ // Steps of the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 are performed on NR Cell 1. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1,noRrcConnectionRelease,false); //@siclog "Step 7" siclog@ // The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 7A-7H" siclog@ // Steps 1-8 as defined in TS 38.508-1 Table 4.5.4.2-3 f_NR5GC_RRCIdleToConnected(nr_Cell1); //@siclog "Step 7I" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell1); f_EUTRA_NR_SendCoOrd(EUTRA,cms_EUTRA_NR_Trigger ("Step 8")); //@siclog "Step 8" siclog@ // The SS changes NR Cell 1, E-UTRA Cell 12 and E-UTRA Cell 13 SSS level according to the row "T2" in table 6.4.3.1.3.2-1/2. f_NR_SetCellPower (nr_Cell1, -98, -98); // Send both authentication and NR sysinfo parameters to EUTRA for f_EUTRA38_TrackingAreaUpdateFromN1 f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); } // fl_TC_6_4_3_1_TestBody