3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230423

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3097 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.2.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_TC_8_2_2_6_1_ENDC_NR : The preamble is done with MCG_Only , so no bearers are established at NR side. Further in the function , fl_TC_8_2_2_6_1_ENDC_NR_TestBody , f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA) is called which in turn calls f_NR_RRC_CipherActTime_GetCurrent --> f_NR_SS_PdcpCount_Get --> and it results in empty v_SecurityActTimeList due to No NR bearers established; this is resulting in run time issue. so it is proposed to remove the function call f_ENDC_GetKeysActivateSecurity from NR test body and the counterpart f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys from EUTRA side . for MCG_Only Preamble cases, this function call is not required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the function calls f_ENDC_GetKeysActivateSecurity from NR test body and the counterpart f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys from EUTRA side . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Calling f_ENDC_GetKeysActivateSecurity from the NR test body results in runtime issue as there are no NR beares established at this time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function fl_TC_8_2_2_6_1_ENDC_NR_TestBody	2

2.2	Correction to the function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function fl_TC_8_2_2_6_1_ENDC_NR_TestBody

| Template name | fl_TC_8_2_2_6_1_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_8_2_2_6_1_ENDC_NR : The preamble is done with MCG_Only , so no bearers are established at NR side. Further in the function , fl_TC_8_2_2_6_1_ENDC_NR_TestBody , f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA) is called which inturn calls f_NR_RRC_CipherActTime_GetCurrent --> f_NR_SS_PdcpCount_Get --> and it results in empty v_SecurityActTimeList due to No NR bearers established; this is result in run time issue. so it is proposed to remove the function call f_ENDC_GetKeysActivateSecurity from NR test body. for MCG_Only Preamble cases, this function call is not required |
| Summary of change | Removed the function call f_ENDC_GetKeysActivateSecurity from NR test body |
| TTCN module | ttcn/develop/ENDC/8_2_2/RRCReconfig_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_2_6_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var DRB_Identity v_DrbId := tsc_NR_DRB8; // EUTRA38_DRB var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) SRB_ToAddModList v_SRB_ToAddModList; // @siclog "Step 1" siclog@ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg")); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg")); v_SRB_ToAddModList := { cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1), // @sic R5-202574 sic@ cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2) }; f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); // @sic R5s200650 sic@ << SKIPPED CODE >> }

After Change:

function fl_TC_8_2_2_6_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var DRB_Identity v_DrbId := tsc_NR_DRB8; // EUTRA38_DRB var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) SRB_ToAddModList v_SRB_ToAddModList; // @siclog "Step 1" siclog@ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg")); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg")); v_SRB_ToAddModList := { cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1), // @sic R5-202574 sic@ cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2) }; //f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); // @sic R5s200650 sic@ //WA#WI=1095629 << SKIPPED CODE >> }

## Correction to the function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody

| Template name | fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Due to change in section 1.1, f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys from EUTRA side has to be removed |
| Summary of change | Removed the function call f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys from EUTRA side. |
| TTCN module | ttcn/develop/ENDC/8_2_2/RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { << SKIPPED CODE >> v_SrbList := { cds_SRB1_Config_NRPDCP, cds_SRB2_Config_NRPDCP }; f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); // @sic R5s200650, R5s201070 sic@ v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); << SKIPPED CODE >> }

After Change:

function fl_TC_8_2_2_6_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { << SKIPPED CODE >> v_SrbList := { cds_SRB1_Config_NRPDCP, cds_SRB2_Config_NRPDCP }; //f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); // @sic R5s200650, R5s201070 sic@ //WA#WI=1095629 v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); << SKIPPED CODE >> }