3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230434

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3021 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC TC 9.1.1.3 and 9.1.1.6 against SNPN-only UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Derivation of keys CK' and IK' for EAP-AKA', and and KAUSF and (X)RES* for 5G AKA requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialize and pass the NID for NGC Cell A (all cells are using the same PLMN) to the functions building the Authentication Request in Step 6, 8, 11, 16, and 53. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_1_3_6_Body() |
| --- | --- |
| Reason for change | Derivation of keys CK' and IK' for EAP-AKA', and and KAUSF and (X)RES* for 5G AKA requires the Serving Network Name which needs to be updated to SNPN identity in case of SNPN. From TS 33.501 I.3.2: SN Id = PLMN ID:NID |
| Summary of change | Initialize and pass the NID for NGC Cell A (all cells are using the same PLMN) to the functions building the Authentication Request in Step 6, 8, 11, 16, and 53. |
| TTCN module | Authentication_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_1_3_6_Body(boolean p_UseEAP ) runs on NR5GC_PTC { /* test body of TC_9_1_1_3 and 9_1_1_6 The only difference is if including an EAP message or not */ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var SecurityHeaderType v_SecurityHeader := tsc_SHT_IntegrityProtected_Ciphered; var integer i; var NR_CellId_Type v_CellId := v_NGC_NASCellA; // @sic R5-199098 sic@ var default v_MyDefaultVar := null; timer t_T3511 := f_NR_SetTimerToleranceMin(v_NGC_NASCellD, nasTimer, 10.0); timer t_T3520 := f_NR_SetTimerToleranceMin(v_NGC_NASCellB, nasTimer, 15.0); // @sic R5s200373 sic@ // Get normal message if (p_UseEAP) { v_MsgToSend := f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN); //9_1_1_3 } else { v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN); } // now update with "old" KSI value v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; *** SKIPPED CODE *** if (p_UseEAP) { //@siclog "Step 53" siclog@ //This is for Testcase 9_1_1_3 v_MsgToSend := f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN); //9_1_1_3 @sic R5s201357 sic@ } else { v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN); //9_1_1_6 @sic R5-199098, R5s201357 sic@ //@siclog "Step 48" siclog@ // This is for Testcase 9_1_1_6 } SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellD, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); *** SKIPPED CODE *** }

After Change:

import from NR_CellInfoInit all; import from NR_Parameters all; function fl_TC_9_1_1_3_6_Body(boolean p_UseEAP ) runs on NR5GC_PTC { /* test body of TC_9_1_1_3 and 9_1_1_6 The only difference is if including an EAP message or not */ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var SecurityHeaderType v_SecurityHeader := tsc_SHT_IntegrityProtected_Ciphered; var integer i; var NR_CellId_Type v_CellId := v_NGC_NASCellA; // @sic R5-199098 sic@ var default v_MyDefaultVar := null; timer t_T3511 := f_NR_SetTimerToleranceMin(v_NGC_NASCellD, nasTimer, 10.0); timer t_T3520 := f_NR_SetTimerToleranceMin(v_NGC_NASCellB, nasTimer, 15.0); // @sic R5s200373 sic@ var NID_r16 v_NID_ASN; var template (omit) hexstring v_NID := omit; if(pc_SNPN_only){ v_NID_ASN := f_NR_InitNDI(v_NGC_NASCellA); // returns a bitstring v_NID := bit2hex(v_NID_ASN); } // Get normal message if (p_UseEAP) { v_MsgToSend := f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, v_NID); //9_1_1_3 } else { v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, -, v_NID); } // now update with "old" KSI value v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; *** SKIPPED CODE *** if (p_UseEAP) { //@siclog "Step 53" siclog@ //This is for Testcase 9_1_1_3 v_MsgToSend := f_Get_EAP_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, v_NID); //9_1_1_3 @sic R5s201357 sic@ } else { v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, v_NID); //9_1_1_6 @sic R5-199098, R5s201357 sic@ //@siclog "Step 48" siclog@ // This is for Testcase 9_1_1_6 } SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellD, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); *** SKIPPED CODE *** }