3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230465

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4722 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 8.2.1.5 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If UECapabilityInformaiton Message is more than 1000bits, message will be segmented into two RLC Segments in case of CAT-M1. Hence, two grants are required to transmit two RLC segments which require additional 10ms to receive the second RLC segment. associated draftProseCR shall be raised in RAN5#99 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Value of variable v_DelayGrantToData is changed from 5 to 15 in order to take care of the segmented data due to higher message size. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant CATM1 UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_RRC_Procedure_Latency |
| --- | --- |
| Reason for change | If UECapabilityInformaiton Message is more than 1000bits, message will be segmented into two RLC Segments in case of CAT-M1. Hence, two grants are required to transmit two RLC segments which require additional 10ms to receive the second RLC segment. #associated draftProseCR shall be raised in RAN5#99 meeting |
| Summary of change | Value of variable v_DelayGrantToData is changed from 5 to 15 in order to take care of the segmented data due to higher message size. |
| TTCN module | EUTRA_CommonProcedures.ttcn |
| MCC160 Comment |  |

Before change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId,

template (value)   SRB_COMMON_REQ p_SRB_COMMON_REQ_1,

template (omit)    SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit,

template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1,

template           SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit,

boolean p_IsFDD,

integer p_N,

charstring p_TestcaseStep,

charstring p_UE_Cat,

//@sic R5-153973 to  be set to true for UE capability sic@

boolean p_BigULGrant := false) runs on EUTRA_PTC return SRB_COMMON_IND

{ //@sic R5-152105 sic@ @sic R5s150734 R5-153973 R5-167569: p_UE_Cat->charstring sic@

var integer v_DL_RetransmissionCnt := 0;

var SYSTEM_IND v_SYSTEM_IND;

var SRB_COMMON_IND v_SRB_COMMON_IND;

var boolean v_NasFlag := ispresent(p_SRB_COMMON_REQ_1.Signalling.Nas);// @sic R5s200666 sic@

var boolean v_T1Flag := false;

var boolean v_T3Flag := false;

var boolean v_T4Flag := (not isvalue(p_SRB_COMMON_REQ_2));     // we only check for a second UL PDU when there is a second DL PDU

var integer v_NoOfGrants := 5;

var integer v_GrantCycle;

var integer v_DelayGrantToData;

var SubFrameTiming_Type v_T0; // Sending of DL RRC PDU(s)

var SubFrameTiming_Type v_T1; // HARQ ACK for DL RRC PDU(s)

var SubFrameTiming_Type v_T2; // First UL grant

var SubFrameTiming_Type v_T3; // First UL RRC PDU received

var SubFrameTiming_Type v_T4; // Second UL RRC PDU received

var SubFrameTiming_Type v_T5; // Last UL grant

var SubFrameTiming_Type v_Tnow;

var TimingInfo_Type v_TimingInfo;

var integer v_UlGrantOffset; //@sic R5-144678 R5s141213 sic@

var integer v_I_MCS:=0;      // Modulation and Coding Scheme Index @sic R5s150734 sic@

var integer v_N_PRB:=1;      // PRBS @sic R5s150734 sic@

var boolean v_RLCstatusFlag := false; // Anritsu TTCN WA for 8.2.1.5 LTE PCT-19703 to ignore UL_HARQ_IND ack for RLC status PDU (for RRC_RES)

timer t_Timer;

if (p_IsFDD) {

v_GrantCycle := 8;          // HARQ RTT = 8

if (pc_FDD_TypeB_HalfDuplex) //@sic R5-182302 R5s180242 sic@

{

v_GrantCycle := 10;

}

v_DelayGrantToData := 5;    // UL data is always sent 4 + 1 TTIs after the corresponding grant @sic R5-144678 sic@

v_UlGrantOffset  := 1;       //@sic R5-144678  1 TTI added due toR5-144678 sic@

} else {

v_GrantCycle := 10;         // HARQ RTT = 10 for SN = 6

v_DelayGrantToData := 8;    // UL data is sent 4 + 3 + 1 TTIs after the corresponding grant @sic R5-144678 sic@

v_UlGrantOffset  := 2;      /* @sic R5-144678 R5s141213 1 TTI added due toR5-144678, but that results in sub frame =5 not suitable for grant in TDD config 1, hence 1 more sub frame added to result in SN 6  sic@ */

}

After change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId,

template (value)   SRB_COMMON_REQ p_SRB_COMMON_REQ_1,

template (omit)    SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit,

template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1,

template           SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit,

boolean p_IsFDD,

integer p_N,

charstring p_TestcaseStep,

charstring p_UE_Cat,

//@sic R5-153973 to  be set to true for UE capability sic@

boolean p_BigULGrant := false) runs on EUTRA_PTC return SRB_COMMON_IND

{ //@sic R5-152105 sic@ @sic R5s150734 R5-153973 R5-167569: p_UE_Cat->charstring sic@

var integer v_DL_RetransmissionCnt := 0;

var SYSTEM_IND v_SYSTEM_IND;

var SRB_COMMON_IND v_SRB_COMMON_IND;

var boolean v_NasFlag := ispresent(p_SRB_COMMON_REQ_1.Signalling.Nas);// @sic R5s200666 sic@

var boolean v_T1Flag := false;

var boolean v_T3Flag := false;

var boolean v_T4Flag := (not isvalue(p_SRB_COMMON_REQ_2));     // we only check for a second UL PDU when there is a second DL PDU

var integer v_NoOfGrants := 5;

var integer v_GrantCycle;

var integer v_DelayGrantToData;

var SubFrameTiming_Type v_T0; // Sending of DL RRC PDU(s)

var SubFrameTiming_Type v_T1; // HARQ ACK for DL RRC PDU(s)

var SubFrameTiming_Type v_T2; // First UL grant

var SubFrameTiming_Type v_T3; // First UL RRC PDU received

var SubFrameTiming_Type v_T4; // Second UL RRC PDU received

var SubFrameTiming_Type v_T5; // Last UL grant

var SubFrameTiming_Type v_Tnow;

var TimingInfo_Type v_TimingInfo;

var integer v_UlGrantOffset; //@sic R5-144678 R5s141213 sic@

var integer v_I_MCS:=0;      // Modulation and Coding Scheme Index @sic R5s150734 sic@

var integer v_N_PRB:=1;      // PRBS @sic R5s150734 sic@

var boolean v_RLCstatusFlag := false; // Anritsu TTCN WA for 8.2.1.5 LTE PCT-19703 to ignore UL_HARQ_IND ack for RLC status PDU (for RRC_RES)

timer t_Timer;

var charstring v_Testcase := testcasename(); // Anritsu TTCN CR for PCT-25109

if (p_IsFDD) {

v_GrantCycle := 8;          // HARQ RTT = 8

if (pc_FDD_TypeB_HalfDuplex) //@sic R5-182302 R5s180242 sic@

{

v_GrantCycle := 10;

}

v_DelayGrantToData := 5;    // UL data is always sent 4 + 1 TTIs after the corresponding grant @sic R5-144678 sic@

if (v_Testcase == "TC_8_2_1_5" and pc_FDD_TypeB_HalfDuplex) // Anritsu TTCN CR CATM1 TC 8.2.1.5

{

v_DelayGrantToData := 15;

}

v_UlGrantOffset  := 1;       //@sic R5-144678  1 TTI added due toR5-144678 sic@

} else {

v_GrantCycle := 10;         // HARQ RTT = 10 for SN = 6

v_DelayGrantToData := 8;    // UL data is sent 4 + 3 + 1 TTIs after the corresponding grant @sic R5-144678 sic@

v_UlGrantOffset  := 2;      /* @sic R5-144678 R5s141213 1 TTI added due toR5-144678, but that results in sub frame =5 not suitable for grant in TDD config 1, hence 1 more sub frame added to result in SN 6  sic@ */

}