3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230470

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4740 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the frequency settings of several NB-IoT bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NB-IoT frequencies for bands 87, 88, 103, 255 and 256 are not implemented correctly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected affected parameters according to the values defined in 36.508 clause 8.1.3.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SS will not be configured with the same frequency as the UE in the affected bands |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NB-IoT Test Suite |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | function f_NBIOT_InitFrequencyLowMidHigh () |
| --- | --- |
| Reason for change | The NB-IoT frequencies for bands 87, 88, 103, 255 and 256 are not implemented correctly |
| Summary of change | Corrected affected parameters according to the values defined in 36.508 clause 8.1.3.1.1 |
| TTCN module | NBIOT_CellInfoFrequency.ttcn |

Before Change:

function f_NBIOT_InitFrequencyLowMidHigh(integer p_Band, NB_OperationMode_Type p_NB_OperationMode) return NBIOT_FrequencyLowMidHigh_Type { var template (value) NBIOT_FrequencyLowMidHigh_Type v_FreqLMH; select (p_Band){ … SKIPPED CODE… case (87) { // Band 87 @sic R5-205078 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.87-1: NB-IoT standalone Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134183, v0, 70547, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134207, v0, 70571, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134231, v0, 70595, v_0dot5); } case (inband_samePCI, inband_differentPCI) { //Table 8.1.3.1.1.87-2: NB-IoT in-band Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134216, v0, 70580, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134216, v0, 70580, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134216, v0, 70580, v1); } case (guardband) { //Table 8.1.3.1.1.87-3: NB-IoT guard-band Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134183, v0, 70547, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134183, v0, 70547, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134231, v0, 70595, v1); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (88) { // Band 88 @sic R5-205078 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.88-1: NB-IoT standalone Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134233, v0, 70597, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134257, v0, 70621, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134281, v0, 70645, v_0dot5); } case (inband_samePCI, inband_differentPCI) { //Table 8.1.3.1.1.88-2: NB-IoT in-band Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134266, v0, 70630, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134266, v0, 70630, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134266, v0, 70630, v1); } case (guardband) { //Table 8.1.3.1.1.88-3: NB-IoT guard-band Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134233, v0, 70597, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134233, v0, 70597, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134281, v0, 70645, v1); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (103) { // Band 103 @sic R5-225776 sic@ select (p_NB_OperationMode){ case (standalone) { if (px_NS_Value == 4) { //Table 8.1.3.1.1.103-1a: NB-IoT standalone Test frequencies for operating band 103 with NS_04 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134284, v0, 70648, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134287, v0, 70651, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134290, v0, 70654, v_0dot5); } else { //Table 8.1.3.1.1.103-1: NB-IoT standalone Test frequencies for operating band 103 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (134283, v0, 70647, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (134287, v0, 70651, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (134291, v0, 70655, v_0dot5); } } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (255) { // Band 255 @sic R5-232011 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.255-1: NB-IoT standalone Test frequencies for operating band 255 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (261505, v0, 228737, v0); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (261674, v0, 228906, v0); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (261843, v0, 229075, v0); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (256) { // Band 256 @sic R5-232011 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.256-1: NB-IoT standalone Test frequencies for operating band 256 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (261845, v0, 229077, v0); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (261994, v0, 229226, v0); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (262143, v0, 229375, v0); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case else { FatalError (__FILE__, __LINE__, "invalid Band"); } } return valueof(v_FreqLMH); }

After Change:

function f_NBIOT_InitFrequencyLowMidHigh(integer p_Band, NB_OperationMode_Type p_NB_OperationMode) return NBIOT_FrequencyLowMidHigh_Type { var template (value) NBIOT_FrequencyLowMidHigh_Type v_FreqLMH; select (p_Band){ … SKIPPED CODE… case (87) { // Band 87 @sic R5-205078 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.87-1: NB-IoT standalone Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70547, v0, 134183, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70571, v0, 134207, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70595, v0, 134231, v_0dot5); } case (inband_samePCI, inband_differentPCI) { //Table 8.1.3.1.1.87-2: NB-IoT in-band Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70580, v0, 134216, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70580, v0, 134216, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70580, v0, 134216, v1); } case (guardband) { //Table 8.1.3.1.1.87-3: NB-IoT guard-band Test frequencies for operating band 87 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70547, v0, 134183, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70547, v0, 134183, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70595, v0, 134231, v1); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (88) { // Band 88 @sic R5-205078 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.88-1: NB-IoT standalone Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70597, v0, 134233, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70621, v0, 134257, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70645, v0, 134281, v_0dot5); } case (inband_samePCI, inband_differentPCI) { //Table 8.1.3.1.1.88-2: NB-IoT in-band Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70630, v0, 134266, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70630, v0, 134266, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70630, v0, 134266, v1); } case (guardband) { //Table 8.1.3.1.1.88-3: NB-IoT guard-band Test frequencies for operating band 88 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70597, v0, 134233, v1); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70597, v0, 134233, v1); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70645, v0, 134281, v1); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (103) { // Band 103 @sic R5-225776 sic@ select (p_NB_OperationMode){ case (standalone) { if (px_NS_Value == 4) { //Table 8.1.3.1.1.103-1a: NB-IoT standalone Test frequencies for operating band 103 with NS_04 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70648, v0, 134284, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70651, v0, 134287, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70654, v0, 134290, v_0dot5); } else { //Table 8.1.3.1.1.103-1: NB-IoT standalone Test frequencies for operating band 103 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (70647, v0, 134283, v_0dot5); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (70651, v0, 134287, v_0dot5); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (70655, v0, 134291, v_0dot5); } } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (255) { // Band 255 @sic R5-232011 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.255-1: NB-IoT standalone Test frequencies for operating band 255 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (228737, v0, 261505, v0); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (228906, v0, 261674, v0); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (229075, v0, 261843, v0); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case (256) { // Band 256 @sic R5-232011 sic@ select (p_NB_OperationMode){ case (standalone) { //Table 8.1.3.1.1.256-1: NB-IoT standalone Test frequencies for operating band 256 v_FreqLMH.FrequencyLow := cs_NB_CarrierFreq (229077, v0, 261845, v0); v_FreqLMH.FrequencyMid := cs_NB_CarrierFreq (229226, v0, 261994, v0); v_FreqLMH.FrequencyHigh := cs_NB_CarrierFreq (229375, v0, 262143, v0); } case else{ FatalError (__FILE__, __LINE__, "invalid NB operation mode"); } } } case else { FatalError (__FILE__, __LINE__, "invalid Band"); } } return valueof(v_FreqLMH); }