3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230494

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3115 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.5.9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-06-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation, after step 6 ,Service Accept message is sent before the reception of the SecurityModeComplete, and after the transmission of DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message. However , it will make more sense to wait for the SecurityModeComplete, and then transmit the Service Accept. It is proposed that the Service Accept message be sent after transmission of SecurityModeComplete, and after transmission of DLDedicatedMessageSegment containing the segments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of sending Service Accept message ahead of receiving SecurityModeComplete, it is proposed that Service Accept is sent from TTCN after reception of SecurityModeComplete and after transmitting all the DLDedicatedMessageSegment containing the segments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service accept will be transmitted in the middle of two segments, and also before reception of SecurityModeComplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_8_1_5_9_2_NR5GC_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_8_1_5_9_2_NR5GC_TestBody

| Template name | f_TC_8_1_5_9_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN Implementation, after step 6 ,Service Accept message is sent before the reception of the SecurityModeComplete, and after the transmission of DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message. However , it will make more sense to wait for the SecurityModeComplete, and then transmit the Service Accept. It is proposed that the Service Accept message be sent after transmission of SecurityModeComplete, and after transmission of DLDedicatedMessageSegment containing the segments. |
| Summary of change | Instead of sending Service Accept message ahead of receiving SecurityModeComplete, it is proposed that Service Accept is sent from TTCN after reception of SecurityModeComplete and after transmitting all the DLDedicatedMessageSegment containing the segments. |
| TTCN module | ttcn/develop/NR5GC/8_1_5/RRC_Others_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_9_2_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // Send first segment and NAS message f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); //@sic R5s221253 sic@ SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE // @siclog "Step 8" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, lastSegment); // @siclog "Step 9" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_9_2_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> // Send first segment and NAS message f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 0, notLastSegment); //@sic R5s221253 sic@ //SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); //WA#WI=1101230 // @siclog "Step 7" siclog@ // Check: Does the UE transmit a SecurityModeComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def))); // receive SECURITY MODE COMPLETE // @siclog "Step 8" siclog@ // The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCReconfigurationEncoded, 1, lastSegment); SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); //WA#WI=1101230 // @siclog "Step 9" siclog@ // Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); << SKIPPED CODE >> }