3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230503

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4743 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose, at Step 21c, the UE is triggered to initiate MO Exception Data. The expectation is that the UE shall not trigger any ESM Data Transport for this, as it has already reached the limit for triggering ESM Data Transport (Step 21D). At Step 22, the additional APN Rate control timer expires, it is checked that the UE is now able to trigger MO Exception Data again (Step 22A) when the next 1 minute of additional APN Rate Control timer starts (Step 23). At this point, a device that has still buffered the MO Exception data trigger (This is UE specific implementation) at Step 21C, might act on it and transmit an ESM Data Transport as well. Thus, after Step 23, there is a chance that some UEs may transmit ESM Data Transport twice: Once for Step 21C and once for Step 22A. Currently the TTCN stops the additional APN rate control timer upon reception of ESM Data Transport at Step 24, and the “second” ESM Data Transport message received causes testcase failure. This needs to be addressed. Note : A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a new Step 24A, where the additional APN rate control timer is timed out. Basically reworked the implementation of Steps 23 – 26. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Function fl_TC_22_5_21_Body	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function fl_TC_22_5_21_Body

| Template name | fl_TC_22_5_21_Body |
| --- | --- |
| Reason for change | In the current Prose, at Step 21c, the UE is triggered to initiate MO Exception Data. The expectation is that the UE shall not trigger any ESM Data Transport for this, as it has already reached the limit for triggering ESM Data Transport (Step 21D). At Step 22, the additional APN Rate control timer expires, it is checked that the UE is now able to trigger MO Exception Data again (Step 22A) when the next 1 minute of additional APN Rate Control timer starts (Step 23). At this point, a device that has still buffered the MO Exception data trigger (This is UE specific implementation) at Step 21C, might act on it and transmit an ESM Data Transport as well. Thus, after Step 23, there is a chance that some UEs may transmit ESM Data Transport twice: Once for Step 21C and once for Step 22A. Currently the TTCN stops the additional APN rate control timer upon reception of ESM Data Transport at Step 24, and the “second” ESM Data Transport message received causes testcase failure. This needs to be addressed. |
| Summary of change | Introduced a new Step 24A, where the additional APN rate control timer is timed out. Basically reworked the implementation of Steps 23 – 26. Please see below. |
| TTCN module | ENBIOT_NAS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 22A" siclog@ f_UT_TriggerMO_ExceptionData(UT); // @sic R5-192266 sic@ //@siclog "Step 23" siclog@ t_Additional_APN_RateControl.start; alt { //@siclog "Step 24" siclog@ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, ?))))-> value v_ReceivedAsp { t_Additional_APN_RateControl.stop; f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 24"); //@siclog "Step 24" siclog@ // @sic R5-184494 sic@ f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1); } [] t_Additional_APN_RateControl.timeout { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 23"); } } //End of Alt } // End of fl_TC_22_5_21_Body

After Change:

<<SKIPPED CODE>> //@siclog "Step 22A" siclog@ f_UT_TriggerMO_ExceptionData(UT); // @sic R5-192266 sic@ //@siclog "Step 23" siclog@ t_Additional_APN_RateControl.start; /*WA#22_5_21 COMMENTED OUT alt { //@siclog "Step 24" siclog@ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, ?))))-> value v_ReceivedAsp { t_Additional_APN_RateControl.stop; f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 24"); //@siclog "Step 24" siclog@ // @sic R5-184494 sic@ f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1); } [] t_Additional_APN_RateControl.timeout { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 23"); } } //End of Alt */ //WA#22_5_21 v_Count := 0; alt { //@siclog "Step 24" siclog@ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, ?))))-> value v_ReceivedAsp { v_Count := v_Count+1; repeat; } //@siclog "Step 24A" siclog@ [] t_Additional_APN_RateControl.timeout { if((v_Count < 1) or (v_Count > 2) ) { //This ensures that ESM Data Transport is received minimum once and maximum twice. f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 23"); }else { f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 24"); } } } //End of Alt //@siclog "Step 26" siclog@ // @sic R5-184494 sic@ f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1); //WA#22_5_21 } // End of fl_TC_22_5_21_Body