3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230517

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3131 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR ENDC MAC testcase 7.1.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Call to function f_NR_CellInfo_SetUplinkBWP after the assignment of the variable v_NR_UplinkBWP_List[0].Common := omit will cause RUNTIME error in function f_TC_7_1_1_6_2_NR_TestBody2 where variable v_NR_ResourceAllocation_CRNTI is being initialised by calling f_NR_ResourceAllocation_UplinkGrant which tries to access the Common IE causing the error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the call to function f_NR_CellInfo_SetUplinkBWP . Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_2_ENDC() |
| --- | --- |
| Reason for change | Call to function f_NR_CellInfo_SetUplinkBWP after the assignment of the variable v_NR_UplinkBWP_List[0].Common := omit will cause RUNTIME error in function f_TC_7_1_1_6_2_NR_TestBody2 where variable v_NR_ResourceAllocation_CRNTI is being initialised by calling f_NR_ResourceAllocation_UplinkGrant which tries to access the Common IE causing the error. |
| Summary of change | Removed the call to function f_NR_CellInfo_SetUplinkBWP . Please see below. |
| TTCN module | MAC_SPS_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_2_ENDC_NR() runs on NR5GC_PTC { // Correct handling of UL grant / configured grant Type 1 <<SKIPPED CODE>> select (v_SubcarrierSpacing) { // The SPS period selected to be less than SR repetition timeout of 100 ms. case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_ConfiguredGrantConfig := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),v_NR_ResourceAllocation1.Imcs, 9))); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_NR_UplinkBWP_List[0].Common := omit; <<SKIPPED CODE>> //@siclog steps 6-7 logsic@ v_ConfiguredGrantConfig := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),v_NR_ResourceAllocation1.Imcs, 39))); v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig := {setup := v_ConfiguredGrantConfig}; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := {setup := v_ConfiguredGrantConfig}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); // @siclog "Steps 8 - 16" siclog@ v_MyDefaultVar := activate ( a_NR_MAC_IgnorePaddingPDU(nr_Cell1, v_DRBId)); f_TC_7_1_1_6_2_NR_TestBody2(v_DRBId,v_NR_ResourceAllocation1); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_2_ENDC_NR() runs on NR5GC_PTC { // Correct handling of UL grant / configured grant Type 1 <<SKIPPED CODE>> select (v_SubcarrierSpacing) { // The SPS period selected to be less than SR repetition timeout of 100 ms. case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_ConfiguredGrantConfig := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),v_NR_ResourceAllocation1.Imcs, 9))); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_NR_UplinkBWP_List[0].Common := omit; <<SKIPPED CODE>> //@siclog steps 6-7 logsic@ v_ConfiguredGrantConfig := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),v_NR_ResourceAllocation1.Imcs, 39))); v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig := {setup := v_ConfiguredGrantConfig}; // REMOVED f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); WA# v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := {setup := v_ConfiguredGrantConfig}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); // @siclog "Steps 8 - 16" siclog@ v_MyDefaultVar := activate ( a_NR_MAC_IgnorePaddingPDU(nr_Cell1, v_DRBId)); f_TC_7_1_1_6_2_NR_TestBody2(v_DRBId,v_NR_ResourceAllocation1); <<SKIPPED CODE>>