3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230691

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3198 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_RRC_ProcedureDelayResume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN implementation of the function f_NR5GC_RRC_ProcedureDelayResume, v_T1 is always calculated using function f_NR_GetNextSendOccasion_DL. For test cases listed in “Clauses affected” at Steps 6a5 - 6a7, Resume message carrying sCellToAddModList is scheduled with v_TimingInfo precalculated. v_TimingInfo is getting overwritten by the calculated v_T1 inside the function f_NR5GC_RRC_ProcedureDelayResume. v_T1 value should be assigned with v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame if the DL message is not having TimingNow so that dl messge can be scheduled with the precalculated v_TimingInfo. Only in case of TimingInfo is equal to TimingNow for the DL message transmission, v_T1 has to be calculated using the function f_NR_GetNextSendOccasion_DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Recalculated v_T1 value only in case TimingInfo is equal to TimingNow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC and 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_NR5GC_RRC_ProcedureDelayResume	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_NR5GC_RRC_ProcedureDelayResume

| Function name | f_NR5GC_RRC_ProcedureDelayResume |
| --- | --- |
| Reason for change | v_T1 value should be assigned with v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame if the DL message is not having TimingNow so that dl messge can be scheduled with the precalculated v_TimingInfo. Only in case of TimingInfo is equal to TimingNow for the DL message transmission, v_T1 has to be calculated using the function f_NR_GetNextSendOccasion_DL. |
| Summary of change | Recalculated v_T1 value only in case TimingInfo is equal to TimingNow. |
| TTCN module | Common/NR5GC/NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_ProcedureDelayResume(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> timer t_Watchdog; // The SS transmits an RRCReestablishment message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; @@ -2147,9 +2149,12 @@ module NR5GC_RRCSteps { @@ -2147,9 +2149,12 @@ module NR5GC_RRCSteps { v_T1.Slot.SlotOffset.Numerology3 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); << SKIPPED CODE >>

After Change:

function f_NR5GC_RRC_ProcedureDelayResume(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> timer t_Watchdog; // The SS transmits an RRCReestablishment message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; @@ -2147,9 +2149,12 @@ module NR5GC_RRCSteps { @@ -2147,9 +2149,12 @@ module NR5GC_RRCSteps { v_T1.Slot.SlotOffset.Numerology3 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else{ v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); << SKIPPED CODE >>