3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230078

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2916 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In TTCN Implementation for test case 7.1.1.3.9, function to configure MAC mode from Test Mode to Normal Mode should be applied in the preamble before any other configuration is applied. However, in the current implementation this is applied after the UL Grant Configuration change. - In TTCN Implementation for test case 7.1.1.3.9, At Step #4, SS Configuration for UL Grant Config is applied with Target Time after applying Configuration to disable SR SYSTEM IND. This might cause delay in applying the UL Grant. It is proposed to apply the Configuration for UL Grant before disabling SR SYSTEM IND Config. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Function _TC_7_1_1_3_9_NR5GC() is modified to correct the order in which MAC test mode is configured back to Normal in the postamble of the test case. - Function f_TC_7_1_1_3_9_NR_TestBody is modified to move the disabling of SR System IND configuration after Applying the UL Grant at step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC, 7.1.1.3.9.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_9_NR5GC () |
| --- | --- |
| Reason for change | In TTCN Implementation for test case 7.1.1.3.9, function to configure MAC mode from Test Mode to Normal Mode should be applied in the preamble before any other configuration is applied. However, in the current implementation this is applied after the UL Grant Configuration change. |
| Summary of change | Function _TC_7_1_1_3_9_NR5GC() is modified to correct the order in which MAC test mode is configured back to Normal in the postamble of the test case. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Rejected. In the test model it is possible to update a grant when MAC is in any mode, there is no ASP sequence dependency. This is done like that in other MAC test cases, e.g. 7.1.1.2.1, 7.1.1.3.1. |

Before Change

function f_TC_7_1_1_3_9_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation ... //@sic R5s220332 sic@ f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_NR5GC_TestBody(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change

function f_TC_7_1_1_3_9_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation ... //@sic R5s220332 sic@ f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_NR5GC_TestBody(v_DRBId); f_NR_TestBody_Set(false); //Postamble //f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // MOVED TO BELOW ==> v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 2

| Function name | f_TC_7_1_1_3_9_NR_TestBody () |
| --- | --- |
| Reason for change | In TTCN Implementation for test case 7.1.1.3.9, At Step #4, SS Configuration for UL Grant Config is applied with Target Time after applying Configuration to disable SR SYSTEM IND. This might cause delay in applying the UL Grant. It is proposed to apply the Configuration for UL Grant before disabling SR SYSTEM IND Config. |
| Summary of change | Function f_TC_7_1_1_3_9_NR_TestBody is modified to move the disabling of SR System IND configuration after Applying the UL Grant at step 4. |
| TTCN module | MAC_TC_Common_NR.ttcn.ttcn |
| MCC160 Comment | Accepted as improvment |

Before Change

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { ... // @siclog "Step 2" siclog@ //The UE transmits a Scheduling Request. SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //Stop SS reporting SR indications //@sic R5s221024 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); // @siclog "Step 3" siclog@ //The SS allocates an UL Grant for HARQ process 1, sufficient for one RLC SDU to be looped back in a slot n, and NDI indicates new transmission and DCI scheduling the PUSCH indicates rvID = 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(1)); // @siclog "Step 4" siclog@ ... };

After Change

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { ... // @siclog "Step 2" siclog@ //The UE transmits a Scheduling Request. SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //Stop SS reporting SR indications //@sic R5s221024 sic@ // f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); // MOVED TO BELOW ==> // @siclog "Step 3" siclog@ //The SS allocates an UL Grant for HARQ process 1, sufficient for one RLC SDU to be looped back in a slot n, and NDI indicates new transmission and DCI scheduling the PUSCH indicates rvID = 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(1)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));//Applied After UL Grant Config // @siclog "Step 4" siclog@ ... };