3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230095

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2931 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GSM test case 10.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current TTCN implementation may fail on UE with IMS as default only, and no internet PDU established in the preamble, ie. the initial test condition in the spec is not fulfilled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TTCN implementation to trigger the Internet PDU if the UE does not request internet PDU by default |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_10_1_2_1_NR5GC |
| --- | --- |
| Reason for change | The test case fails when run on UE with IMS as default, due to no internet PDU established in the preamble, ie. the initial condition in the spec is not fulfilled |
| Summary of change | Changes added to trigger the Internet PDU if the UE does not request internet PDU by default |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TEST_LOOPModeB_ON); // @sic R5-198972 sic@ //Main test body fl_TC_10_1_2_1_Body(); // Finish f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; var template (omit) NR_DRBInfo_Type v_DRBInfo; f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TEST_LOOPModeB_ON); // @sic R5-198972 sic@ // check if internet PDU session established in preamble v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId) if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate – SEE CHANGE 3 if(not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } } //Main test body fl_TC_10_1_2_1_Body(); // Finish f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

## Change 2

| Function name | fl_TC_10_1_2_1_Body |
| --- | --- |
| Reason for change | The TTCN implementation uses the default PDU info, which will fail if UE has IMS as default, rather than internet |
| Summary of change | Updated implementation to get the internet PDU session info. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_10_1_2_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; ......................... v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //@siclog "Steps 1" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, -, -, -, -, -, v_QoS_Rules); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 2" siclog@ ......................... }

After Change

function fl_TC_10_1_2_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; /*Skipping the code */ v_PDUSessionIdToModify := f_GetLastConfiguredInternetPDUSessionID(); v_QoS_Id := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession()); //@siclog "Steps 1" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, -, -, -, -, -, v_QoS_Rules); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 2" siclog@ ......................... }

## Change 3

| Function name | f_NR5GC_Activate_TestMode |
| --- | --- |
| Reason for change | Since the SRB1 is hardcoded and only used in preamble in current TTCN where it is always called right after RRC SMC but in this case we now calling it after RRC reconfiguration, SRB2 shall be use |
| Summary of change | Code change done to use the SRB2. |
| TTCN module | NR5GC_LoopBack.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_NR5GC_Activate_TestMode(NR_CellId_Type p_NR_CellId, UE_TestLoopMode_Type p_UE_TestLoopMode) runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NR_ACTIVATE_TEST_MODE(p_UE_TestLoopMode)))); SRB.receive (car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NR_ACTIVATE_TEST_MODE_COMPLETE))); f_NR5GC_SetLoopbackIsActivated(true); }

After Change

function f_NR5GC_Activate_TestMode(NR_CellId_Type p_NR_CellId, UE_TestLoopMode_Type p_UE_TestLoopMode, NR_RadioBearerId_Type p_SRB:= tsc_NR_RbId_SRB1) runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := p_SRB; SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NR_ACTIVATE_TEST_MODE(p_UE_TestLoopMode)))); SRB.receive (car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NR_ACTIVATE_TEST_MODE_COMPLETE))); f_NR5GC_SetLoopbackIsActivated(true); }