3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230101

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0178 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR POS testcase 9.3.4.4.21s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add POS test case 9.3.4.4.21s to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.3.4.4.21s required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.4.21s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Verification Test Summary	2

2	Changes required to verify test case 9.3.4.4.21s	2

2.1 Correction to f_TC_9_3_4_4_DlTdoa_NR5GC_TestBody	2

2.2 Correction to LPP_Templates	3

2.3 Correction to f_POS_NR_SimulateDlPrsPOSifSupported	5

2.4 Correction to f_POS_NR_CellConfig_DlPrs	6

2.5 Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP	7

3	Branches executed	8

4	Execution Log Files	8

4.1	Mediatek MT6879 UE	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC POS test case 9.3.4.4.21s which is part of the POS test suite in iWD_TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	9.3.4.4.21s.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Changes required to verify test case 9.3.4.4.21s

## 2.1 Correction to f_TC_9_3_4_4_DlTdoa_NR5GC_TestBody

| Function name | f_TC_9_3_4_4_DlAoD_NR5GC_TestBody |
| --- | --- |
| Reason for change | The template cr_LppProvideLocationInformation_ DlTdoa used to be compared o the LPP Provide Location Information message from the UE leads to wrong “LPP Message do not match to template” fail verdicts, even if UE reports measurements. The template cr_LppProvideLocationInformation_ DlTdoa requires the cr_CommonIEsProvideLocationInformation to be present, which is not the case for certain UEs. This lead into the wrong fail verdicts described above. |
| Summary of change | Change the way the LPP Provide Locatrion information message is matched to a template, in a similar way as done for TC 9.3.4.4.21s. |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. See below. |

Before

<<SKIPPED CODE>>

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, cr_LppProvideLocationInformation_DlTdoa (tsc_POS_TransactionNumber));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 9.3.4.4 Step 5");

//@siclog "Step 5a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 5

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

v_Octetstring := f_POS_NR_ReceiveEncodedLppMessage();

v_LppMessage := f_POS_NR_LppMessage_DecodeAndMatch(v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)));

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.nr_DL_TDOA_ProvideLocationInformation_r16)) {

if (match (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.nr_DL_TDOA_ProvideLocationInformation_r16, cr_NR_DL_TDOA_ProvideLocationInformation)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 9.3.4.4 Step 5");

}

else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Wrong DL-TDOA Location Information");

}

}

else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "DL-TDOA Location Information not present");

}    //@siclog "Step 5a" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 5

f_POS_NR_SendLppAck(nr_Cell1, tsc_POS_RoutingId, v_LppMessage);<<SKIPPED CODE>>

MCC160 proposed implementation

template (present) LPP_Message cr_LppProvideLocationInformation_DlTdoa(TransactionNumber p_TransactionNumber) :=

cr_LppProvideLocationInformation_Common(p_TransactionNumber, cr_ProvideLocationInformation_r9_IEs(cr_CommonIEsProvideLocationInformation(omit, omit, omit, omit) ifpresent, omit, omit, omit, omit, omit, omit, omit, ?));

## 2.2 Correction to LPP_Templates

| Function name | LPP _Templates |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-CombSizeN-AndReOffset-r16 is 0 for all the TRPs. Apart from that, the value of dl-PRS-NumSymbols-r16 is set to n2. Since the RE offset is same for all TRPs (is overlapped) and the symbol length is set only to 2, a conformant UE may not decode PRS sequences from different Physical Cell IDs. Therefore, a different dl-PRS-CombSizeN-AndReOffset-r16 value is needed per TRP and the value of dl-PRS-NumSymbols-r16 needs to be increased to allow proper decoding of the PRS signals. |
| Summary of change | Modify templates cs_NR_DL_PRS_Info_Def, cs_NR_DL_PRS_ResourceSet and cs_NR_DL_PRS_Resource to include p_ReOffset as an input parameter and change dl_PRS_NumSymbols_r16 to n4 |
| TTCN module | LPP_Templates.ttcn |
| MCC160 Comment | Accepted conditional to agreement of prose CR. |

Before

<<SKIPPED CODE>>

template (value) NR_DL_PRS_Info_r16 cs_NR_DL_PRS_Info_Def(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset,

integer p_PrsSequenceId) := //@sic R5s221043 sic@

{//Acc. Table 8.4.1.6-3: NR-DL-PRS-Info

nr_DL_PRS_ResourceSetList_r16 := {cs_NR_DL_PRS_ResourceSet(p_PeriodicityAndOffset, p_PrsSequenceId)}

};

template (value) NR_DL_PRS_ResourceSet_r16 cs_NR_DL_PRS_ResourceSet(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset,

integer p_PrsSequenceId) := //@sic R5s221043 sic@

{

nr_DL_PRS_ResourceSetID_r16 := 0,

dl_PRS_Periodicity_and_ResourceSetSlotOffset_r16 := p_PeriodicityAndOffset,

dl_PRS_ResourceRepetitionFactor_r16 := omit,

dl_PRS_ResourceTimeGap_r16 := omit,

dl_PRS_NumSymbols_r16 := n2,

dl_PRS_MutingOption1_r16 := omit,

dl_PRS_MutingOption2_r16 := omit,

dl_PRS_ResourcePower_r16 := 0,

dl_PRS_ResourceList_r16 := {cs_NR_DL_PRS_Resource(p_PrsSequenceId)}

};

template (value) NR_DL_PRS_Resource_r16 cs_NR_DL_PRS_Resource(integer p_PrsSequenceId)  := //@sic R5s221043 sic@

{

nr_DL_PRS_ResourceID_r16 := 0,

dl_PRS_SequenceID_r16 := p_PrsSequenceId,

dl_PRS_CombSizeN_AndReOffset_r16 := {

n2_r16 := 0

},

dl_PRS_ResourceSlotOffset_r16 := 0,

dl_PRS_ResourceSymbolOffset_r16 := 0,

dl_PRS_QCL_Info_r16 := omit,

dl_PRS_ResourcePrioritySubset_r17 := omit /* @sic R5s221180 BASELINE MOVING 2022, Rel-17 sic@ */

};<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template (value) NR_DL_PRS_Info_r16 cs_NR_DL_PRS_Info_Def(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset,

integer p_PrsSequenceId, integer p_ReOffset) := //@sic R5s221043 sic@

{//Acc. Table 8.4.1.6-3: NR-DL-PRS-Info

nr_DL_PRS_ResourceSetList_r16 := {cs_NR_DL_PRS_ResourceSet(p_PeriodicityAndOffset, p_PrsSequenceId, p_ReOffset)}

};

template (value) NR_DL_PRS_ResourceSet_r16 cs_NR_DL_PRS_ResourceSet(NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 p_PeriodicityAndOffset,

integer p_PrsSequenceId, integer p_ReOffset) := //@sic R5s221043 sic@

{

nr_DL_PRS_ResourceSetID_r16 := 0,

dl_PRS_Periodicity_and_ResourceSetSlotOffset_r16 := p_PeriodicityAndOffset,

dl_PRS_ResourceRepetitionFactor_r16 := omit,

dl_PRS_ResourceTimeGap_r16 := omit,

dl_PRS_NumSymbols_r16 := n4,

dl_PRS_MutingOption1_r16 := omit,

dl_PRS_MutingOption2_r16 := omit,

dl_PRS_ResourcePower_r16 := 0,

dl_PRS_ResourceList_r16 := {cs_NR_DL_PRS_Resource(p_PrsSequenceId, p_ReOffset)}

};

template (value) NR_DL_PRS_Resource_r16 cs_NR_DL_PRS_Resource(integer p_PrsSequenceId, integer p_ReOffset)  := //@sic R5s221043 sic@

{

nr_DL_PRS_ResourceID_r16 := 0,

dl_PRS_SequenceID_r16 := p_PrsSequenceId,

dl_PRS_CombSizeN_AndReOffset_r16 := {

n2_r16 := p_ReOffset

},

dl_PRS_ResourceSlotOffset_r16 := 0,

dl_PRS_ResourceSymbolOffset_r16 := 0,

dl_PRS_QCL_Info_r16 := omit,

dl_PRS_ResourcePrioritySubset_r17 := omit /* @sic R5s221180 BASELINE MOVING 2022, Rel-17 sic@ */

};<<SKIPPED CODE>>

## 2.3 Correction to f_POS_NR_SimulateDlPrsPOSifSupported

| Function name | f_POS_NR_SimulateDlPrsPOSifSupported |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-CombSizeN-AndReOffset-r16 is 0 for all the TRPs. Since the RE offset is same for all TRPs (is overlapped), a conformant UE may not decode PRS sequences from different Physical Cell IDs. Therefore, a different dl-PRS-CombSizeN-AndReOffset-r16 value is needed per TRP to allow proper decoding of the PRS signals. |
| Summary of change | Include the template change from section 2.2 and set the value of p_ReOffset based on the NR Cell, setting a value of 0,1 and 2 for the 1st, 2nd and 3rd cell respectively |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted conditional to agreement of prose CR, but according to the revised prose CR, the value of p_ReOIffset should be ‘0’ for Cell 3. |

Before

<<SKIPPED CODE>>

function f_POS_NR_SimulateDlPrsPOSifSupported(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) { //@sic R5s221043 sic@

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_SimulateDlPrsPOSifSupported(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) { //@sic R5s221043 sic@

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0, 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1, 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2, -0);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

}<<SKIPPED CODE>>

## 2.4 Correction to f_POS_NR_CellConfig_DlPrs

| Function name | f_POS_NR_CellConfig_DlPrs |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-CombSizeN-AndReOffset-r16 is 0 for all the TRPs. Since the RE offset is same for all TRPs (is overlapped), a conformant UE may not decode PRS sequences from different Physical Cell IDs. Therefore, a different dl-PRS-CombSizeN-AndReOffset-r16 value is needed per TRP to allow proper decoding of the PRS signals. |
| Summary of change | Include the template change from section 2.2 and set the value of p_ReOffset based on the NR Cell, setting a value of 0,1 and 2 for the 1st, 2nd and 3rd cell respectively |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted conditional to agreement of prose CR, but according to the revised prose CR, the value of p_ReOIffset should be ‘0’ for Cell 3. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CellConfig_DlPrs(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) { //@sic R5s221043 sic@

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

} <<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CellConfig_DlPrs(NR_CellId_Type p_CellId) runs on NR5GC_POS_PTC

{

var template (value) NR_DL_PRS_Info_r16 v_DlPrsInfo;

select (p_CellId) { //@sic R5s221043 sic@

case (nr_Cell1) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 0, 0);

}

case (nr_Cell2) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 1, 1);

}

case (nr_Cell3) {

v_DlPrsInfo := cs_NR_DL_PRS_Info_Def(f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId), 2, 0);

}

case else { FatalError(__FILE__, __LINE__, "Invalid NR CellId"); }

}

}

<<SKIPPED CODE>>

## 2.5 Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP

| Function name | f_POS_NR_Set_DlPrsAssistanceDataPerTRP |
| --- | --- |
| Reason for change | According to 3GPP 37.571-2 Table 8.4.1.6-3, the value of dl-PRS-CombSizeN-AndReOffset-r16 is 0 for all the TRPs. Since the RE offset is same for all TRPs (is overlapped), a conformant UE may not decode PRS sequences from different Physical Cell IDs. Therefore, a different dl-PRS-CombSizeN-AndReOffset-r16 value is needed per TRP to allow proper decoding of the PRS signals. |
| Summary of change | Include the template change from section 2.2 and set the value of p_ReOffset based on the NR Cell, setting a value of 0,1 and 2 for the 1st, 2nd and 3rd cell respectively |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted conditional to agreement of prose CR, but according to the revised prose CR, the value of p_ReOIffset should be ‘0’ for Cell 3. |

Before

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 0, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0)); //@sic R5s221043 sic@

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1)); //@sic R5s221043 sic@

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2)); //@sic R5s221043 sic@

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 0, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0, 0)); //@sic R5s221043 sic@

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1, 1)); //@sic R5s221043 sic@

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2, 0)); //@sic R5s221043 sic@

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}<<SKIPPED CODE>>

# Branches executed

## This POS TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n71” using NIA2 Integrity and NEA2 ciphering algorithms

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_9_3_4_4_21s_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230102: Supporting information for addition of NR POS testcase 9.3.4.4.21s |
| --- | --- |