3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230108

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2939 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC 7.1.1.4.2.x test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the prose CR R5-227416, the proposed changes was to use the default 38.508-1 configuration for PDSCH-TimeDomainResourceAllocationList - The current TTCN implementation has been incorrectly updated for PUSCH-TimeDomainResourceAllocationList, this has to be aligned to the prose - The default 38.508-1 configuration uses SLIV of 53 (S=2, L=12 )as the first entry and sliv as 72 for the second entry (S=2, L=6). In order to ensure that the max TBS is chosen for the DL transmission the DL grant configuration needs to use SLIV of 53 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - pdsch_TimeDomainAllocationList is removed so that the default value from 38.508-1 can be used. - startSymbolAndLength of 52 and K2=2 is configured for FR1, startSymbolAndLength of 42 and K2=4 is configured for FR2. v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList is updated with the above values - In f_NR_DLGrantConfiguration modified the 5th parameter to use index 0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.NR5GC, 7.1.1.4.2.1.ENDC, 7.1.1.4.2.3. ENDC, 7.1.1.4.2.4. ENDC, 7.1.1.4.2.5. ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required Uplink TBS test cases	3

2.1	Correction to f_NR_Init_7_1_1_4_2_x	3

2.2	Correction to   f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody	7

2.3	Correction to f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required Uplink TBS test cases

## Correction to f_NR_Init_7_1_1_4_2_x

| Function name | f_NR_Init_7_1_1_4_2_x |
| --- | --- |
| Reason for change | - According to the prose CR R5-227416, the proposed changes was to use the default 38.508-1 configuration for PDSCH-TimeDomainResourceAllocationList - The current TTCN implementation has been incorrectly updated for PUSCH-TimeDomainResourceAllocationList, this has to be aligned to the prose |
| Summary of change | - pdsch_TimeDomainAllocationList is removed so that the default value from 38.508-1 can be used. - startSymbolAndLength of 52 and K2=2 is configured for FR1, startSymbolAndLength of 42 and K2=4 is configured for FR2. v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList is updated with the above values |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_Init_7_1_1_4_2_x(NR_CellId_Type p_NR_CellId, NR_SearchSpace_SearchSpaceType_Type.ue_Specific.dci_Formats p_DciFormats, template (omit) PUSCH_Config.mcs_Table p_Mcs_Table, template (omit) PUSCH_Config.transformPrecoder p_TransformPrecoder,//@sic R5s201144 sic@ template (omit) RACH_ConfigCommon.msg3_transformPrecoder p_Msg_TransformPrecoder := omit) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var template (value) NR_DownlinkBWP_Type v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_NR_CellId, v_BWP_Id); var integer v_SearchSpaceIndex := f_NR_DownlinkBWP_GetSearchSpace_Index(v_DownlinkBWP, tsc_NR_SearchSpaceId_USS); //@sic R5s190927 sic@ // var integer v_K2 := 4; // FR2 and Fr1 //var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; var NR_CellInfo_Type v_NR_CellInfo :=f_NR_CellInfo_Get(p_NR_CellId); //@sic R5s201144 sic@ var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_StartSymbolAndLength := 52;} //@sic R5-227416 sic@ //v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_Table := p_Mcs_Table; //qam64 if omit v_UplinkBWP.Dedicated.R15.pusch_Config.setup.transformPrecoder :=p_TransformPrecoder; v_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_71142x(v_SubcarrierSpacing)); //@sic R5-210321 sic@ v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; if (match(enabled, p_TransformPrecoder)) { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_TableTransformPrecoder := omit; } if (ispresent (p_Msg_TransformPrecoder)){//@sic R5s201144 sic@ v_UplinkBWP.Common.R15.rach_ConfigCommon.setup.msg3_transformPrecoder := p_Msg_TransformPrecoder; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.msg3_transformPrecoder := enabled; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } } v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex].SearchSpaceConfigAtUE.R15.searchSpaceType.ue_Specific.dci_Formats := p_DciFormats; if(p_DciFormats == formats0_1_And_1_1) { if (pc_dynamicSwitchRA_Type0_1_PUSCH) //@sic R5s202610 sic@ { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation := dynamicSwitch; } else { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation :=resourceAllocationType1 } } //@sic R5-216170 sic@ v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x; f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); f_NR_CellInfo_SetDownlinkBWP(p_NR_CellId, v_DownlinkBWP, v_BWP_Id); f_NR_CellInfo_Pdsch_ConfigCommon(p_NR_CellId, cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x); //@sic R5s220098 sic@ };

After Change:

function f_NR_Init_7_1_1_4_2_x(NR_CellId_Type p_NR_CellId, NR_SearchSpace_SearchSpaceType_Type.ue_Specific.dci_Formats p_DciFormats, template (omit) PUSCH_Config.mcs_Table p_Mcs_Table, template (omit) PUSCH_Config.transformPrecoder p_TransformPrecoder,//@sic R5s201144 sic@ template (omit) RACH_ConfigCommon.msg3_transformPrecoder p_Msg_TransformPrecoder := omit) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var template (value) NR_DownlinkBWP_Type v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_NR_CellId, v_BWP_Id); var integer v_SearchSpaceIndex := f_NR_DownlinkBWP_GetSearchSpace_Index(v_DownlinkBWP, tsc_NR_SearchSpaceId_USS); //@sic R5s190927 sic@ var integer v_K2 := 4; // FR2 var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; var NR_CellInfo_Type v_NR_CellInfo :=f_NR_CellInfo_Get(p_NR_CellId); //@sic R5s201144 sic@ var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_K2 := 2; v_StartSymbolAndLength := 52; } // according to the prose dedicated timedomainresource allocation is specific to the tes case v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_Table := p_Mcs_Table; //qam64 if omit v_UplinkBWP.Dedicated.R15.pusch_Config.setup.transformPrecoder :=p_TransformPrecoder; v_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_71142x(v_SubcarrierSpacing)); //@sic R5-210321 sic@ v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; if (match(enabled, p_TransformPrecoder)) { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_TableTransformPrecoder := omit; } if (ispresent (p_Msg_TransformPrecoder)){//@sic R5s201144 sic@ v_UplinkBWP.Common.R15.rach_ConfigCommon.setup.msg3_transformPrecoder := p_Msg_TransformPrecoder; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.msg3_transformPrecoder := enabled; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } } v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex].SearchSpaceConfigAtUE.R15.searchSpaceType.ue_Specific.dci_Formats := p_DciFormats; if(p_DciFormats == formats0_1_And_1_1) { if (pc_dynamicSwitchRA_Type0_1_PUSCH) //@sic R5s202610 sic@ { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation := dynamicSwitch; } else { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation :=resourceAllocationType1 } } //@sic R5-216170 sic@ // v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x; f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); f_NR_CellInfo_SetDownlinkBWP(p_NR_CellId, v_DownlinkBWP, v_BWP_Id); //WA#WI comment out as we should use the default value in 38.508-1 according to the prose //f_NR_CellInfo_Pdsch_ConfigCommon(p_NR_CellId, cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x); //@sic R5s220098 sic@ };

## Correction to   f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody

| Function name | f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody |
| --- | --- |
| Reason for change | The default 38.508-1 configuration uses SLIV of 53 (S=2, L=12 )as the first entry and sliv as 72 for the second entry (S=2, L=6). In order to ensure that the max TBS is chosen for the DL transmission the DL grant configuration needs to use SLIV of 53 |
| Summary of change | In f_NR_DLGrantConfiguration modified the 5th parameter to use index 0 |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_TBSizeLow := 136; //lowest allowed for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB),//@sic R5s220558 sic@ int2bit(1, 4), //@sic R5s211325 sic@ cs_NR_TransportBlockRetransmissionList_Def(28), //@sic R5s220797 sic@ -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1

After Change:

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_TBSizeLow := 136; //lowest allowed for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB),//@sic R5s220558 sic@ int2bit(0, 4), //@sic R5s211325 sic@ // to use the first entry in the time domain list. (K0=0 and SLIV 53) cs_NR_TransportBlockRetransmissionList_Def(28), //@sic R5s220797 sic@ -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1

## Correction to f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody

| Function name | f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody |
| --- | --- |
| Reason for change | The default 38.508-1 configuration uses SLIV of 53 (S=2, L=12 )as the first entry and sliv as 72 for the second entry (S=2, L=6). In order to ensure that the max TBS is chosen for the DL transmission the DL grant configuration needs to use SLIV of 53 |
| Summary of change | In f_NR_DLGrantConfiguration modified the 5th parameter to use index 0 |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ var integer v_TBSize :=0; // transport block size in bits var integer v_I_MCS :=0; // Modulation and Coding Scehme Index var integer v_N_PRB :=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var IntegerList_Type v_NoOfSymbols := {4,14}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_Max_Imcs :=p_Max_Imcs; //@sic R5s211564 sic@ var integer v_TBSizeLow := 136; //for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(1, 4), //@sic R5s211325 sic@ cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_1_Params(v_SCS)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1

After Change:

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ var integer v_TBSize :=0; // transport block size in bits var integer v_I_MCS :=0; // Modulation and Coding Scehme Index var integer v_N_PRB :=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var IntegerList_Type v_NoOfSymbols := {4,14}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_Max_Imcs :=p_Max_Imcs; //@sic R5s211564 sic@ var integer v_TBSizeLow := 136; //for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), //@sic R5s211325 sic@ // to use the firs entry in the time domain list. (K0=0 and SLIV 53) cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_1_Params(v_SCS)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1