3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230158

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2976 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 8.2.6.2.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.2.6.2.2 to the IWD_22wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.6.2.2 required for approval. See the detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_NR5GC_RRC_ProcedureDelay ()	4

2.2	Function f_TC_8_2_6_2_2_NR5GC_TestBody ()	13

3	Branches executed	19

4	Execution Log Files	19

4.1	Snapdragon X70 5G Modem-RF System	19

5	References	19

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.2.6.2.2  which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D22wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	8.2.6.2.2

ATS Version:	iwd-TTCN3-B2022-09_D22wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Snapdragon X70 5G Modem-RF System, Qualcomm Technologies’ 5th generation modem-to-antenna 5G solution

Verification Status:	PASS

- Corrections required

  - Function f_NR5GC_RRC_ProcedureDelay ()

| Function name | f_TC_8_2_6_2_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - In the current TTCN Implementation , handling of Pdu session modification complete is done outside of the function f_NR5GC_RRC_ProcedureDelay() This behaviour is not working stably with different UE behaviours . - Some UEs are too quick in sending the Pdu Session Modification Complete along with reconfiguration complete - As the grant is periodic inside the function f_NR5GC_RRC_ProcedureDelay , before stopping the grant , there could be corner case of sending second UL grant which will help UE to send the Pdu Session Modification Complete , and causes the test case to fail ( unexpected Message received ) To avoid this race condition happening, It is proposed to handle the Pdu Session Modification Complete inside the function f_NR5GC_RRC_ProcedureDelay() in case of NR-DC ; |
| Summary of change | - To avoid the instability during the reception of Pdu Session Modification complete , it is proposed to handle the Pdu Session Modification Complete inside the function f_NR5GC_RRC_ProcedureDelay() by using the boolean flag nrdc_pdusession which is set only in case of 8.2.6.2.2 ( NR DC Procedure delay test case ) |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted, implemented slightly differently: the new parameter p_NRDC_PduSession is defined as a Boolean with default value fase and the runs on is changed to NR5GC_PTC. See MCC160 proposed implementation. |

Before Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }

After Change:

import from NG_NAS_Common all; //WA#WI=988940 import from NG_NASTemplateFunctions all; //WA#WI=988940 function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, template (omit) boolean nrdc_pdusession := omit //WA#WI=988940 ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //WA#WI=988940 timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } if(isvalue(nrdc_pdusession)) //WA#WI=988940 { if(nrdc_pdusession) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }

MCC160 proposed implementation:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep,

template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def,

boolean p_NRDC_PduSession := false

) runs on NR5GC_PTC return NR_SRB_COMMON_IND

{ //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 sic@

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1;  //@sic R5s210842 change 2.1 sic@

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN);  //@sic R5s230158 sic@

timer t_Watchdog;

//@sic R5s210842 sic@

if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) {

v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

} else {

v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame);

}

v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

v_K1 := 7;

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

v_K1 := 7;

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

v_K1 := 5;

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

v_K1 := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

//Calculate T3

v_T3Slots := v_T1Slots + v_Nslots + v_Delta1;  //@sic R5s210842 change 2 sic@

v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe);

//T1: send DL Msg

SRB.send(v_SRB_COMMON_REQ);

//Enable HARQ ack/nack reporting

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@

//T3 configure SS with periodic UL Grant at T3

f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity));

// Start WatchDog timer

v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId);

t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0);  /* 50ms added to be at the safe side */

alt {

/* HARQ NACK */

[]SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) {

v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1;

if (v_DL_RetransmissionCnt > 4) {

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4");

} else {

repeat;

}

}

/* HARQ ACK */

[]SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND {

v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame;

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq);    /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */

v_T2Flag := true;

repeat;

}

/* HARQ error indication */

[] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) {

f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test");

t_Watchdog.stop;

}

[] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND {

v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_T4Flag := true;

t_Watchdog.stop;

}

[] t_Watchdog.timeout {                                                                                     /* timeout => after T5 */

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout");

}

}

//@sic R5s230158 sic@

if (p_NRDC_PduSession) {

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed");

}

}

//Stop UL Grant

f_NR_ULGrantConfiguration_Stop(p_NR_CellId);

if (not v_T2Flag) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received");

}

if (not v_T4Flag) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received");

}

//Check the Delay

v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots;

v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots;

//Initialise Delta2

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_Delta2 := 0;

} else { //TDD

select(v_SCS) {

case(kHz15) {

v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5;

}

case(kHz30) {

v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10;

}

case(kHz120) {

v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep);

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep);

}

return v_ReceivedNR_SRB_COMMON_IND;

}

  - Function f_TC_8_2_6_2_2_NR5GC_TestBody ()

| Function name | f_TC_8_2_6_2_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - In the current Implementation , at step 2 reconfiguration message is sent to add NR PSCell with SCG DRB. While sending reconfiguration message , SKCounter value has to be filled - f_NR5GC_RRC_ProcedureDelay is passed with nrdc_pdusession = true specific to test case 8.2.6.2.2 to handle the Pdu Session Modification Complete ; this is done corresponding to the reasoning given in section 2.1 - handling of Pdu Session Modification Complete, is moved inside the function f_NR5GC_RRC_ProcedureDelay due to differences in UE behaviour - UL grant is stopped inside the function f_NR5GC_RRC_ProcedureDelay which is not started again to handle the remaining procedures in the test case. so Ul grant has to be started again once out of the function f_NR5GC_RRC_ProcedureDelay(). |
| Summary of change | - At Step 2, while sending reconfiguration message , SKCounter value has to be filled - f_NR5GC_RRC_ProcedureDelay is passed with nrdc_pdusession = true - handling of Pdu Session Modification Complete is removed from the function f_TC_8_2_6_2_2_NR5GC_TestBody and moved inside the function f_NR5GC_RRC_ProcedureDelay - Ul grant is started again once finished with f_NR5GC_RRC_ProcedureDelay() |
| TTCN module | 8_2_6\RRC_Processing_delay_NRDC_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_8_2_6_2_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (omit) RadioBearerConfig v_RadioBearerConfig1 := omit; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (omit) CellGroupConfig v_CellGroupConfigMCG := omit; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (omit) NR_RadioBearerList_Type v_SS_ConfigMCG := omit; var template (omit) NR_RadioBearerList_Type v_SS_ConfigSCG := omit; var template (value) SDAP_Config v_SDAP_ConfigSCG; var integer v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(nr_Cell10); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var RNTI_Value v_RNTI_ValueSecondary := tsc_C_RNTI_Value4; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; // @siclog "Step 1" siclog@ // SS stops default uplink grant transmission. f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 2-4" siclog@ //The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add NR PSCell with SCG DRB. //The SS transmits one uplink grant after (16*2microsec+1+TDL) slots after step 2 to schedule PUSCH. //Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 2? if (isvalue(v_GSM_MobilityInfo)) { v_QoSFlow := cs_QoS_Flow5; v_QoSFlow.qfi := '000101'B; // QFI = 5 v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow})); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); v_DRBToAddMod_SCG := cs_38508_NRDRB_ToAddMod(v_SDAP_ConfigSCG, v_SCG_DRB_Id, cs_38508_PDCP_Config); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_SCG_DRB_Id, v_RNTI_ValueSecondary); // Build info for SCG and store in DRB list fl_StoreSCG_InDRBList (valueof(v_DRBToAddMod_SCG), valueof(v_SS_ConfigSCG[0]), valueof(v_CellGroupConfigSCG)); // Calculate security keys for SCG v_SecurityParams := f_NR_Security_InitSCG(v_SecurityParams); f_NR_Security_Set(v_SecurityParams); f_ReconfigureSS_DRBs_NRDC( nr_Cell1, nr_Cell10, -, v_SS_ConfigMCG, v_SS_ConfigSCG, MCG_SCG_SecondaryKeys ); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2 ); v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig1, -, omit, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, v_RrcMsg, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?)))), 16, "Step 4"); //@siclog "Step 5" siclog@ //The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } }

After Change:

function f_TC_8_2_6_2_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (omit) RadioBearerConfig v_RadioBearerConfig1 := omit; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (omit) CellGroupConfig v_CellGroupConfigMCG := omit; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (omit) NR_RadioBearerList_Type v_SS_ConfigMCG := omit; var template (omit) NR_RadioBearerList_Type v_SS_ConfigSCG := omit; var template (value) SDAP_Config v_SDAP_ConfigSCG; var integer v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(nr_Cell10); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var RNTI_Value v_RNTI_ValueSecondary := tsc_C_RNTI_Value4; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; // @siclog "Step 1" siclog@ // SS stops default uplink grant transmission. f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 2-4" siclog@ //The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add NR PSCell with SCG DRB. //The SS transmits one uplink grant after (16*2microsec+1+TDL) slots after step 2 to schedule PUSCH. //Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 2? if (isvalue(v_GSM_MobilityInfo)) { v_QoSFlow := cs_QoS_Flow5; v_QoSFlow.qfi := '000101'B; // QFI = 5 v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow})); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); v_DRBToAddMod_SCG := cs_38508_NRDRB_ToAddMod(v_SDAP_ConfigSCG, v_SCG_DRB_Id, cs_38508_PDCP_Config); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_SCG_DRB_Id, v_RNTI_ValueSecondary); // Build info for SCG and store in DRB list fl_StoreSCG_InDRBList (valueof(v_DRBToAddMod_SCG), valueof(v_SS_ConfigSCG[0]), valueof(v_CellGroupConfigSCG)); // Calculate security keys for SCG v_SecurityParams := f_NR_Security_InitSCG(v_SecurityParams); f_NR_Security_Set(v_SecurityParams); f_ReconfigureSS_DRBs_NRDC( nr_Cell1, nr_Cell10, -, v_SS_ConfigMCG, v_SS_ConfigSCG, MCG_SCG_SecondaryKeys ); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2,-,0 ); //WA#WI=988940 v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig1, -, omit, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, v_RrcMsg, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?)))), 16, "Step 4", -,true); //@siclog "Step 5" siclog@ //The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. /*SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); }*/ f_NR_ULGrantConfiguration_Start(nr_Cell1); //WA#WI=988940 }

- Branches executed

## This NR TC was executed in NR-DC FR1+FR2 Band Combination DC_n79A_n257A with NEA1 and NIA1 ciphering and integrity algorithms.

- Execution Log Files

  - Snapdragon X70 5G Modem-RF System

The Snapdragon X70 5G Modem-RF System, Qualcomm Technologies’ 5th generation modem-to-antenna 5G solution passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_2_6_2_2_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s230159 Supporting information for addition of NR-DC test case 8.2.6.2.2 in FR1+FR2 |
| --- | --- |